Delivered to the Government in Accordance with Contract FA8650-10-C-7080

Gray-Box Optimization

Motivation:

In system models, explicit mathematical models are not always available or accessible for all components/sub-systems. Such models are called black-box models. Some of the classic examples of black box models are high-fidelity simulation codes (e.g. FEM or CFD), and models provided by suppliers that are in executable format.

A number of black-box optimization techniques focus on handling such models, e.g. Genetic Algorithm (GA). These black-box optimization techniques are primarily focused on avoiding the use of gradients, which are difficult to generate for black-box functions.

However, there are many instances when the entire system model is not in black-box format. Many times, mathematical equations are indeed available for a subset of the system, such as component connectivity information. We call such models as Gray-box models. This nomenclature indicates that a system model is comprised of mix of black-box (mathematical formulation NOT available) and white-box (mathematical formulation available) functions.

Although significant progress has been made in the area of mathematical and black-box optimization, no clear principles exists on how to formulate optimization for system models that contain both white and black-box functions (i.e. gray- box functions).

We have developed a seamless optimization approach that can handle any types of systems models – White, Gray, and Black box. The approach appears to be robust. Also, we demonstrate that formulating gray-box structure that maintains dependency (our approach) is faster than treating entire model as black-box function (conventional approach).

Figure 1: Comparison of optimization approaches

Demonstration platform for Gray-Box Optimization:

For the demonstration purposes, we use AMPL as the modeling language and IPOPT as the optimization package.

AMPL is the modeling language and system for formulating, solving and analyzing large-scale mathematical (white-box) optimization problems. It is developed by Bell Labs. AMPL’s two key features used in this study are (1) ability to integrate user-defined functions (black-box for us) and (2) ability to generate fast automatic differentiation for white box part. Additionally, AMPL provides interfaces to popular and sophisticated solvers including IPOPT, which is used in this study.

Ipopt (Interior Point OPTimizer) is a software package for large-scale nonlinear optimization. It is designed to find (local) solutions of mathematical optimization problems. Ipopt is written in C++ and is released as open source code.

Implementing Gray/Black-box function in AMPL:

Figure 2 shows the implementation of black-box function in AMPL environment. The wrapper (amplfunc.dll) allows different functions (black-box calls) to be used within AMPL. Functions defined through the wrapper can be called just like standard functions though AMPL.  The wrapper function (written in C) has been implemented such that it returns (1) function value, and (2) gradient value calculated using finite difference scheme. We do not pass any hessian values, as we tune IPOPT to estimate approximate hessian using the gradient values.

Figure 2: Black-box function implementation in AMPL

Formulating Gray-box optimization:

To understand how we formulate gray-box optimization problem, it is critical to understand the mechanism inside IPOPT algorithm. The optimization problem formulation is given as:

Where f is the objective function, c is the set of constraints, and x is the set of design variables.

In each iteration, IPOPT calculates a search direction {dk} to explore the optimal solution. To identify the search direction {dk}, IPOPT solve the systems of linear equations given by

As we can see, a number of elements in the above equations results from the gradient estimation. Also, the size of the above matrix will grow with the size of the problem. To handle the computational complexity, IPOPT uses sparse matrix solvers, e.g., MUMPS. To define the sparsity, IPOPT uses the dependency structure information provided by AMPL.

However, for black-box model, complete dependency is assumed. We explain this statement using Figure 3. As shown in Figure 3, the black-box function takes n input variables (x1 to xn) and outputs m values (F1 to Fm).

Figure 3: Complete dependency assumed for black-box function

Jacobian must be a fully populated matrix for black-box function. In other word, each output Fi from black-box is assumed to be function of every input variable (x1 to xn). The Jacobian obtained from the black-box function is used to populate the matrices used for estimating search direction.

As we can see, if we do not pay attention to the dependency structure while defining black-box or bray-box functions, the jacobian will be a fully populated matrix. As a result the matrix size will grow – not as a result of problem size, but as a result of poor black-box definition.

Results for Gray-box optimization:

To demonstrate the effect of dependency structure in defining gray-box optimization, we use an example of two-3-bar-truss, as shown in Figure 4.

Figure 4: Two-3-bar-truss example

The optimization problem is given as

Where, Wt1 and Wt2 are weights, 1 through 4 are stresses in the elements, and x1 through x4 are cross-section areas, which are also design variables. Table 1 shows the dependency structure for this problem. As we can see, functions Wt1, 1, and 2 are not functions of x3 and x4. Similarly, Wt2, 3, and 4 are not functions of x1 and x2.

Table 1: Dependency structure for two-3-bar-truss

|  | x1 | x2 | x3 | x4 |
| --- | --- | --- | --- | --- |
| Wt1 | Y | Y | -- | -- |
| 1 | Y | Y | -- | -- |
| 2 | Y | Y | -- | -- |
| Wt2 | -- | -- | Y | Y |
| 3 | -- | -- | Y | Y |
| 4 | -- | -- | Y | Y |

Table 2: Results of different Gray-box formulations

| Box Type | Iterations | Black-box function definition |
| --- | --- | --- |
| White | 54 | NA |
| Gray # 1 | 54 | 1(x1,x2) in black-box#1 3(x3,x4) in black-box# 2 |
| Gray # 2 | 55 | Wt1 ,1, 2 (x1,x2) in black-box #1 Wt2,3, 4 (x3,x4) in black-box #2 |
| One Black Box | 92 | Wt1 ,1, 2, Wt1 ,3, 4 (x1,x2,x3,x4) in single black-box |

Table 2 shows the results of different gray-box formulation. We can see that for cases Gray # 1 and Gray # 2, the number of iterations needed to get the optimal is very close to that needed for white box. In either of these cases, the black-box part is defined such that dependency structure is maintained.

However, when entire problem is defined in one black-box, the dependency structure is lost. In other word, output 1 is considered as a function of all the variables from x1 through x4. As results, higher number of iterations were needed to get the same optimal.

This demonstrates that (1) Gray-box optimization cannot be treated as black-box optimization, and (2) violating dependency structure while defining gray-box formulation can results in inefficient optimization.

Implementation for Engine problem

Gray-box optimization has been implemented in engine example. This implementation requires no change to software architecture. Here are the set of files and their purpose.

- funcadd.c

This implements different functions (black-box calls) to be used within AMPL (e.g. FAN, square and quad2 (sum of squares of 2 arguments).  We have written the functions to return the function value and gradient. The hessian value is set = 0, as we use hessian approximation available in IPOPT.

- makefile.vc

This is the makefile that can be run using:	nmake -f makefile.vc

This creates the amplfunc.dll which allows your AMPL models to call the functions implemented in funcadd.c.  For the command to execute you will need to be in a command prompt that is provided by Microsoft Visual Studio.  You can access that through: Start -> Microsoft Visual Studio 200x -> Visual Studio Tools -> Visual Studio 200x Command Prompt.

- ipopt.opt

This is an options file.  Currently, it is set to use limited memory hessians.

- two_truss.txt

This is the two-three bar truss example. The user has to uncomment different parts to run white/gray/black-box models.

- EngineModel_GBO.txt

This is the engine example. The user has to uncomment different parts to run white/gray/black-box models. Search for text “# Gray-Box” for all the gray-box functions. The user has to uncomment different parts to run white/gray/black-box models.