Cost Models based on empirical data:

Notations:

T = Thrust (lbs)

TSFC = Thrust Specific Fuel Consumption

Wt = dry engine weight (lbs)

Dev_Cost = Development Cost in 2001 US$ (millions)

Mfg_Cost = Manufacturing cost or first unit cost in 2001 US$(million)

- Mfg_Cost = 0.289 + 0.00014*T+1.33*TSFC-0.29*Wt ->  (R^2 = 0.89; R^2_adj =0.87)

- Mfg_Cost (w/o Weight) = 0.054+0.00011*T+1.226*TSFC -> (R^2 = 0.86; R^2_adj =0.85)

- Wt = -5388.1+0.71*T+10615*TSFC-0.922*T*TSFC -> (R^2 = 0.9; R^2_adj =0.88)

Considering “Newly-designed” engines alone, the Development cost is:

- Dev_Cost = 0.11*T+474.71*TSFC-0.395*Wt-138.14 -> (R^2 = 0.74; R^2_adj =0.69)

Considering all enhines, the development cost is:

- Dev_Cost = 0.00012*T+1163*TSFC+0.31*Wt–731.54 -> (R^2 = 0.54; R^2_adj =0.52)

** The basic database for development cost is of spurious nature since some programs seem to be “derivative” engines with substantially low development cost compared their magnified thrust level. This distorted the significance of thrust level on development cost, significantly impairing statistical significance.