# Architecture Enumeration and Evaluation (AEE):                   Design-Rule Specification Language

By

Lawrence E. Zeidner

United Technologies Research Center, East Hartford, CT 06108

# Introduction

This document describes the Design-Rule Specification Language (DRSL) used within AEE to express Design rules.  Design rules are used by AEE to rapidly explore a design space by rapidly ruling out large logical sections of the design space as necessarily infeasible.  Rather than constructing every complete system architecture in the combinatorically large design space, and then using “evaluative” rules to check them for feasibility, “generative rules” are used to build and test partial architectures progressively, checking them as each new technology option is added.  These rules are called “generative” because the majority of the full-system architectures they build are feasible, the infeasible ones having been ruled out, without explicitly visiting most of them, as the partial architectures were being built.  Ultimately, as the last technology option is assembled, the partial architecture becomes afull-system architecture.

AEE first finds all of the combinations of devices that satisfy the device rules. Then, for each feasible combination of devices, it finds all of the combinations of flows between these devices that satisfy the flow rules.  The result of this second design-space exploration (DSE) is the list of ALL feasible full-system architectures, including devices and flows.  The second DSE is usually significantly more computationally intensive than the first.

AEE does not actually “enumerate” each potential architecture in the combinatorically large design space.  It is fast primarily because it is able to rule out large sections of the design space, based on particular partial architectures failing specific design rules.  AEE is also fast because it doesn’t check every design rule for every partial architecture that it constructs.  AEE determines, before it starts exploring the design space, for each design rule, and for each type of device or flow that is being assembled on to the previously feasible partial architecture checked last, whether the design rule could possibly fail.  During design space exploration, AEE consults this information to check only a small fraction of the design rules for each partial architecture.  Finally, AEE achieves additional speed by only evaluating each value described in the rules, at most, once per partial architecture.

# Constructs

Design rules describe both the mission to be performed, and compatibility among the technology options.  These two classes of design rules are often segregated because rules about compatibility among the technology options are reusable and can be selected automatically based on the selection of relevant technology options, while each design problem includes its own new mission.  In this description, we will not segregate based on this distinction.

Design rules are expressed in terms of statements that specify logical relationships and comparisons between real and Boolean values. In DRSL, three types of statements are defined.  Real-valued “quantities” can be defined, logical “premises” can be defined, and “rules” can be expressed in terms of comparisons between quantities or between premises.  Further, quantities and premises can contain either scalar values, vector values, or a tree-like nested vector of values.  These will all be called vectors of different dimensionality, i.e., scalar: V(0), vector: V(1), vectors of vectors: V(2), V(3), etc.  For example, some simple quantity values:

- Scalar quantity (quantity vector of dimension 0, i.e., QV(0)): 3.75

- Quantity vector of dimension 1 (i.e., QV(1)): (2.5, 3.14, 0.2, 100.6)

- Quantity vector of dimension 2 (i.e., QV(2)): (6.1, (8.7, 9.2, 124.0), (-45.2, 15.0), -19.7)

- Quantity vector of dimension 3 (i.e., QV(3)): (8.6, -72.2, (81.0, -25.3, (101.2, -14.2), -99.1), 0.0, -8.1)

Examples of simple premise values:

- Scalar premise (quantity vector of dimension 0): 1   [Note: 1 represents True here; 0 represents False]

- PV(1): (0, 1, 1, 0, 1)

- PV(2): (1, 0, (1, 1, 0, 1), 0, 1, (1, 1), 0, 1)

- PV(3): (1, 1, 1, 0, 1, (0, 0, (1, 1, 0), 0), 0, 1, 1)

In DRSL, each statement can specify a rule, a premise, or a quantity.  A rule is simply a premise that is enforced to be true.  For vector values of premises, rules are enforced to be completely true at the deepest level only, so the following examples show how premise values would be evaluated as rules:

- True PV(0): 1

- False PV(0): 0

- True PV(1): (1, 1, 1, 1)

- False PV(1): (1, 0, 0, 1)

- True PV(2): (1, 1, (1, 1, 1), 0, 1, (1, 1, 1), 0, 0, 0)

- False PV(2): (1, 1, (1, 1), 1, 1, (1, 0), 1)

# Device-Rule Statement Syntax

Since premises and rules use the same syntax, there are really only two types of device-rule statements:

<device-rule-statement> ::=	<dr-define-premise-or-rule> | <dr-define-quantity>

## Defining Premises or Rules

There are multiple types of device-rule statements to define premises or rules.

<dr-define-premise-or-rule> ::=	   <dr-premise-rule-simple> | <dr-premise-rule-comparison>

| <dr-premise-rule-mutual-excl> | <dr-premise-rule-logical-fn>

### Defining a premise based on a constant

A premise can be set to a constant value.

<dr-premise-rule-simple>  ::=	<dr-id-act-defpr-name>  “Simple” “Constant” <t-f>

where the following non-terminals are defined:

<dr-id-act-defpr-name> ::=	<stmt-id> <y-n> <dr-def-premrule-name>

All device-rule statements begin with a statement id, which is a sequence number

<stmt-id> ::=			<positive-integer-sequence-number-for-rules>

Next is a term to indicate whether or not the statement is active.  This is a practical consideration, since it allows the user to turn statements on and off to test and use the design rules for AEE.

<y-n> ::= 			"Y" | "N"

Next are two terms to indicate whether this statement is defining a premise or a rule.  If it is a rule, then its truth will be enforced.  If it is a premise, then it must have a name to be referenced in subsequent statements.  If it is a rule, this name is optional.

<dr-def-premrule-name> ::= 	<dr-def-rule> | <dr-def-premise>

<dr-def-rule>::=			"DefineRule" <pname> | “Define Rule”

<dr-def-premise> ::=		"DefinePremise" <pname>

<pname> ::=			<name>

<name> ::= 			<text-without-blanks>

<t-f> ::=				"True" | "False"

For example, a device-rule statement to define a premise based on a constant would be:

1, Y, DefinePremise, Embedded, Simple, Constant, True

This statement would define the premise Embedded to be true.

### Defining a premise based on a comparison

A premise can be set to a constant value.

<dr-premise-rule-comparison>  ::=	<dr-id-act-defpr-name>  “Comparison” <quantity> <ineq> <quantity>

where the following non-terminals are defined:

<quantity> ::=			"Constant" <realnumber-value>

| "Quantity" <qname>

| "#DeviceInstances" <device-name>

| "#BlockInstances" <device-blockname>

<qname> ::=			<name>

<ineq> ::=			"<" | "<=" | "<>" | "=" | ">=" | ">"

<device-name> ::= 		"Device_Name" <text-name-from-device-table>

<device-blockname> ::= 		"Device_BlockName" <text-blockname-from-device-table>

For example, a device-rule statement to define a premise based on a comparison between two quantities would be:

2, Y, DefinePremise, Requirement_Satisfied, Comparison, Actual, >=, Required

This statement would define the premise Requirement_Satisfied to be true if the quantity Actual is >= the quantity Required.

### Defining a premise based on Mutual Exclusion

A premise can be set to be true if a set of only one of a set of other premises is true, i.e., the set of premises is mutually exclusive.

<dr-premise-rule-mutual_excl> ::=	<dr-id-act-defpr-name>  “MutualExcl” <pname> <pname>

| <dr-id-act-defpr-name>  “MutualExcl” <pname> <pname> <pname>

For example, a device-rule statement to enforce mutual exclusion among three premises would be:

3, Y, DefineRule, , MutualExcl, TurboProp, TurboFan, TurboJet

This statement would ensure that a jet engine architecture can’t have propeller(s) together with fan(s), but can have either propeller(s) or fan(s), or neither (turbojet).

### Defining a premise based on a Logical Function

A premise can be defined based on a logical function applied to 1-3 premises:

<dr-premise-rule-logical_fn> ::=	<dr-id-act-defpr-name>  <logic1> <pname>

| <dr-id-act-defpr-name> “BoolFn” <pname> <logic2> <pname>

| <dr-id-act-defpr-name> “IF” <pname> “THEN” <pname>

| <dr-id-act-defpr-name> “IF” <pname> “THEN” <pname> “ELSE” <pname>

<logic1> ::=			"NOT"

<logic2> ::=			"AND" | "OR" | "NAND" | "NOR" | "IF/THEN"

For example, a device-rule statement to define a premise based on a logical function applied to 2 premises would be:

4, Y, DefinePremise, Multiple_Fans, BoolFn, Multiple_Fans_Ducted_to_Core, OR, Multiple_Fans_Not_Ducted_to_Core

This statement would define a premise that included multiple fans whether or not they were ducted to the core.

## Defining Quantities

There are multiple types of statements to define device-rule quantities.

<dr-define-quantity> ::=	 	  <dr-quantity-simple> | <dr-quantity-math-fn>

| <dr-quantity-simple-#dev> | <dr-quantity-simple-#devblk>

| <dr-quantity-expression>

### Defining a quantity based on a constant

A device-rule quantity can be set to a constant value.

<dr-quantity-simple-constant>  ::=	<dr-id-act-defpr-name>  “Simple” “Constant” <real-number>

For example, a device-rule statement to define a quantity based on a constant would be:

5, Y, DefineQuantity, Required_TSFC, Simple, Constant, 0.8

This statement would define the required thrust-specific fuel consumption of a jet engine to be 0.8.

### Defining a quantity based on the number of instances of a device type

A device-rule quantity can be set to the number of instances of a specific device type in a partial architecture.

<dr-quantity-simple-#dev>  ::=	<dr-id-act-defpr-name>  “Simple” “#Device_Instances” <device-name>

<device-name> ::= 		"Device_Name" <text-name-from-device-table>

For example, a device-rule statement to define a quantity based on the number of generator instances would be:

6, Y, DefineQuantity, nGenerators, Simple, #Device_Instances, Generator

### Defining a Quantity to be the Number of Device Instances in a Device Block

A device-rule quantity can be set to the number of device instances that are members of a specific device block in a partial architecture.   Device blocks are contiguous collections of device types that have something in common.  For example, if  PrimGen90 is a 90 kW primary generator, and PrimGen120 is a 120kW primary generator, then both of these devices could be in the block PrimGen, which includes all primary generators.

<dr-quantity-simple-#devblk>  ::=	<dr-id-act-defpr-name>  “Simple” “#Block_Instances” <device-blockname>

<device-blockname> ::= 		"Device_BlockName" <text-blockname-from-device-table>

For example, a device-rule statement to define a quantity based on the number of load device instances that are AC vs DC would be:

7, Y, DefineQuantity, nAC_Loads, Simple, #Block_Instances, AC_Load

### Defining a Quantity Based on a Device Parameters or Variables

A device-rule quantity can be set to the value of a specific device-type parameter or variable, or a function of the value of a specific device parameter across all device instances.  Device types are defined in the device table, along with parameters and variables that may apply to each device type.  Device parameters are specified in device-table columns.  The parameter values exist within that column, and correspond to the row’s device type.

<dr-quantity-expression>  ::=	<dr-id-act-defpr-name>  “Function” “Expression” <dev-expression-mathfn2>

| <dr-id-act-defpr-name>  “Function” “Expression” <dev-expression-devparvar>

<dev-expression-mathfn2> ::= 	<math-fn2>”.Devices." <dr-parvar>

<dev-expression-devparvar> ::= 	<dr-dev-name> “."<dr-parvar>

<dr-parvar> ::= 			<text-parameter-name-from-device-table>

| <text-variable-name-from-device-table>

<dr-dev-name> ::=		<text-name-from-device-table>

For example, a device-rule statement to define a quantity based on an a mathematical function applied to a specific device parameter of all of the device instances in the partial architecture would be:

8, Y, DefineQuantity, Max_Individual_Cruise_Load, Function, Expression, MAX.Devices.power

A device-rule statement to define a quantity based on a particular device type’s parameter value would be:

9, Y, DefineQuantity, PrimGen90_Power, Function, Expression, PrimGen90.power

## Example Device-Rule Statements

The following is a set of device-rule statements used as input to AEE to enumerate Layer 0 device-only architectures for an aircraft Electric Power System (EPS).

# Flow-Rule Statement Syntax

Since premises and rules use the same syntax, there are really only two types of statements:

<flow-rule-statement> ::=		<define-premise-or-rule> | <define-quantity>

## Defining Premises or Rules

There are multiple types of statements to define premises or rules.

<define-premise-or-rule> ::=	   <premise-rule-reachout> | <premise-rule-reduction>

| <premise-rule-filters> | <premise-rule-comparison> | <premise-rule-pairing>

### Increasing Dimensionality of Nested Vectors

Figure 1: A simple Electric Power System (EPS) example

PG  1

PG  2

APU 1

DC Load1

DC Load2

AC Load1

AC Load2

EP1

EP2

EP3

EP4

EP5

Nested premise vectors are created by a mechanism called “reach out.”  In AEE, if we start with a premise vector corresponding to the set of all device instances in a partial architecture, we can reach out from those devices to the set of all flows emanating from them.  For example, if we have 3 generator instances in the partial architecture: PG1, PG2, APU1, and various flows of electrical power leading from them to various loads: DCLoad1, DCLoad2, ACLoad1, ACLoad2, as shown in Figure 1,

There are 7 device instances of multiple device types, and 5 flow instances.  In this example, a premise vector describing the generators would be a PV(1) called Generators:

(1,1,1,0,0,0,0),

where the three 1s represent the 3 generators and the four 0s represent the 4 loads.  If we use the reachout mechanism to reach out from the generators, we can produce a PV(2) called Gen_Outflows:

((1,1,0,0,0),(0,0,1,0,0),(0,0,0,1,1),0,0,0,0).

This PV(2) has 7 terms at the 1st level, which correspond to the terms in Generators.  For each 1 in Generators, there is a nested vector of 5 terms, corresponding to the 5 flow instances.  For PG1, this nested vector is:

(1,1,0,0,0)

Since PG1 has EP flows out to DCLoad1 and DCLoad2.  Likewise, PG2 and APU1 each have their own nested vectors:

(0,0,1,0,0) and (0,0,0,1,1)

Corresponding to PG2’s EP flow out to ACLoad1, and APU1’s EP flows out to ACLoad1 and ACLoad2.

Reachout can extend both forward to consider flows leaving a device, or backward to consider flows entering a device.  Reachout can also start from flows.  For example, in figure 1, we can start with a PV(1) called EP_Flows:

(1,1,1,1,1),

Where the 1s correspond to the flows EP1, EP2, …, EP5.  If we use the reachout mechanism to reach out from these EP flows, considering the devices on their input ends, we can produce a PV(2) called EP_Flows_InDevs:

((1,0,0),(1,0,0),(0,1,0),(0,0,1),(0,0,1)),

where each EP flow has one input device, so there are 5 nested vectors, each with only one 1.  The first and second nested vectors correspond to EP1 and EP2, and they both  are:

(1,0,0),

since their input devices are PG1.

Reachout can extend multiple levels, for example starting with Generators, reaching out to create the nested vector of their outflows, Gen_Outflows, and then reaching out to the devices on those outflows output ends, to create the PV(3): Gen_Outflows_Loads:

(((0,0,0,1,0,0,0),(0,0,0,0,1,0,0),0,0,0),(0,0,(0,0,0,0,0,1,0),0,0),(0,0,0,(0,0,0,0,0,1,0),(0,0,0,0,0,0,1)),0,0,0,0).

At the deepest level in this PV(3), there are 5 nested vectors, corresponding to the 1s in the PV(2) Gen_Outflows.  Each of these 5 nested vectors has 7 terms, corresponding to the 7 device instances.  For example the first one is:

(0,0,0,1,0,0,0),

referring to the fact this output device is ACLoad1, which is the output of flow EP1, out from PG1.

The syntax for statements to define premise vectors via reachout is:

<premise-rule-reachout> ::=	<pr-reachout-flows-6> | <pr-reachout-devs-7>

<pr-reachout-flows-6> ::=		<id-act-defpr-name> <pv-pvname> <io-flows>

<pr-reachout-devs-7> ::=		<id-act-defpr-name> <pv-pvname> <io-devices>

<io-flows> ::=			"Input_Flows" | "Output_Flows"

<io-devices> ::=			"Input_Devices" | "Output_Devices"

where the following non-terminals are defined:

<id-act-defpr-name> ::=		<stmt-id> <y-n> <def-premrule-pvname>

All statements begin with a statement id, which is a sequence number

<stmt-id> ::=			<positive-integer-sequence-number-for-rules>

Next is a term to indicate whether or not the statement is active.  This is a practical consideration, since it allows the user to turn statements on and off to test and use the design rules for AEE.

<y-n> ::= 			"Y" | "N"

Next are two terms to indicate whether this statement is defining a premise or a rule.  If it is a rule, then its truth at its deepest level will be enforced.  If it is a premise, then it must have a name to be referenced in subsequent statements.  If it is a rule, this name is optional.

<def-premrule-pvname> ::= 	<def-rule> | <def-premise>

<def-rule>::=			"DefineRule" <pvname> | “Define Rule”

<def-premise> ::=		"DefinePremise" <pvname>

<pvname> ::=			<name>

<name> ::= 			<text-without-blanks>

Next are two terms to reference the premise vector argument for reachout.  The first is simply a keyword “PV” signaling that the next term is the name of a premise vector.  The next term can either be a premise vector defined in another statement, or it can be one of two keywords.  If it is “Partial_Devices” then the premise vector will be a PV(1) consisting of all 1s, with one term per device instance in the partial architecture.  If it is “Partial_Flows” then the premise vector will have one term per flow instance.

<pv-pvname> ::= 		"PV" <prior-pv>

<prior-pv> ::= 			<pvname> | "Partial_Devices" | "Partial_Flows"

### Reducing dimensionality of nested vectors

A nested premise vector can be reduced by applying a dyadic logical or mathematical function along the elements of its deepest dimension.  For example, if we begin with the vector describing the generators in the example above, it is a PV(1) called Generators:

(1,1,1,0,0,0,0),

where the three 1s represent the 3 generators and the four 0s represent the 4 loads.  If we use reduction with the AND logical function along its one and only dimension, we will get:

(1 AND 1 AND 1 AND 0 AND 0 AND 0 AND 0),

which results in a False.  Note that 1s represent True and 0s represent False here.  The meaning of this reduction in this example is to ask the question, “Are all of the devices generators?”  In this case there are 3 generators and 4 loads, so the answer is No.

The syntax for PV reduction is

<premise-rule-reduction> ::=	<pr-reduce-logic-8>

<pr-reduce-logic-8> ::= 		<id-act-defpr-name> <pv-pvname> "Reduce" <logic2> <target-depth>

where the following non-terminals are defined:

<logic2> ::=			"AND" | "OR" | "NAND" | "NOR" | "IF/THEN"

<target-depth> ::=		<non-negative-integer>

Target depth provides a degree of control to the user, since reachout doesn’t always add a new dimension.  For example, if we had constructed the PV(1): Interim_Result, it might have the following content for certain but not all partial architectures:

(0,0,0,0,0,0,0)

and if we applied reachout to it, we would get the same exact PV(1) as the result, though in general, we’d expect reachout from a PV(1) to produce a PV(2).  This situation-dependent uncertainty is why the target depth construct is provided, to introduce certainty back into the reduction operation.  The user specifies the target depth that he intends to reduce down to.  If the PV is already at that dimensionality, no action is taken (as in the case of the reachout from Interim_Result, above).  If the PV is at a higher dimensionality, the reduction is applied as many times as is necessary, each time at the deepest level, until the target depth is reached.

### Filtering a premise based on corresponding information

A premise vector can be filtered, at its deepest level, by checking a specific condition.

<premise-rule-filters> ::=		<pr-filter-dfname-1> | <pr-filter-dfblkname-2>

| <pr-filter-dfgrpname-3a> | <pr-filter-dfparvar-ineq-q-3>

| <pr-filter-dfparvars-ineq-4> | <pr-filter-logic-5>

#### Filtering by Device/Flow Type Name

A premise vector representing, at its deepest level, the device/flow instances in a partial architecture can be filtered by specifying a device/flow type name; the resulting premise vector’s deepest-level terms will be unchanged from the original premise vector’s terms for terms that correspond to device/flow instances with that name, and set to false (0) otherwise.

<pr-filter-dfname-1> ::=		<id-act-defpr-name> <pv-pvname> <df-name>

<df-name> ::= 			"Device_Name" <text-name-from-device-table>

| "Flow_Name"   <text-name-from-device-table>

So, in the example above, filtering based on a device name would enable creation of a PV(1): Primary_Generators, simply by starting from “Partial_Devices”:

(1,1,0,0,0,0,0).

The statement would be:

1, Y, DefinePremise, Primary_Generators, PV, Partial_Devices, Device_Name, PrimaryGenerator

#### Filtering by Device/Flow Block Name

Similarly, a premise vector can be filtered by specifying a device/flow block name; the resulting PV’s deepest level will be unchanged from the original premise vector’s terms that correspond to device/flow instances with that block name, and set to false (0) otherwise.  A block is a continguous collection of device types listed in the Device table, all listed there with the same block name.  Device block names are listed in one column within the Device table.

<pr-filter-dfblkname-2> ::= 	<id-act-defpr-name> <pv-pvname> <df-blockname>

<df-blockname> ::= 		"Device_BlockName" <text-blockname-from-device-table>

| "Flow_BlockName"   <text-blockname-from-flow-table>

For example, to create the PV(1): Generators, explained above, the statement would be:

2, Y, DefinePremise, Generators, PV, Partial_Devices, Device_BlockName, Generator

#### Filtering by Device/Flow Group Name

Similarly, a premise vector can be filtered by specifying a device/flow group name; the resulting PV’s deepest level will be unchanged from the original premise vector’s terms that correspond to device/flow instances that are members of that group, and set to false (0) otherwise.  A group is similar to a block, except that its member device types are not necessarily listed as a contiguous collection in the Device/Flow table.  In addition, it is specified differently within the Device/Flow table.  While one specific column in the Device/Flow table is always devoted to the list of all block names, each group is specified by an individual parameter column, with 1s as parameter values, vs. blanks or zeros for non-member device/flow types.

<pr-filter-dfgrpname-3a> ::= 	<id-act-defpr-name> <pv-pvname> <df-groupname>

<df-groupname> ::=		"Device_Group" <text-groupname-from-device-table>

| "Flow_Group"   <text-groupname-from-flow-table>

For example, to create a PV(1): Essential_Loads,

(0,0,0,1,0,0,1),

based on a device parameter “Essential”, in the example above, the statement would be:

3, Y, DefinePremise, Essential_Loads, PV, Partial_Devices, Device_Group, Essential

#### Filtering by Inequalities

##### Filtering by Inequalities between PVs

A premise vector can be filtered by specifying an inequality between it and another PV.  This syntax can also be used to enforce a design rule.

<pr-comp-pvpv-17> ::=		<id-act-defpr-name> <pv-specificpv> <ineq> <pv-specificpv>

| <id-act-defpr-name> "Comparison" <pv-specificpv> <ineq> <pv-specificpv>

<pv-specificpv> ::=		"PV" <pvname>

For example, we could ensure that all load with a single supply are powered by generators that supply only one load, by comparing the PV(1): Single_Source_Loads,

(0,0,0,1,1,0,1),

with the PV(1): Lds_Powd_by_1Load_Gens,

(0,0,0,0,0,1,0).

In the example above, the result would be the PV(1): Single_Supply_Consume_Lds,

(0,0,0,0,0,0,0),

since the only generator supplying only one load is PG2, and it powers DCLoad1, which is powered by both PG2 and APU1.

The statement to define this PV(1) would be:

4, Y, DefinePremise, Single_Supply_Consume_Lds, PV, Single_Source_Loads, =, Lds_Powd_by_1Load_Gens

The statement to use this comparison to enforce a rule would be:

5, Y, DefineRule, , PV, Single_Source_Loads, =, Lds_Powd_by_1Load_Gens

##### Filtering by Inequalities between Device/Flow Parameters/Variables and a Quantity

A premise vector can also be filtered by specifying an inequality between a device/flow parameter or variable, and a specific quantity.

<pr-filter-dfparvar-ineq-q-3> ::= 	<id-act-defpr-name> <pv-pvname> <df-parvar> <ineq> <quantity>

<df-parvar> ::= 			"Device_Parameter" <text-parameter-name-from-device-table>

| "Device_Variable"   <text-variable-name-from-device-table>

| "Flow_Parameter"    <text-parameter-name-from-flow-table>

| "Flow_Variable"       <text-variable-name-from-flow-table>

<quantity> ::=			"Constant" <realnumber-value>

| "Quantity" <qvname>

<qvname> ::=			<name>

<ineq> ::=			"<" | "<=" | "<>" | "=" | ">=" | ">"

For example, we could filter the PV(1): Essential_Loads above by specifying an inequality that the device’s Availability must be >= 0.999999.  This could produce a PV(1): Hi-Avail_Essential_Loads:

(0,0,0,1,0,0,0).

The statement to accomplish this filtering would be:

6, Y, DefinePremise, Hi_Avail_Essential_Loads, PV, Essential_Loads, Device_Parameter, >=, 0.999999

##### Filtering by Inequalities between two Device/Flow Parameters/Variables

A premise vector can also be filtered by specifying an inequality between two device/flow parameters or variables.

<pr-filter-dfparvars-ineq-4> ::=	<id-act-defpr-name> <pv-pvname> <df-parvar> <ineq> <df-parvar>

For example, we could filter the PV(1): Essential_Loads above by specifying an inequality that the device’s Actual_Availability (a device variable computed by examining the availability of sources and transmission paths) must be >= its Required_Availability (a device parameter based on the role of the load in this particular mission).  This could produce a PV(1): Avail_Loads:

(0,0,0,0,0,0,1).

The statement to accomplish this filtering would be:

7, Y, DefinePremise, Avail_Loads, PV, Essential_Loads, Actual_Availability, >=, Required_Availability

#### Filtering by Logical Functions

One, two or three premise vectors can also be filtered by applying a logical (Boolean) function (monadic, dyadic or triadic) between them.

<pr-filter-logic-5> ::=		<id-act-defpr-name> <pv-specificpv> "Function" <logic1>

| <id-act-defpr-name> <pv-specificpv> "Function" <logic2> <pv-specificpv>

| <id-act-defpr-name> <pv-specificpv> "Function" <logic3> <pv-specificpv>

<pvname>

<logic1> ::=			"NOT"

<logic2> ::=			"AND" | "OR" | "NAND" | "NOR" | "IF/THEN"

<logic3> ::=			"IF/THEN/ELSE"

For example, we could find the PV(1): Non-Essential_Loads above by using the monadic NOT function.  The statement to accomplish this filtering would be:

8, Y, DefinePremise, Non_Essential_Loads, PV, Essential_Loads, Function, NOT

### Comparisons between Quantities

Most rules are created as comparisons between quantities.  These comparisons could also be used to define premise vectors:

<premise-rule-comparison> ::=	<pr-qvq-ineq-19> | <pr-qvqv-ineq -18>

#### Comparing a QV with a Quantity

A quantity vector can be compared with a quantity:

<pr-qvq-ineq-q-19> ::=		<id-act-defpr-name> <qv-specificqv> <ineq> <quantity>

| <id-act-defpr-name> "Comparison" <qv-specificqv> <ineq> <quantity>

For example, we could find the generators with only one essential load, defining a PV(1): Gens_w/1_Ess_Load,  by comparing the QV(1): nEss_Loads_per_Gen,

(2,0,1,0,0,0,0),

with the constant: 1.  The statement to accomplish this filtering would be:

9, Y, DefinePremise, Gens_w/1_Ess_Load, QV, nEss_Loads_per_Gen, =, Constant, 1

#### Comparing a QV with a QV

A quantity vector can also be compared with another QV:

<pr-qv2pv-ineq-qv-18> ::= 	<id-act-defpr-name> <qv-specificqv> <ineq> <qv-specificqv>

| <id-act-defpr-name> "Comparison" <qv-specificqv> <ineq> <qv-specificqv>

<qv-specificqv> ::=		"QV" <qvname>

For example, we could ensure that all generators’ available capacity meets or exceeds the power draw of their essential loads, defining a PV(1): Gens_w/_Capacity,  by comparing the QV(1): Gen_Capacity,

(90.0,90.0,65.5,0,0,0,0),

with the QV(1): Gen_Ess_Ld_Power,

(84.2,82.0,45.5,0,0,0,0).

The statement to create a PV(1) from this filtering would be:

10, Y, DefinePremise, Gens_w/_Capacity, Comparison, QV, Gen_Capacity, =, QV, Gen_Ess_Ld_Power

The statement to enforce this rule would be:

11, Y, DefineRule, , Comparison, QV, Gen_Capacity, =, QV, Gen_Ess_Ld_Power

### Flow Pairing

A premise vector can also be created, or a rule enforced, by checking whether the collection of relevant flows have certain attributes, called their “pairing”:

<premise-rule-pairing> ::=		<pr-pv2yn-pairing-10.5>

<pr-pv2yn-pairing-10.5> ::=    	<id-act-defpr-name> <pv-specificpv> "Pairing"

<pairing-direction> <pairing-pattern>

<pairing-direction> ::=		"Low_to_High" | "High_to_Low"

<pairing-pattern> ::=		"Sequential" | "Concentric"

For example, for a jet-engine application, we could enforce a rule that the shaft-power flows from turbines to compressors go in a specific direction and are concentric (nested pairing).

The statement to enforce this rule would be:

12, Y, DefineRule, , PV, SP_Flows_from_T_to_C, Pairing, High_to_Low, Concentric

### User-Defined Filtering

A premise vector can also be created, or a rule enforced, by checking whether the collection of relevant flows satisfy certain criteria, according to an external function call:

<premise-rule-user-defined> ::=	<id-act-defpr-name> <pv-specificpv> <user-defined-function><udf-arg-list>

<user-defined-function> ::=	<text-name-of-an-external-function-call>

<udf-arg-list> ::=			<comma-separated-text-list-of-arguments-to-user-defined-function>

For example, for an electrical circuit partial architecture, we could enforce a rule that prohibits electrically “shorted” components (those with a direct very low-impedence path across their leads).

The statement to enforce this rule would be:

13, Y, DefineRule, , PV, Electical_Circuit_Flows, No_Shorts, ,

## Defining Quantities

There are also multiple types of statements to define quantities.

<define-quantity> ::=		<quantityv-defn> | <quantityv-reduction>

### Mathematical Functions between Quantities

A quantity vector can be created using monadic or dyadic mathematical functions:

<quantityv-defn> ::=		<q-parvar2q-11> | <q-qv-math-12> | <q-qvq-math-13>

<q-qv-math-12> ::=		<id-act-defq-name>  <qv-specificqv> "Function" <math1>

| <id-act-defq-name>  <qv-specificqv> "Function" <math2> <qv-specificqv>

<id-act-defr-name> ::=		<stmt-id> <y-n> <def-rule> <pvname>

<id-act-defq-name> ::=		<stmt-id> <y-n> <def-quantity> <qvname>

<id-act-defq-vname> ::=		<stmt-id> <y-n> <def-quantity> <text-variable-name-from-flow-table>

<q-qvq-math-13> ::=		<id-act-defq-name>  <qv-specificqv> "Function" <math2> <quantity>

<def-quantity> ::= 		"DefineQuantity"

<math1> ::=			"SIN" | "COS"

<math2> ::=			"+" | "-" | "*" | "/" | "MIN" | "MAX"

### Device or Flow Parameters and Variable Quantities

A device/flow parameter or variable quantity can be used to define a QV.

<q-parvar2q-11> ::=		<id-act-defq-name>  <pv-pvname> <df-parvar>

### Device or Flow Parameters and Variable Quantities

A quantity vector can be reduced in dimensionality to create another lower-dimensionality QV by reduction using a mathematical function.

<quantityv-reduction> ::=		<q-reduce-math-14>

<q-reduce-math-14> ::=		<id-act-defq-name>  <qv-specificqv> "Reduce" <math2> <target-depth>

## Assigning Quantities to Variables

A quantity vector’s or premise vector’s terms can be assigned to the corresponding device/flow instances’ variable values.

<quantity-to-parvar> ::=		<q-qv2var-15> | <q-pv2var-16>

<q-qv2var-15> ::=		<id-act-defq-vname> <qv-specificqv> "QV_to_Var"

<q-pv2var-16> ::=		<id-act-defq-vname> <pv-specificpv> "PV_to_Var"

## Example Flow-Rule Statements

The following is a set of flow-rule statements used as input to AEE to enumerate Layer 1 full-system architectures for an aircraft engine.

Delivered to the Government in Accordance with Contract FA8650-10-C-7080