| META II Project Memo |  | META II CM 11-02 | 30-Mar-11 |
| --- | --- | --- | --- |
| Title: | TMS Model Description |  |  |
| Author: | Chuck Lents, Blake Moffitt |  |  |
| To: | Team Distribution |  |  |
| Ref: |  |  |  |

# Introduction

This model predicts the performance of the generic baseline fuel based thermal management system (TMS) shown in Figure 1.  This system uses fuel as the primary heat sink.  Fuel is taken out of the tank, routed through various aircraft system load heat exchangers, driven by a main fuel pump (MF Pump) which provides the pressure rise necessary to move fuel through the main loop and pressurize the main engine fuel nozzle.  The actuation pump (Act Pump), which further boost fuel pressure, moves fuel in a recirculating loop, supplying engine actuators with flow.  This pump is sized for the worst case actuator demand flow, thus at most operating points it provides more fuel flow than is required by the actuators.  The excess flow is returned to the main fuel loop downstream of the main fuel pump.  The flow exiting the engine actuators is returned to the main fuel loop upstream of the main fuel pump.  After the split to the actuator pump, the main loop supplies fuel to the engine oil cooler (Fuel Oil Cooler).  The fuel is then burned in the engine (To Gas Generator).

Figure 1: Generic TMS Schematic

If the fuel temperature after the engine oil cooler (Fuel Oil Cooler)exceeds the maximum allowable fuel temperature, excess flow is taken from the tank.  After the engine oil cooler, the excess flow is returned back to the tank, while the burn flow goes to the gas generator.  The return fuel is cooled back to tank temperature by the air fuel cooler.  This fuel cooler is supplied with outside ram air through a ram air scoop.  During ground operations a fan pulls air through the scoop and fuel cooler.

The acronyms used in the Figure 1 schematic are provided below:

ECS – Environment Control System:  ECS heat is added to the TMS at this heat exchanger

Hydraulics: Hydraulic oil heat is added to the TMS at this heat exchanger

Gen – Generator: generator oil heat is added to the TMS at the heat exchanger

MF Pump – Main Fuel Pump: This pump supplies fuel to the main engine combustor

AB Pump – Afterburner Pump: This pump supplies fuel to the engine afterburner used during high speed flight

Act Pump – Fuel Actuation Pump: This pump boost the pressure supplied by the MF Pump for fuel flow used in the engine actuators

Fuel Oil Cooler: This heat exchanger cools engine oil with fuel

MFTV – Main Fuel Throttle Valve: This valve controls flow to the engine combustor and to the return to tank loop.

# System Model

This system is modeled in the Matlab m-file “BaselineTMS”.  This model is comprised of component models for each of the components described above and essentially hooks the output from one component to the input of the next.  The input to this model is the structured variable “System”, which is also returned as the output.  System must contain a “Conditions” structure “System.Conditions” and a “Param” structure “System.Param”.  The “Conditions” structure holds the fixed environmental inputs to the “BaselineTMS” model, while the “Param” structure holds the variational parameters whose values must be determined by an equation solver.  These variational parameters are primarily needed to close the implicit loops in the model.  At the end of the “BaselineTMS” model a series of equality constraints are calculated.  These constraints must be driven to zero (within some convergence tolerance) by the equation solver as it varies the variational parameters (detail on the variation parameters and equality constraints will be discussed later).

The “Conditions” structure is created by the Matlab m-file “TMSMission”.  This fill creates a structured cell array variable “Conditions”.  The cell array has dimensions equal to the number of mission points to be analyzed.  For example Conditions{1}.Altitude holds the altitude at mission point 1, while Conditions{9}.Gen.Heat holds the generator heat load at mission point 9.  All of the input variable held in “Conditions” (standard SI units) are provided below:

Conditions{i}.MPNumber	- mission point number

Conditions{i}.MissionPoint 	- mission point description.

Conditions{i}.Altitude	- altitude

Conditions{i}.Mach	- mach number

Conditions{i}.Duration	- mission point duration

Conditions{i}.DayType	- temperature delta above standard day temperature

Conditions{i}.Tank.Temp	- temperature of fuel delivered out of the fuel tank

Conditions{i}.GGFlow	- fuel flow rate needed for the engine combustor (burn flow)

Conditions{i}.ABFlow	- fuel flow rate needed for the afterburner

Conditions{i}.EngActFlow	- fuel flow rate needed for engine actuation

Conditions{i}.N2	- engine high spool speed

Conditions{i}.ECS.Heat	- ECS heat load

Conditions{i}.ECS.DeltaP	- Pressure drop across ECS heat exchanger

Conditions{i}.'Hyd.Heat	- Hydraulic oil heat load

Conditions{i}.Hyd.DeltaP	- Pressure drop across hydraulic oil heat exchanger

Conditions{i}.'Gen.Heat	- Generator oil heat load

Conditions{i}.Gen.DeltaP	- Pressure drop across generator oil heat exchanger

Conditions{i}.'EngOil.Heat	- Engine oil heat load

Conditions{i}.EngOil.DeltaP	- Pressure drop across engine oil heat exchanger

Conditions{i}.'EngOil.Flow	- Engine oil flow rate (not used)

Conditions{i}.'EngOil.MaxTemp	- Engine oil max temperature (not used)

Conditions{i}.Tank.Fluid	- Fuel type descriptor

Conditions{i}.Tank.Press	- Pressure of fuel out of fuel tank

Conditions{i}.MaxFuelTemp	- Max allowable fuel temperature    Conditions{i}.MFPumpGearRatio	- Gear ratio between engine N2 speed and MF Pump speed

Conditions{i}.ABPumpGearRatio	- Gear ratio between engine N2 speed and AB Pump speed

Conditions{i}.ActPumpDeltaP	- Actuator pump pressure rise

Most of these values are imported into the “Conditions” structure within the TMSMission code by a command called “ImportCSV”.  This command imports the inputs contained in the file “TMSInput.csv” into the “Conditions” structure.

## Solving for system state points (pressure, temperature and flow rate

The “BaselineTMS” model must be driven by a non-linear equation solver to find a converged solution.  The variational parameters, which must be iterated on by the equation solver and contained in the “System.Param” structure, are described below and shown in the schematic Figure 2

System.Param.EngActTempOut	- temperature at the exit of engine actuation load (Tv1)

System.Param.ActPumpRecircTemp	- temperature in the act pump recirculation loop (Tv2)

System.Param.ActPumpRecircFlow 	- flow rate in the actuator pump recirculation loop (Mv2)

System.Param.RTTFuelFlow	- return to tank fuel flow rate (Mv3)

Initial guess and bounds for the variational parameters are provided below:

initial guess	min				max

EngActTempOut:		400.00		TMS.Conditions.Tank.Temp	2 * TMS.Conditions.Tank.Temp

ActPumpRecircTemp:	400.00		TMS.Conditions.Tank.Temp	2 * TMS.Conditions.Tank.Temp

ActPumpRecircFlow:	0.5		0.01				1

RTTFuelFlow:		0.2		0.0				2.0

The equation solver must drive the following equality constraints to zero.  The locations in the model where equality constraints are enforced are shown on Figure 2.

1) System.Opt.EqConstr(1) – difference between Tv1 and Te1

= abs(System.Param.EngActTempOut - System.EngAct.Outlet.Temp)/50

2) System.Opt.EqConstr(2) – difference between Tv2 and Te2 =abs(System.Param.ActPumpRecircTemp   -System.ActPump.Outlet.Temp)/50

3) Sys.ActPump.Bnd.Res – difference between actuation pump inlet and exit flow rate

calculated within the pump component model

4) System.Opt.EqConstr(3) – difference between Te3 and the max allowable fuel temperature abs(System.MFTV.Outlet{2}.Temp-System.Conditions.MaxFuelTemp)/50

Figure 2: TMS Model Variation Parameters and Equality Constraints

The model outputs the pressure, temperature and flow rate at each state point around the loop, pump power and the heat that must be removed at the air fuel cooler.

Not all of the variational parameters and equality constraints are in force for all operating modes of the “BaselineTMS” model.  A sample script file “NoHEATSTMSScript” has been created to demonstrate execution of the TMS analysis.

First the “Conditions” structure is created.  The first step is to size the system pumps.  The mode is set to design (Sys.Opt.Model = 'size').  The desired pump pressure rise for the MF Pump and AB Pump is set by

Sys.ABPump.Pars.DeltaP = ConvertUnits('psia', 1000.0, 'import');

Sys.MFPump.Pars.DeltaP = ConvertUnits('psia', 1000.0, 'import');

In sizing mode there is no fuel return to tank (fuel burn flow is very high thus the fuel will not over heat) and there in no excess actuation flow.  Thus only variational parameters Tv1 and Tv2 are used along with equality constraints 1 and 2

The maximum fuel flow rates are contained in the pump design case mission point 17 (mission point “D3”).  Thus “Sys.Conditions” is set equal to “Conditions{17}”.  The equation solver must then find a converged solution for Tv1 and Tv2.  The converged solution is then stored in “TMSDesign”.

With the pumps designed, the model is now run in performance mode, with no return to tank flow (Sys.Opt.Model = 'performanceRTToff').  The variational parameters are now Tv1, Tv2 and Mv2 and the equality constrains 1, 2 and 3 are used.  For each mission point Sys.Conditions” is set equal to “Conditions{i}” and the equation solver must find a converged solution for Tv1, Tv2 and Mv2.  The converged solution is then stored in “TMS{i}”.

The final step is to rerun the model for cases where the Te3 (TMS{i}.EngOil.Outlet.Temp) violated the maximum fuel temperature constrain.  For these cases all four variational parameters and equality constraints are active.  For each mission point to be rerun, Sys.Conditions” is set equal to “Conditions{i}” and the equation solver must find a converged solution for Tv1, Tv2, Mv2 and Mv3.  The converged solution is then stored in “TMS{i}”.

The above procedure essentially provides the unique solution for the baseline TMS state points.  Pump power and heat rejection required at the air fuel cooler is also predicted. The Matlab m-file “ModelScript performs the above non-linear equation solver functionality in the UTRC HEATS environment.

## Optimizing ram air flow and heat exchanger pressure drop

With the state points solved, the air fuel cooler and fan can be sized.  However, the size of these components depends on the air flow and pressure drop through the air fuel cooler heat exchanger.  Low air flow and pressure drop drives heat exchanger weight up, but results in low fan power and weight.  The opposite is true with high air flow and pressure drop.  Thus there are optimal values of these parameters that minimize the combination of weight and power draw.

The second half of the ModelScript file contains code for sizing the system air fuel cooler and fan based on the value of three optimization parameters. They are:

RTTCoolerPinchTemp	– temperature difference between the fuel inlet and air exit

RTTCoolerColdSideDP	– air side pressure drop

RTTCoolerHotSideDP	– fuel side pressure drop

The RTTCooler Pinch Temp parameter essentially controls the air side flow rate.  Consider Figure 3.  As the pinch temperature increases, the air exit temperature decreases as does the air temperature rise.   With less temperature rise in the air for the same heat transferred for the fuel to the air, the air flow rate must increase.  It is better to optimize the RTTCoolerPinchTemp than the air flow rate directly, as a value for the air flow rate could be selected by the optimizer that is too low to transfer all the heat.

Figure 3: Air Fuel Cooler Pinch Temperature

The limits for the three optimization parameters are provided below

RTTCoolerPinchTemp		0.0	T Fuel In – T Air In

RTTCoolerColdSideDP		0.0	2 .0 (psia)

RTTCoolerHotSideDP		0.0	30.0 (psia)

This ooptimization problem is set up in ModelScript, after the comment:

% size ram circuit components (ram scoop, fan, fuel cooler) at hot day ground condition

An initial guess for the three optimization parameters is provided.  The code then selects operating point 15, hot day ground idle, as the sizing point.  The ambient conditions are set, the heat exchanger is sized and finally the fan is sized.  The three outputs of interest are:

TMS{15}.Fan.Pars.Power	– fan power

TMS{15}.Fan.Mass.Total	 - fan weight

TMS{15}.RTTCoolerHX.Mass.Total;	- air fuel cooler weight

The total system shaft power (pumping power), electrical power (fan) and weight are summed in the structures TMS{i}.Sys.PowerShaft, TMS{i}.Sys.PowerElec,  and TMS{i}.Sys.Mass/200.  Two penalty functions are formed from these values.  The performance penalty function (TMS{i}.Sys.PenaltyP) accounts for the affect of power and weight on aircraft fuel burn.  The cost penalty function is (TMS{i}.Sys.PenaltyC).

The optimizer needs to find the optimal values of RTTCoolerPinchTemp, RTTCoolerColdSideDP, and RTTCoolerHotSideDP which minimize TMS{15}.Sys.PenaltyP and  TMS{15}.Sys.PenaltyC  ( the value of the penalty functions at the 15th operating point, hot day ground operation)