META-II Report

1. Vehicle Selection and Set-up

One of Pacelab SysArc’s key calculation capabilities is designing an entire aircraft. Starting with the selection and modification of an aircraft configuration and applying typical design requirements in all major areas such as geometry, flight performance, weights and aerodynamics, the resulting aircraft can then be used as a basis for systems architecture investigations, as it is done in the course of the META-II project.

In order to be free of any proprietary data the following fictive UAV has been designed, using only publically available data:

Its configuration and shape is driven by given requirements to be able to fly at super-sonic speed and carry a certain payload over a certain distance/duration. Its main characteristics are

MTOW = 4800 kg

Payload = 200 kg

Mission Range = 1860 NM

Mission Duration = 12 h

Left Wing

Right Wing

Right EPS Compartment

Avionics Bay

Left EPS Compartment

Ceiling

TMS Compartment

Compartments

In order to investigate the UAV‘s internal heat flow - as a result of the components‘ inefficiency and their heat emission - the vehicle‘s interior geometry space has been divided up into dedicated volumetric zones referred to as “Compartments“. By simply placing a component (or load) inside of it, the application automatically updates these “Compartment-Component-Associations“ (see graph-network on the right) resulting (in this case) in an automatic heat accumulation over all involved objects inside of the compartment. Those properties are key for further analysis and design, such as cooling systems or ECS. In the course of this project, seven compartments as depicted in the picture below were used:

Once the outer shape is defined and/or calculated, three-dimensional spaces/volumes (so called “Compartments”) inside of the given vehicle geometry are defined. Pacelab SysArc has the capability to automatically detect whenever the user places a component inside of such a compartment and performs – based on those component-compartment-associations – corresponding calculations such as heat summation over all components inside each compartment for example. Those emitted waste heat loads play a key role in the architecture investigation since they determine the necessary performance of a heat exchanger for example.

2. Models for EPS, TMS, Engine, Mission

Electrical Power System (EPS)

All EPS components are going to be modeled using Thévenin’s theorem which states that any combination of resistors, electric current and voltage sources is electrically equivalent to a single voltage source and a single series resistor.

The entire EPS architecture is modeled/assembled in a so called “EPSDefinition”-Graph which – in accordance with physical laws – automatically propagates the electric current I upstream and the voltage V downstream; supporting three different voltage classes: 115VAC, 28VDC and 270VDC represented by different port types.

The following table gives a descriptive overview of all the different types of EPS components used in the course of this META-II project:

| Type | Given by component | Given by architecture | Ports | Formulas |
| --- | --- | --- | --- | --- |
| Source | nominal voltage Vnom resistance R efficiency η | electric current I | shaft input electrical output | Vout=Vnom-R∙I Pout=Vout∙I Pheat=1-η∙Pout Pin=Pheat+Pout |
| Load | resistance R efficiency η | input voltage Vin | electrical input | I=VinR Pin=Vin∙I Pheat=1-η∙Pin |
| Switch | resistance R efficiency η | input voltage Vin electric current I | electrical input electrical output | ΔV=R∙I Vout=Vin-ΔV Pout=Vout∙I Pheat=ΔV∙I Pin=Pheat+Pout |
| Converter | (nom.) output voltage Vout resistance R efficiency η | electric current I | electrical input electrical output | Pout=Vout∙Iout Pin=Poutη Iin=PinVin Pheat=1-η∙Pin |
| Trans-mission Line (Cable) | length l linear resistance ϱ | input voltage Vin electric current I | electrical input electrical output | R=l∙ϱ ΔV=R∙I Vout=Vin-ΔV Pheat=ΔV∙I Pout=Vout∙I Pin=Pheat+Pout |

Thermal Management System (TMS)

For modeling all TMS components it is assumed that simple physical relations of the complexity level that fits the ideal gas are okay for this phase of the project

The entire TMS architecture is modeled/assembled in a so called “TMSDefinition”-Graph which – in accordance with physical laws – automatically propagates the medium’s mass flow m upstream and the medium’s temperature T, pressure p and massic heat capacity cp downstream; supporting one generic TMS port type.

The following table gives a descriptive overview of all the different types of TMS components used in the course of this META-II project:

| Type | Given by component | Given by architecture | Ports | Formulas |
| --- | --- | --- | --- | --- |
| RamAir | cross-section area A | Heat exchanger cold side | TMS input TMS output | TMSin=atmosphere |
| Heat Exchanger | effectiveness η cold side pressure change ∆pc hot side pressure change ∆ph | Heat Load Q cold side input pressure pin,c cold side input temperature Tin,c cold side output mass flow mout,c hot side input pressure pin,h hot side input temperature Tin,h hot side output mass flow mout,h | TMS cold input TMS cold output TMS hot input TMS hot output | ∆Tc=Qmc∙cp,c Tout,c=Tin,c+∆Tc ∆Th=∆Tcη Tout,h=Tin,h+∆Th pout,h=pin,h+Δph pout,c=pin,c+Δpc |
| FuelTank | - | input pressure pin input temperature Tin output mass flow mout | TMS input TMS output | TMSin = TMSout (no losses) |
| HeatLoad | temperature change ∆T | Heat Load Q input pressure pin input temperature Tin output mass flow mout | TMS input TMS output | Q=Qcompartment ∆T=Qm∙cp Tout=Tin+ΔT |
| Pump | pressure change ∆p | input pressure pin input temperature Tin output mass flow mout | TMS input TMS output shaft input | Pshp=mρ∙η pout=pin+Δp Pheat=1-η∙Pshp |
| Splitter | - | input pressure pin input temperature Tin output mass flow mout1 and mout2 | TMS input TMS output1 TMS output2 | min=mout1+mout2 |
| Trans-mission Line (Pipe/Duct) | length l | input pressure pin input temperature Tin output mass flow mout | TMS input TMS output | TMSin = TMSout (no losses) |

Engine

The engine component is modeled as a special type of an EPS-source. It has a mechanical shaft-power output port in order to provide shaft-power to a connected generator and a TMS input port to provide the fuel. One of the most important aspects that is going to be investigated is the additional fuel consumption due to shaft-power and bleed-air extraction by a given system architecture. In order to allow the user to perform this task a dedicated multidimensional data-table is provided with the engine that computes “thrust (∆T) and fuel-flow (∆FF) penalties” as a function of (next to ISA-Deviation, Altitude and Mach-Number) drawn shaft-power and bleed-air. The total thrust T and fuel-flow FF for a given point during flight is therefore calculated as follows:

T=Tplain∙1+∆T

FF=FFplain∙1+∆FF

The “plain” values are considered as values without any existing architecture and originate out of the underlying aircraft predesign tool Pacelab APD.

Flight Performance – Mission Profile

In order to successfully design an aircraft a so called “Design Mission” is used so that one of the most important functional relations in the course of airplane design can be solved:

MTO=MOEW+MPAX+MFUEL

Having the payload MPAX given and estimating the Operating Empty Weight MOEW based on the structure and the Takeoff Weight MTO, a subjacent Design Mission is used to compute the necessary fuel MFUEL so that the “Mass-Performance-Loop” (before mentioned equation) can be closed iteratively.

This Design Mission itself is broken down into two flight profiles (inbound and outbound), both with its dedicated function and further breakdown into so called flight segments. The following picture gives a schematic overview of the entire mission with its profiles and segments:

The same topology approach is used for any other defined mission in Pacelab SysArc: one mission can have as many flight profiles as desired which on their part can consist of again as many flight segments as desired. Pacelab SysArc provides a comprehensive library of predefined flight segments and assembling them to such a non-trivial mission can be accomplished very easily by simply connecting them interactively with their ports in the desired order (blue lines) and is a standard functionality of the software. Adding additional constraints (here for example assuring the distance flying outbound equals the distance flying inbound) is applied in the same fashion (green line).

3. Architecture Instance

Building up and investigating an architecture in Pacelab SysArc can be done either fully automatically or manually interactively (or a mixture of both techniques). In both cases the following steps need to be performed:

- Creating desired component instances out of the repository and placing them in dedicated graph views, e.g. “PrimaryGenerator1” in “EPSDefinition”, “HeatExchanger1” in “TMSDefinition”, …

- Establishing logical connections between components using their ports, e.g. “MainLeftACBus <-> LeftOuterActuatorLoad”, …

- Assigning pathways to those connections as possible locations for transmission lines (cables/pipes)

- Trigger automatic routing mechanism (finding the shortest distance)

- Data post-processing, e.g. determine power requirement, fuel consumption, …

In the course of this META-II project the following diagram-view exemplifies a simplified architecture that contains the TMS (green components on the left hand side) and the EPS (orange components on the right hand side) of our UAV:

Placing all those components inside the vehicle geometry (see left picture below) leads to another graph that depicts the previously mentioned component-compartment-association (see right picture below) which is used

to track down how much total waste heat load is accumulated by each components and impacts therefore the performance of your cooling system (heat exchanger).

4. Results

Enumerating through a list of different architectures (that still satisfy the same bus load), it is now of special interest to see which architecture (i.e. combination of components) not only meets the design requirements but also works in the most efficient way in terms of for example heat emission, power and voltage losses etc. to finally draw conclusions about flight mission properties such as distance, duration, fuel consumption etc. The following list describes exemplarily the “gains and penalties” for three different architectures.

| Operating Condition | Distance | Duration | Fuel Consumption |
| --- | --- | --- | --- |
| 2 Generators, 1 fails | +0.6% | +0.4% | +0.9% |
| 4 Generators, all working | -1.2% | -0.9% | +0.6% |
| 4 Generators, 2 fail | -1.3% | -1.2% | +1.1% |