META II Final report (Task 1)

Contents

2	Challenge problem definition	3

2.1	Informal description of the mission requirements	4

2.1.1	Vehicle assumptions	5

2.1.2	Details of the mission phases	5

2.2	Baseline system architecture	7

2.3	Formal definition of the mission requirements	8

3	Component models	10

3.1	Thermal management system	10

3.1.1	Schematic Representation	11

3.1.2	Detailed component models	12

3.1.3	Abstract component models	12

3.2	Electric power system	14

3.3	Engine	14

3.4	Software	14

3.4.1	Decision Diagram Based Complexity of Software Models	14

4	Contract based System Engineering	18

4.1	General definition of contracts	18

4.2	19

4.3	A language for contract specification	20

4.3.1	Syntax	20

4.3.2	Semantics	23

4.4	Examples of contracts	26

5	Definition of a design flow	26

5.1	Definition of an abstraction layer	26

5.2	Supporting tools for the design activity	28

6	Abstraction layers design	28

6.1	General criteria for the selection of abstraction layers	29

6.2	Definition and computation of abstract models	30

6.3	Application to thermal management	30

7	Design flow for the challenge problem	31

# Challenge problem definition

The design methodology and tools to be developed under the DARPA META 2 contract will provide significant reduction of design cost and schedule for future aircraft over traditional methods. This novel design flow will be demonstrated on a challenge problem that encompasses the most critical sub-systems that constitute an aircraft: the propulsion system (i.e. the engine), the electric power system, the thermal management system, and the control and communication platform that are important elements for more-electric aircraft. The high level diagram of the prototypical aircraft is shown in Figure 1.

Figure 1: High level diagram describing the challenge problem

We start from an abstraction level where the system can be decomposed into 4 sub-systems. The propulsion system comprises the engine as its main component. The engine exposes several interfaces including the control interface, the fuel interface, the mechanical shaft interface, and the heat interface – that can be refined into an oil interface to be connected to the thermal management system. We consider generators as part of the electric power sub-system. Generators are connected to the engine shaft and provide electric power to the aircraft. Storage elements might be present on board of the aircraft. The thermal management system uses several heat sinks to reject the heat produced by the engine, by the electric power system and by the environmental control system. The control sub-system includes also the embedded control platform that supports the execution and interaction among control algorithms distributed on the aircraft.

The objectives of this study are the followings:

- Determine what are the requirements driving  the complexity of these sub-systems

- Define a set of requirements at the system level that will be used as reference for a design flow which will demonstrate the advantages of the proposed methodology. These requirements will be such that the system is expected to be complex, meaning that the solution is not obvious to expert designers.

- For what possible, partition the requirements over the 4 subsystems. For example, the weight of the aircraft together with the type of missions to be executed will provide inputs to determine the maximum thrust that the engine(s) should provide.

The derived requirements will be used as the input for the sub-sequent refinement steps leading to an optimal design.

## Informal description of the mission requirements

A mission defines the abstracted expected dynamic behavior of an aircraft. It can be captured in terms of mission phases (e.g. take-off, ascend, fast descend etc.) and transitions among them. Notice that in general not all transitions are necessarily allowed. Each phase is characterized by a set of requirements for the aircraft. Some of them are imposed by the mission itself. For example, the aircraft might be required to execute short take-offs (within a given maximum distance), or it might be constrained to climb at a certain rate so that a target altitude can be reached is a relatively short time.  Each phase can also be characterized by quantities such as flight time, availability and safety requirements, or more generally with probabilities that a mission phase can be safely accomplished.

The mission that we selected for this challenge problem comprises two major segments: an Intelligence, Surveillance and Reconnaissance (ISR) segment, and a high speed strike segment. These two segments impose very different requirements on the components of the aircraft. At high altitude loiter the TMS will have a large low temperature heat sink, while during strike operations only a high temperature sink will be available.  Thus the TMS will have to adapt to a radically changing environment.  The EPS will have to provide power at high altitude where engine power available to drive a generator may be limited, and will be subject to a rapidly changing load characteristic and engine shaft speed during strike operations. This leads to a relatively complex EPS & TMS with a rich set of potential technology and component options.

The proposed aircraft would allow performing this type of missions with high accuracy (i.e. with high probability of begin able to identify and engage a target). Today, two vehicles are used. One vehicle constantly monitors a zone from high altitude. If targets are identified, then another vehicle is required to intervene. Besides logistic problems of having several types of vehicles, the success rate is low because the target might move from the position where it was first identified.

For the ISR segment, endurance is the most important concern. Endurance is achieved by running the aircraft in a very efficient region where the engines consume very little fuel (they run almost idle). Typically, this is achieved with high-bypass engines. The electrical power system will have to provide the necessary power for sensors which might be very high (laser sensors and radars). The thermal management system can rely on cold outside air (although air density is low and fuel flow rate is low as well).

In the strike mode, the aircraft has to reach a very high speed. In this regime, the engines will have to provide very different performance. For a safe strike maneuver, the speed of the aircraft needs to be very high (Mach 1.5 or above) which requires low bypass engines. The electrical power system and the thermal management system will have to switch to a different regime. In particular, the TMS will now have to provide a very high fuel rate and reject high heat with minimal availability of heat sinks (due to high speed and low thermal signature requirements). Notice that a design of the aircraft user the two regimes might not be the best solution. In fact the heat accumulated in the fuel after a strike might be rejected during a sub-sequent high altitude flight. This makes the design tools in use today inadequate since typically only steady state conditions are considered.

The requirements being very different in the two phases poses challenges in the selection of an optimal architecture because it is difficult to have mechanical parts (and electric parts) that are efficient and high performance at the same time. For example, gear pumps are more efficient at low flow rates, but generate a large amount of heat at high flow rates, whereas variable volume pumps don’t. Thus, one might end up selecting hybrid architectures with both components that are activated depending on the mission to be performed. This makes the system more complex.

Figure 2: Detailed description of the mission phases.

### Vehicle assumptions

We will assume that the engine has been selected, meaning that there exists an engine capable of delivering a certain level of performance required for the mission. We also assume that the airframe is given and that the payload has been fixed. We fix the following high level properties of for the aircraft that will set the context of the challenge problem:

- Take-off Gross Weight (TOGW) 1360 kg (3000 lbm)

- Mission endurance 10 hr

- Maximum altitude 15.24 km (50 kft)

- Cruise speed 0.75 Mn

- Strike speed 1.5 Mn

- Payload  900 kg (2000 lbm)

- Low observability

### Details of the mission phases

Figure 2 show a detailed description of one sample trajectory of the mission profile. The mission includes the taxi mode where the vehicle is waiting to take off; a climb phase to ascend to a target altitude and to get to a loitering phase. After roughly three hours, the aircraft rapidly descend to strike phase which lasts as long as 20 minutes. It then climbs again to continue its ISR mission and eventually lands.

To capture a range of possible mission, we include probabilistic transitions among the phase so that several possible behaviors can be captured.

Figure 3: Description of the mission phases and transitions among them

Figure 3 shows a representation of the mission where each phase is represented as a mode of operation. Transitions among modes can be guarded by some conditions that might refer to the state of the aircraft. This formalism will be used to formally capture the mission. The intent in this document is to show what are the parameters that we introduce to capture of family of missions. We assume that the taxi time is uniformly distributed between 5 and 15 minutes, and that the strike phase can begin anytime between 1 and 420 minutes from the beginning of the loitering mode. Other parameters might be considered later if the mission specification will need to be refined.

Table 1 describes the requirements in terms of power and heat loads that are fixed as assumed known in the system. The first 5 columns summarize the known power requirements that include weapons, sensors and vehicle management system, avionics and radar, and actuators. The second part of the table describes the heat loads that are assumed fixed. The first column is the heat load that needs to be rejected by the low temperature air circuit. The actuators and the payload are cooled using a low temperature liquid. The last column is the heat load coming from the engine that is transferred to the oil which in turn needs to be cooled to guarantee a maximum temperature of 325 F.

Table 1: Requirements for each phase of the mission.

## Baseline system architecture

Figure 4 shows the architecture of a system that we will use as a baseline to compare the performance of our design. The system is divided into the electric power system (green and yellow), a thermal management system (brown), and an environmental control system (upper light cyan box) which in this design also integrates an auxiliary power unit. The engine is represented in the center of the figure.

Figure 4: Baseline system architecture

## Formal definition of the mission requirements

The functional requirements are captured in the form of a contract. A contract is specified by a set of variables, a set of assumption and a set of guarantees. In this section we present a single model for the assumptions and guarantees of the system.

Figure 5: Inputs, states, outputs and parameters of the system requirements specification

Figure 5 shows the inputs, outputs, states and parameter set of the formal representation of the functional requirements. The contract for the system requirements includes explicitly the fuel as an essential component of an aircraft. The fuel fixes most of the boundary conditions that need to be satisfied since it is the only source of energy and it is also used as a fluid for some of the actuators and as heat sink. The inputs to the system requirement specification are the total heat delivered to the fuel tank HT, the fuel consumed by the engine(s) mf, the temperature at the combustor Tf, the thrust provided by the engine F, and the voltage provided by the generators V. The outputs are the altitude h, the speed v,  the heat generated by the known loads H, the temperature of the fuel in the tank T, the total fuel mass in the tank M, and the current absorbed by the actuators I. Notice that the voltage V and the current I are actually vectors where each entry corresponds to a specific actuator or load. Also notice that inputs and outputs are waveforms namely functions from the positive real numbers to appropriate domains. These functions don’t need to be continuous, but we assume only left continuity.

This contract has a certain parameter set P that includes the total weight of the aircraft not considering fuel and the weather conditions. We distinguish state variables from parameters although parameters can be seen as state variables that do not change in time.

The contract is specified as follows. Let u=(HT,F,mf,Tf,V) be the vector of input signals and let x=O=(h,v,H,I,T,M) be the vector of state signals. Also, let the set of discrete modes of the system be Q={ES,TX,TO,C,CO,L1,DS,St,CS,L2,CI,CrI,D,L,E,Err}, standing for Engine Start, Taxi, Take Off, Climb, Cruise Out, first Loiter, Descent to Strike, Strike, Climb from Strike, second Loiter, Climb In, Cruise In, Descend, Land, End of Mission, and Error, respectively. In each mode q∈Q, the state variables evolve according to a set of (possibly stochastic) differential and algebraic equations  x=fq(x,u,ξ), gx,u=0  (where ξ is a possibly multi-dimensional random process). The discrete state Err is an accepting state indicating that the assumptions of the contract have been violated. Transitions among discrete states Tr⊆Q∖{Err}×Q∖Err∪Q×{Err} are associated with guard conditions and reset maps. In general, guard conditions can be probabilistic.

A switching function Sw :Q×Rn→Dist(Q) maps the hybrid state to a probability distribution over the discrete state space. A reset map Res:Q×Rn×Q→Dist(BRn) maps a hybrid state to a probability distribution over the set of states of a new mode.  The set of transitions includes transitions to the accepting state. These transitions are taken whenever assumptions are violated. For example, in each mode there is a transition to the accepting state where the guard is define by M≤0, meaning that the hybrid dynamical system transitions to the accepting state if the aircraft runs out of fuel. The same transitions can be enables under conditions on the maximum fuel temperature, climb rate, strike speed etc.

The guarantees provided by the contract are modeled by the dynamics in each mode and by transitions not leading to the accepting state.  Consider for instance the TX state and for the sake of this discussion consider the loads lumped as in Table 1. The state evolves as follows:

h=0, v=0

i28= v28L-R i28

H=HA+ HL

HA=3 kW,HL=27.7 kW

M=-mf

T=HTcfM-mf(Tf-T)M

With initial conditions h0= 0, v0=0, i28,0=0, T0 a random parameter that depends on the weather conditions, and M0 the initial fuel mass. We have only considered a lumped RL circuit as 28 DC load in this case. The relations between current and voltage of the other loads also need to be considered.  A transition from the TX mode to the Err mode is guarded by the following condition:

|v28-28|≥1

T≥315

Other conditions can be included by constraining for instance the derivative of the voltage. Clearly, the electric power system, the environmental control system and the thermal management system will be connected in a closed loop with this contract. The probability of entering the accepting state is the probability that the contract is violated by the system implementation.

# Component models

We start our design experiments by first building libraries of components for each of the domain involved in this study. The main domains we are concerned with are thermal management, electric power, propulsion and controls. For each domain we look at models with different fidelity. Typically, two abstraction layers are considered: steady state models and dynamical models. For the thermal management domain, we will not consider dynamical models because the time scale associated with this type of systems is very slow compared to the time scale of the other sub-systems. For the electric power system, instead, we consider both transient and steady state models. The same applies to the propulsion sub-system. For the controls domain, we will model the transition functions of software modules. We consider switching logics rather than continuous time controllers because the former is more complex than the latter.

## Thermal management system

A thermal management system (TMS) consist of a set of heat loads and heat sinks where generally heat is either added to the system or removed from the system through heat exchangers, pumps and fans for pressurizing the flow, and finally pipes/ducts and valves for distributing and controlling the flow.  For the challenge problem air vehicle there are two primary heat sinks available, ram air and fuel, and one intermediate heat sink available, oil (oil ultimately has to reject heat to ram air or fuel).  For this example problem, four heat loads are considered:

A baseline fuel based TMS is shown in Figure 6.  This system uses fuel as the primary heat sink.  Fuel is taken out of the tank, routed through various aircraft system load heat exchangers, driven by a main fuel pump (MF Pump) which provides the pressure rise necessary to move fuel through the main loop and pressurize the main engine fuel nozzle.  The actuation pump (Act Pump), which further boost fuel pressure, moves fuel in a recirculating loop, supplying engine actuators with flow.  This pump is sized for the worst case actuator demand flow, thus at most operating points it provides more fuel flow than is required by the actuators.  The excess flow is returned to the main fuel loop downstream of the main fuel pump.  The flow exiting the engine actuators is returned to the main fuel loop upstream of the main fuel pump.  After the split to the actuator pump, the main loop supplies fuel to the engine oil cooler (Eng Oil).  The fuel is then burned in the engine (To Engine).  If the fuel temperature after the engine oil cooler exceeds the maximum allowable fuel temperature, excess flow is taken from the tank.  After the engine oil cooler, the excess flow is returned back to the tank, while the burn flow goes to the engine.  The return fuel is cooled back to tank temperature by the air fuel cooler.  This fuel cooler is supplied with outside ram air through a ram air scoop.  During ground operations a fan pulls air through the scoop and fuel cooler.

There are six types of components in the TMS; heat exchangers, pumps, fans, pipes/ducts, valves, heat loads and splitters/mergers.  For each type, at each abstraction layer, component models are formulated that predict the component outlet state (pressure, temperature and flow rate) and power consumption, given inlet conditions and a set of characteristic parameters (e.g. pump efficiency or a pump efficiency map). At specific mission sizing conditions, these component models also size the component, providing weight and off-design characteristic parameters.

Figure 6: Baseline architecture of a thermal management system.

### Schematic Representation

A system model is formed by joining component inputs and outputs, according to a set of rules.  To facilitate this, we use a simple line diagram. For the example in Figure 6, the simple line diagram is shown in Figure 7 where sources are represented by solid circles and sinks by hatched circles.  Components are connected between the sources and the sinks.

Figure 7: Line diagram of the baseline system.

### Detailed component models

The most detailed abstraction level that we consider includes several details such as the detailed geometry of each component, the materials and the type of fluid. These models are already available as part of the HEATS tools developed at UTRC. In this tool, models are available as both equation-based models and scaled performance maps based on their sizing point. Heat Exchangers models are physics based and can be used for sizing and performance analysis.

### Abstract component models

We will develop abstract models starting from detailed ones in later sections in this document. However, standard abstractions can be constructed by making some additional assumptions on the environment in which components are going to be used. Specifically, we assume

the pressure rise across a pump to be fixed. Efficiency, power consumption and fuel temperature rise are calculated from the characteristics of the pump. The weight is derived as function of the maximum input power. We use the following convention for variables: Xji refers to the X variable at the input of component j. We use F for flow rate, P for pressure,  T for temperature, Q for heat and  W for power.  For a pump we use the following abstract model

where η is the efficiency of the pump and Cp is the specific heat of the fuel. Assuming ΔPi=6.9Mpa, the fluid density ρ=800 Kg/m3 and efficiency η=0.3 off design point and η=0.6 at design point, then we obtain the following simple model:

W=ΔPiFiρη

Weight=0.3 Wmax

where Wmax is the power required for the maximum fuel rate which is 1.9 Kg/s for a main fuel pump, 3.2 Kg/s for an afterburner pump, and 0.64 Kg/s for the actuator pump. Moreover, the efficiency for the actuator pump is 0.05 off design point and 0.6 at design point.

Heat Exchangers are modeled by using a fixed effectiveness number. The liquid side pressure drop is also considered fixed while the air side pressure drop varies. The weight is derived as function of effectiveness and the air side pressure drop. We use the superscript h to the hot side of the heat exchanger and c for the cold side. The following set of equations represents the model:

The parameters and weight models for the heat exchangers depends on their geometric properties and type (e.g. plate fin or tube fin). Consider for example the air/fuel heat exchanger used to cool down fuel right before sending it back to the tank. Then, a surrogate model for the weight is the following:

Weight=0.8-0.22lnΔPic(639eff3.3)

where eff is the effectiveness of the heat exchanger and the pressure drop on the cold side (i.e. air) is assumed to be ΔPic=3450 Pic

Fans are modeled using a fixed efficiency number. The weight is derived as a function of maximum input power

The power and weight models are as follows:

Wi=FiCpTiiρfγ-1γ- 1η

Weight=250Wi

Nominal values for the parameters are  γ=1.4 and η=0.7.

Finally, pipes and ducts are assumed to be ideal (i.e. no pressure drop):

These models will be checked an refined using automatic tools later in the section dealing with abstraction layers.

## Electric power system

An aircraft electric power system (EPS) consists of four basic building blocks: generators, switches & transmission-lines, converters, and electrical loads. These building blocks are interconnected to form aircraft EPS topologies.  The main function of an EPS topology is to generate and deliver stable continuous power to its electrical loads.  During EPS operation, certain parameters must be maintained within predefined limits. Those parameters include voltage, current, power, frequency, and heat throughout the system.  Other parameters associated with an EPS topology include weight, volume, and cost, which depend on power flow and those parameters can be approximated during the design phase via look-up tables.   For example, after electrical load power requirements are defined, power flow analysis can be used to define or fine tune the required power rating of transmission lines and switches for a given topology.  Then, power flow analysis can be used again to ensure that requirement compliance is maintained. This document presents a method to evaluate an aircraft EPS topology with the use of two evaluation tools; a steady state EPS model and a dynamic EPS model.  The tools evaluate the power flow throughout the EPS topology which reveals whether an EPS topology meets design requirements.  Furthermore, it is expected that many EPS topologies will satisfy the design requirements which will allow for an optimal EPS topology to be chosen among them. For example, the optimal EPS topology may be optimized for weight, cost, volume, losses (heat) or some combination of them.

### EPS Library

This document focuses on the tools developed to evaluate an EPS topology, but first a method for creating a topology is established.  Typically, an aircraft EPS topology is composed of a primary and a secondary distribution system.  A simplified generalization of the two systems is that the primary distribution system will source major loads, while the secondary system sources many smaller loads that can be lumped together and modeled as major loads to the primary distribution system.  The modeling work done for the META II project has focused on the primary distribution system components and their interconnections.  The major components that make up an EPS topology have been identified from aircraft systems literature and expert knowledge from Hamilton Sundstrand [3], [4].  Table 1 summarizes the various components that make up a primary distribution system which are categorized into the four basic building blocks of an EPS.  Each EPS component will have variants, for example a generator is defined as any source of electric power such as a primary generator, auxiliary power unit generator, ram air turbine generator, external power source, or a battery.  Transmission lines will vary in their wire gauge (AWG) which depends on the power flow requirement.  Switches vary with power requirement and their functionality, for example, a battery and a load may use different switch types despite the same or similar power level requirement. The fourth component type is electric loads which are used as inputs to an EPS topology formulation.  Many factors are considered when construction an EPS topology, currently legacy designs and expert knowledge are the main drivers in the design process.  This work formulates a sample topology by interpreting a set of composition rules derived from experts at Hamilton Sundstrand in combination with trends found in aircraft systems literature.

Table 1.  EPS Library: four basic building blocks.

| EPS Component | Symbol | Component Options |
| --- | --- | --- |
| - Generator |  | Primary Generator (G), Auxiliary Power Unit Generator (GAPU), Ram Air Turbine Generator (GRAT), External power source (PEXT), Battery Power (Batt) |
| - Switches & Transmission-Lines |  | Generator Circuit Breaker (GCB), Bus Tie Breaker (BTB), Auxiliary Power Breaker (APB), External Power Contactor (EPC), Electronic Load Control Unit (ECLU), Transmission Line (TL) |
| - Converters |  | DC to AC Converter - Static Inverter (DC/AC), AC to DC Converter – TRU (AC/DC) |
| - Electrical Loads |  | Load 115VAC - 230VAC 3Ø (LDAC), Load 28VDC (LDDC) |

### Composition Rules/Requirements

Formulation of an EPS topology begins with the electrical load definitions which define the system power requirements.  The EPS topology must be able to support the loads in all operating modes.  Furthermore, steady state and dynamic bus voltage limits must be upheld in all operating modes. In addition to the EPS library of Table 1, electrical load specifications and composition rules are required in order to create a candidate EPS topology.  The electrical load specifications must be defined for all operating modes such as maintenance-mode, take-off-mode, cruise-mode, emergency-mode, landing-mode, etc.  Electrical load specifications include their rated capacity in kVA and their power factor (PF), their reliability (essential vs. non-essential), and their electricity type which indicates the required input voltage magnitude and type whether it is an AC or DC load. The composition rules were derived from the EPS requirements list below:

- Satisfy power quality under all conditions ( voltage levels, distortion, etc)

- Load capacity of any component shall not be exceeded in steady state

- AC buses shall never be paralleled

- Faulted buses must be permanently isolated

- Meet all load reliability requirements while aircraft is available for revenue dispatch

- All buses must be kept powered  in case of one failure beyond Minimum Equipment List (MEL)

- A select set of critical buses must remain powered in the case of two failures beyond MEL

- Breaks in an AC subsystem must be less than x milliseconds

- Breaks in a DC subsystem must be less than y milliseconds

- Only part of the aircraft shall be powered when in maintenance mode (e.g. left DC side)

- Do not parallel transformer rectifier units (TRU) in steady state

- Minimize contactor actuations during transfers

- Use the APU only during take-off-mode, landing-mode or emergency-mode

- In-air start of APU limited to less than z feet of altitude

- No single point failure shall cause the loss of all critical busses

- Power to select loads must be maintained for thirty minutes if loss of all mechanical sources

The EPS requirements list is examined in order to extract details that can be used to formulate an EPS topology. However, first the minimum equipment list (MEL) is further explained; a MEL is defined as the list of EPS components that are required to be operational in order for an aircraft to be dispatched for revenue. For convenience, a MEL can be defined by the components that an aircraft may fly without.  For the items listed above, the MEL is defined by one failed generator and one failed converter (TRU).  Further explanation/interpretation of the EPS requirements is provided next.

- Satisfy power quality under all conditions ( voltage levels, distortion, etc)

Once a topology is finalized, the steady state voltage magnitude at every bus must be maintained within a given limit of the nominal voltage magnitude.  For the sample topology in the next section, ±6.5% was chosen.  A tolerance of ±6.5% was chosen because it would allow the rated current (2 x 350A) to flow through 1/0 AWG wire up to 100ft while staying within tolerance). The tolerance will vary from one EPS design to another, but ultimately the tolerance must be specified by the customer.  Topology voltage drops are calculated via load flow analysis with the steady stated evaluation tool. Transient evaluation is also performed on a topology to observe the behavior of the bus voltages when switches are actuated.  Acceptable voltage waveforms as a function of time must be specified by the customer or component supplier for each component.  In addition to the voltage requirement, a frequency variation limit is imposed not to exceed ±2.5% of the nominal.  The acceptable variation for the sample topology is based on electric power systems literature [6], however the actual limit for a given EPS design must also be provided by the customer.

- Load capacity of any component shall not be exceeded in steady state

This requirement addresses the distribution of loads among the various power sources. Composition rules must ensure that the topology does not overload any component (e.g. generator, switch, transmission line, and converter).  Composition rules will try to evenly distribute the loads among generators by imposing a maximum load limit per generator.  Non-primary distribution buses will also have a limited power flow capacity which will drive the topology design. Also maximum load per TL and SW are determined by their respective rated capacity.

- AC buses shall never be paralleled

Never paralleling AC buses means that no two or more AC source should ever have a common conductive path.  For example, if and EPS consists of two primary generators and two buses where Gen1 is powering Bus1 and Gen2 is powering Bus2, then Bus1 and Bus2 should never have a common conductor connecting them.  However, a “standby” connector (switch and transmission line) is allowed between AC buses.  That will allow for either bus to be powered in case one of the generators fail.

- Faulted buses must be permanently isolated

This requirement implies that a topology must be designed in such a way that in the case of a bus failure (e.g. short to ground), sufficient switches must exist so that the faulted bus can be electronically disconnected from the EPS network.

- Meet all load reliability requirements while aircraft is available for revenue dispatch

Load reliability is addressed in terms of independent lanes of electricity (ILE) and independent sources of electricity (ISE). The required ILE of a load is defined as the number of independent conductive paths between the load’s bus and the powering generator’s bus. Essential loads will generally require more than 1-ILE, however it should be noted that only 1-ILE is active at any given time.  The other ILE’s are put in place to supply the load incase the first lane fails.  The required ISE of a load refers to the number of sources that can power a load via independent or common lanes of electricity. The sample topology consists of loads with two reliability levels; (a) essential loads require 2-ILE and 3-ISE and (b) non-essential loads require 1-ILE and 2-ISE.  For any other EPS design, the customer must specify the ILE and ISE for each load. An aircraft is considered available for revenue dispatch as long as the MEL is fully functional.  In other words, for this example as long as only one generator and one converter has failed, then the aircraft EPS topology must be capable of powering all loads and must meet their ILE and ISE requirements.

- All buses must be kept powered  in case of one failure beyond Minimum Equipment List (MEL)

When one failure (any component) beyond the MEL occurs all buses must continue to be powered. Sufficient switches must be put in place to isolate a failed component and ILE’s must be added to energize any bus that may have lost power due to the failure.  In the case that a bus fails (e.g. thermal fatigue or short to ground), then that bus must be isolated via switches and the loads connected to that bus will be lost.  It is assumed that the lost load functions are fulfilled by mechanical backup systems.  If a generator fails, then it may be necessary to shed non-essential loads because the total generation capacity may be significantly reduced.  A list of loads to be shed in case of a generator failure beyond MEL must be provided by the customer.  In summary, in case of one failure beyond the MEL, the topology must be able to power all buses (except for failed buses).  Load shedding of non-essential loads is allowed and once the aircraft lands, the failure must be repaired before it can be dispatched for revenue again.  Lastly, the ILE and ISE requirements are reduced when the additional failure occurs.  Only one ILE and two ISE are required for essential loads while non-essential loads can lose their ILE.

- A select set of critical buses must remain powered in the case of two failures beyond MEL

This requirement requires the topology to have SW’s and TL’s in place to isolate two additional failures beyond MEL and power select buses as specified by the customer.  In the sample topology, all essential buses are considered critical and should be powered at all times (except if the failures are essential buses).  Further load shedding is allowed if generation capacity is reduced and once the aircraft lands, the failures must be repaired.

- Breaks in an AC subsystem must be less than x milliseconds

This requirement sets the maximum time allowed for the EPS controller to detect a de-energized bus and to actuate the appropriate switches to re-energize the bus.  The composition rules are not directly impacted by the time requirement since it is assumed that the EPS controller will be able to complete the actuation.  However, the time requirement is used to carry out dynamic simulations of the switch actuations.  The simulation results are then used to determine whether bus voltages are maintained within specified limits.

- Breaks in a DC subsystem must be less than y milliseconds

Similar to the AC actuation time, the detection and actuation of a de-energized bus must be completed within a time limit specific by the customer.  The specified also used for dynamic simulations and the results are used to determine whether bus voltages are maintained within specified limits.

- Only part of the aircraft shall be powered when in maintenance mode (e.g. left DC side)

In maintenance mode, the aircraft is on the ground and is power through its external power connection (EPC) to a ground power unit (GPU).  A list of loads that must be powered while in maintenance mode is provided by the customer.  The total maintenance mode load capacity will determine how much power will be needed from the GPU(s).  Normally one GPU will deliver a max power of 90kVA, therefore if the load capacity exceeds 90kVA then an addition EPC will be needed. The limit of 90kVA is typical of most airport GPU’s according to aircraft systems literature [1].

- Do not parallel transformer rectifier units (TRU) in steady state

Similar to the requirement that prohibits paralleling AC buses, the idea is to not parallel DC buses that are energized by TRUs while in steady state operation.  For example, if a topology contains two TRU’s and two buses were TRU-1 powers bus-1 and TRU-2 powers bus-2, then bus-1 and bus-2 should not have a common conductive path.  The exception to this rule is during transient operation, if another requirement exists which states that no break can occur on dc buses, then it is necessary to momentarily parallel TRU’s while the appropriate switch actuation occurs.

- Minimize contactor actuations during transfers

Minimizing contactor/switch actuations is desired for two reasons; (a) to minimize wear of the actuator and (b) to minimize dynamics in the EPS. Metrics for comparison may be number of switches actuating, total time of actuation, or a measure of the voltage/current transients.

- Use the APU only during take-off-mode, landing-mode or emergency-mode

This requirement will drive the power rating requirement of the APU generator.  The load capacity during take-off-mode must be supplied by the APU generator, therefore the APU generator capacity must be sized accordingly.  The sizing of the APU generator is also influenced by the MEL.  When a primary generator fails, the APU generator must be able to offset the loss of generation capacity which will allow for the aircraft to be dispatched for revenue service.  A primary generator is typically sized to operate at 80% of rated generating capacity when power all assigned loads [3], [4].

- In-air start of APU limited to less than z feet of altitude

The maximum altitude may be expressed as a percentage of the cruise altitude in a flight mission/profile.  This requirement means that if the aircraft is above the maximum APU starting altitude then the aircraft must climb down before starting the APU and then climb back to cruising altitude.  Load shedding may be necessary (if primary generator has failed) and only critical loads may be powered until the APU generator is operational.

- No single point failure shall cause the loss of all critical busses

If one component fails, then at least one critical bus must be energized.  Testing this requirement may require power availability tests of all critical buses after removing an EPS component.  The test shall be repeated for all EPS components.

- Power to select loads must be maintained for thirty minutes if loss of all mechanical sources

If all mechanical loads sources are lost, then all generators will fail. Therefore backup battery power must exist in order to power all critical loads for a minimum of thirty minutes.  Therefore, the battery capacity will be determined by the critical loads’ capacity.

### Sample topology

Thus far, definitions for an EPS component library and composition rules have been presented. To develop and EPS sample topology, a set of aircraft loads is required which will serve as the input to the topology development. A set of thirty six loads was derived from typical loads found on a Boeing-777 commercial aircraft [5]. All loads are listed in Table-2.  The table contains the load capacity and power factor at rated power. Some of the loads are essential loads while others are non-essential loads.  The essential loads are those powered by buses whose label is preceded by and “E” (e.g. E_B3 signifies essential bus number three); (a) essential loads require 2-ILE and 3-ISE and (b) non-essential loads require 1-ILE and 2-ISE. The EPS requirements from the previous section were applied to the load list in Table-2 to produce the EPS sample topology illustrated in Figure 1.  The topology consists of three sources and thirty six loads with a network of switches and transmission lines interconnecting them.  In order to evaluate the topology with the steady state and dynamic evaluation tools, a topology netlist format was created to capture the EPS topology network interconnections. Table-3 lists the netlist content which captures the necessary data for evaluation. There are five sets of data (matrices) describing the interconnection of the sources, switches, transmission lines, converters, and loads.  In addition to connectivity information, the data contains load capacity data and the type of electricity require by each component.  A sixth matrix (EPS_Code) captures the different types of electricity that may exist in a topology.  For example, a topology may consist of AC power at different voltage magnitudes and/or number of phases.  All the information contained in the netlist utilized by the evaluation tools.

Table 2.  Sample topology load list.

| Bus No | Load Index | Load Description | Power (kVA) | Power Factor |
| --- | --- | --- | --- | --- |
| E_B3 | Load_1 | Navigation Lights (NL) | 2.56 | 0.97 |
|  | Load_3 | Fuel Pump (FP) | 6 | 0.97 |
|  | Load_5 | Avionics (A) | 2.56 | 0.97 |
|  | Load_7 | Ice/Rain protection (IRP) | 2.6 | 0.97 |
|  | Load_9 | Equipment Cooling Fan (ECF) | 4.64 | 0.9 |
|  | Load_11 | Fuel boost pump (FBP) | 7.04 | 0.9 |
| E_B4 | Load_13 | Hydraulic pump (HP) | 14.24 | 0.9 |
|  | Load_15 | Window Heat (WH) | 0.8 | 0.97 |
| E_B11 | Load_17 | DC Load | 6.08 | 1 |
| B7 | Load_19 | Navigation Lights (NL) | 2.56 | 0.97 |
|  | Load_21 | Fuel Pump (FP) | 6 | 0.97 |
|  | Load_23 | Avionics (A) | 2.56 | 0.97 |
|  | Load_25 | Ice/Rain protection (IRP) | 2.6 | 0.97 |
|  | Load_27 | Equipment Cooling Fan (ECF) | 4.64 | 0.9 |
|  | Load_29 | Fuel boost pump (FBP) | 7.04 | 0.9 |
| B8 | Load_31 | Hydraulic pump (HP) | 14.24 | 0.9 |
|  | Load_33 | Window Heat (WH) | 0.8 | 0.97 |
| B13 | Load_35 | DC Load | 6.08 | 1 |
| E_B5 | Load_2 | Navigation Lights (NL) | 2.56 | 0.97 |
|  | Load_4 | Fuel Pump (FP) | 6 | 0.97 |
|  | Load_6 | Avionics (A) | 2.56 | 0.97 |
|  | Load_8 | Ice/Rain protection (IRP) | 2.6 | 0.97 |
|  | Load_10 | Equipment Cooling Fan (ECF) | 4.64 | 0.9 |
|  | Load_12 | Fuel boost pump (FBP) | 7.04 | 0.9 |
| E_B6 | Load_14 | Hydraulic pump (HP) | 14.24 | 0.9 |
|  | Load_16 | Window Heat (WH) | 0.8 | 0.97 |
| E_B12 | Load_18 | DC Load | 6.08 | 1 |
| B9 | Load_20 | Navigation Lights (NL) | 2.56 | 0.97 |
|  | Load_22 | Fuel Pump (FP) | 6 | 0.97 |
|  | Load_24 | Avionics (A) | 2.56 | 0.97 |
|  | Load_26 | Ice/Rain protection (IRP) | 2.6 | 0.97 |
|  | Load_28 | Equipment Cooling Fan (ECF) | 4.64 | 0.9 |
|  | Load_30 | Fuel boost pump (FBP) | 7.04 | 0.9 |
| B10 | Load_32 | Hydraulic pump (HP) | 14.24 | 0.9 |
|  | Load_34 | Window Heat (WH) | 0.8 | 0.97 |
| B14 | Load_36 | DC Load | 6.08 | 1 |

Figure 1. Sample topology used for EPS evaluation tool development.

Table 3.  Netlist data structure.

|  | Matrix Columns |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Input | 1 | 2 | 3 | 4 | 5 | 6 |
| Source_data | Bus No. | Bus Code | EPS Code | Real power | Reactive power | - |
| SW_data | SW No. | EPS Code | Node 1 | Node 2 | Impedance | - |
| TL_data | TL No. | EPS Code | Node 1 | Node 2 | Impedance | - |
| Conv_data | Conv No. | Bus In | Bus Out | EPS Code In | EPS Code Out | Efficiency |
| Load_data | Bus No. | Bus Code | EPS Code | Real power | Reactive power | - |
| EPS_Code | Code No. | Voltage | No. of Phases | DC = 1, AC = 2 | - | - |

### Evaluation Tools: Steady state EPS model & Dynamic EPS model

Given electrical load specifications, components from the EPS library were interconnected in way to uphold the composition rules to finally yield a sample EPS topology.  Performance metrics can be assigned to the topology through the application of two evaluation tools, steady state and dynamic EPS models.  Both types of models calculate the voltage and power at each bus, which are then compared to the bus voltage/power limit requirements.  In general, when evaluating multiple feasible topologies, it is expected for all the feasible topologies to meet the voltage limit requirements since those limits where abstracted into the composition rules.  However, some topologies will have less voltage deviation from nominal compared to others and that is how one EPS topology may be considered superior.  Figure 2 below summarizes the evaluation tools.

Figure 2.  EPS topology evolution tools summary.

#### Steady State EPS Model

An EPS topology is a circuit network interconnecting sources and loads.  Load power requirements are known rather than impedances which results in network equations in terms of power known as power flow equations.  Power flow equations are complex nonlinear algebraic equations; therefore they must be solved by iterative methods.  A set of equations (1) & (2) shown in Figure 3 are written for each bus in the topology.  The equations are coupled through the admittance vales (Yij) which captures the component interconnections.  Solving the two equations for all nodes, produces the power flow solution [4]. With the power flow solution in hand, the voltage and current for all EPS components under steady state conditions are known.  The steady state analysis is carried out for any combination of switch states.  For example, if the steady state bus voltages are of interest only during maintenance-mode where only part of the EPS is powered, then the switch data matrix in the netlist can be modified to “turn-off” the appropriate switches which models a maintenance mode state and repeat the load flow analysis.  Other switch states may model other modes such as take-off-mode, cruise-mode, emergency-mode, etc.  Each set of results reveals how well the bust voltages adhere to the bus voltage limits that are defined in the EPS requirements.

Figure 3.  Power flow analyses governing power equations.

#### Dynamic EPS model

Similar to the steady state EPS model, the input for the dynamic EPS model is the netlist outlined in Table-3.  The procedure created to automatically formulate the state space model of an EPS topology is outlined in Figure 4.  Common graph theory techniques were implemented in order to represent the EPS network with a set of interconnecting branches.  A matrix called the connectivity matrix “C” was generated to capture the branch interconnections [5].  Equation 3 in Figure 4 shows voltage loop equations in terms of the representative branches can be used to formulate a governing equations that can be represented in reduced-row-echelon form (RRE).  The RRE representation of the governing equations yield the state space coefficient matrices “A” and “B” which are then used to evaluate dynamic behavior of the EPS topology [6], [7].  Once the network is represented in with a state space model, one or more external sources can power the model which represents a complete EPS topology. All dynamic EPS components are modeled with an inductance (L), resistance (R), and a voltage source (S) connected in series. For each dynamic simulation, an EPS topology network is represented by a state space model whose states are the electrical load currents (the number of states will equal the number of loads).  Solution of the state space yields the instantaneous load currents which are used to calculate voltages of interest throughout the EPS topology.  The results are used to verify that the EPS dynamic requirements are met and can be fed to an optimization tool to reveal the better performing topology(s).

Figure 4.  Netlist to state space formulation process.

## Engine

## Software

In this section, we present our decision diagram based complexity metric in an intuitive way. We will focus on answering two questions: how this approach can be applied to different cases and why the decision diagram reflects the complexity of that case. Specifically, we will intuitively show and explain how the approach models and reveals the complexity of state transitions, scheduling, and mapping.

### Decision Diagram Based Complexity of Software Models

From past research works on Binary Decision Diagrams (BDD), we know that any logic function can be represented using BDDs (we assume all the BDD or Decision Diagrams (DD) mentioned in this report are variable-ordered.) which reflects the complexity of the logic function and has numerous good properties. Inspired by this, we use a more general DD to represent the complexity of models. If a model can be represented by a discrete function, it can be also represented by a Decision Diagram (DD). The overall flow is shown as follows:

Model⟶Descrete Transition Function⟶Decision Diagram

Since Discrete Functions and Decision Diagrams (DD) are equivalent, the main problem is how to convert a model into DTF or DD that reflects the complexity.

Complexity of Internal Logics. One basic complexity of a model is the internal logics that transform data or signals. In order to measure the complexity, we represent this transformation in DD, which would reveal how the inputs and outputs are related. Simple relations would result in simple DD while complex relation would cost more nodes, edges, and paths in the DD representation.

Figure 8 shows a finite state machine with 3 states and 3 transitions. It can be represented as discrete function (Figure 9) by adding the current states as the inputs and the next states as the outputs. The function thus represents all the possible transition of the FSM. By representing the discrete function in this way, the complexity of the model is exposed. Because complex transitions would result in larger DD and similar transitions would be ‘compressed’ in DD.

Figure 8: Model 1: A finite state machine model.

Figure 9: The decision diagram corresponding to Mode 1

(a) Model 1: a three-state finite state machine with one input and one output.

(b) Decision diagram representation of Model 1. State transitions are modeled by inputs and outputs.

Figure 1.1: Basic decision diagram representation of models

Complexity of Scheduling. Besides internal logics, scheduling is another source of complexity. In our model, scheduling is considered as a special function that takes the states of the system as input and outputs the next process to be scheduled. It is usually combined with the DD of other components (functions) in the discrete function. As shown in Figure 10 Figure 11, a data flow model with a scheduler can be represented as a DD composed of scheduler DD and component DDs.

In this example, suppose we have three components: a controller feeds two embedded programs. The scheduler executes the controller if no control signal has been generated, otherwise it executes either EP1 or EP2. This scheduling process can be represented as a function by introducing a random input variable (scheduling input as shown in Figure 11) to eliminate the non-determinism. And like BDD, we enumerate every possible scheduling by enumerating the scheduling input to represent the complexity of the non-determinism.

Figure 10: Model 2: A dataflow model with non-deterministic scheduler.

Figure 11: Decision diagram corresponding to Model 2

Complexity of Composition. If a system is composed of more than one sub-system of which complexities are known, the complexity of the system can be calculated by composing the corresponding Decision Diagrams. As shown in Figure 12, input variable is replaced by an output DD if this input is determined by that output.

Figure 12: DD composition of Model 1 and Model 2. The scheduler inputs are replaced by the outputs of the state machine.

Complexity of Mapping. Mapping is the bridge that connects the functional model and architectural model. Implementing the same functional model on different architectures would result in different complexities. To model this effect, the mapping is seen in two ways:

•	A couple of functional components are replaced by a architectural component with more implementation details;

•	The scheduling of functional components is constrained by the resources on the architecture.

Figure 13 shows a functional model of a ring network with four functions connected by four FIFO channels and its DD, which is composed of all the component DDs and scheduler DD.

•	Suppose these four FIFO channels are mapped to a bus with adaptors as shown in Figure 15. The necessary changes in the view of complexity are shown in Figure 16: the DDs of FIFO channels are replaced by the DDs of the bus with adaptors. The DD of scheduler is updated with the bus scheduling.

•	Suppose the four functions are mapped onto four tasks respectively and these four tasks are allocated onto two processors as shown in Figure 17. In the view of complexity, we need to update the DD of scheduler with constraints given by the shared processors as shown in Figure 18.

Figure 13: Model 3 : A functional model of a ring network with four functions connected by FIFO channels.

Figure 14: DD representation of Model 3, which is composed of all the DDs of the components.

Figure 15: Mapped Model 3: FIFO channels are mapped onto a bus with adapters.

Figure 16: Decision diagram representation of mapped Model 3. The DDs of FIFO channels and scheduling are replaced by the DD of the bus and adapters. The DD of the scheduler is also updated.

Figure 17: Mapped Model 3: functions are mapped onto tasks on two embedded processors

Figure 18: Decision diagram representation of mapped Model 3. The DD of the scheduler is updated as the tasks are scheduled by the embedded processor.

# Contract based System Engineering

We use the notion of contracts associated to models for system design and integration. A contract provides the ability to formally define the interfaces of a model. In particular, a contract specifies the assumptions on the environment of a component (i.e. the rest of the system) under which the component is able to perform. The behavior and performance of the component are defined by the guarantees. In this section, we first define a simple notion of contracts that we will use in the rest of the report. The general definition captures both steady state and dynamic behavior. We then restrict our work to steady state models and we develop a language to specify contracts. We call the language tinyCSL and will be used as the main vehicle for formally defining component libraries and composition rules.  A platform is a library (i.e. a set of component contracts) and a set of rules that must be satisfied by any composition of library elements. A platform defines a family of possible architectures.

## General definition of contracts

We begin with a definition of contracts by dividing variables into different kinds, which extends on the similar notion from [2]. A contract C is a tuple (V,A,G) in whichV(U,X) is the set of variables partitioned into uncontrollable variables U  taking on values in DU and controllable variables X taking on values in DX (thus V takes on values in DU×DX), A⊆DU is a set of assumptions on the uncontrollable variables and G⊆DU×DX is a set of guarantees or relations that hold between the uncontrollable variables U and the controllable variables X.

Given two contracts, we first need to check whether they are compatible or not.

Two contracts  and  are compatible if: , and

and

A simple test for compatibility is to verify that the two sets G1A2c and G2A1c are both empty. This condition is easy to check if the assumptions and the guarantees are algebraic functions of the variables.

Once compatibility is checked, we can define the parallel composition of two contracts  and , which is another contract C((U,X),A,G) that can be derived via the following rules:

1.	Pre-condition: ,

2.	,

3.	,

4.	, and

5.	A≔A1↑U1∪U2∩A2↑U1∪U2∪Gc↓U.

Using the concept of parallel composition along with compatibility check from Definition 2, the concept of composition can be extended to multiple contracts.

## A language for contract specification

The objective of the language is to provide an interchange model that is able to capture the specification and the library of component at each abstraction layer. In this language we do not provide any notion of time. We restrict the language to static properties, although we will provide some guidelines and the necessary infrastructure to model modes of operation. The language targets static and quasi static properties and it is used in preliminary design. We first define the language informally using a pseudo-BNF and plain English language to define its meaning. We start with the definition of a platform (keywords are written in bold font):

platform <ID> {

library <ID> {

component <ID> {

var <ID> : <TYPE>

…

par <ID>: <TYPE> value <VAL>

….

view <ID> {

assumption {

constraint : <CONSTR>

…

constraint: v = model(<PATH>,<ARGS>)

}

guarantee {

constraint : <CONSTR>

…

}

abstracts : <PATH>

}

….

}

….

}

rules {

<CONSTR>

….

}

}

A platform contains a library and a set of rules. The library contains several components. Each component is characterized by a set of variables with a given type. In addition to variables, a component has parameters that have a type and a value associated with them. A component contains a set of views, each characterized by assumptions and guarantees. These are lists of constraints on the variables of the components. We also allow a variable to be defined by an external model for which we provide a path and the arguments that need to be passed. The model should be a function that returns a value for the variable. A view is an abstraction of a detailed model that can be also provided as part of the description of a view. The set of rules define the valid instances of components from the library. The rules can refer to an architecture instance to its components and can use quantifiers such as "for all components of type comp1 in the architecture, the sum of their weights should not be greater than x". Moreover, an architecture instance comes with a connection contract that simply defines what are the variables that are equal to each other. For example, consider an architecture with two components comp1 and comp2. Also, let comp1 have two variables comp1.x and comp1.y. Let comp2 also have two variables comp2.x and comp2.y. Then, a connection contract can be expressed as a guarantee such that

comp1.x=comp2.x

comp1.y=comp2.y

The rules should define what the allowed connection contracts are. For example, "for ACGgen and DCGen, the voltage variables of ACGen cannot be connected to the voltage variable of DCGen".

In BNF form, the platform definition language is the following :

(1) <platform>               ::= platform <identifier> '{' <library> <rules> '}'

(2) <library>                ::= library <identifier> '{' <component>+ '}'

(3) <rules>                  ::= rules '{' <rule_assumption> <rule_guarantee> '}'

(4) <component>              ::= component <identifier> '{' <line>*  <view>*  '}'

(5) <line>                   ::= { <variable> | <parameter> } ';'

(6) <variable>               ::=  <input_variable> | <output_variable> | <internal_variable>

(7) <input_variable>         ::= var input <identifier> ':' <type>  [: <unit> ]

(8) <output_variable>        ::= var output <identifier> ':' <type> [ : <unit>]

(9) <internal_variable>      ::= var internal <identifier> ':' <type> [ : <unit>]

(10) <type>                   ::= boolean | integer | real  | complex | <matrix>

| <enum> | <range> | component

(11) <matrix>                 ::= matrix '(' <numerical_value> ',' <numerical_value> ')'

(12) <enum>                   ::= enum '{' <enum_element>* '}'

(13) <range>                  ::= range '[' <numerical_value> ',' <numerical_value> ']'

(14) <unit>                   ::= <identifier>

(15) <enum_element>           ::= <identifier>

(16) <parameter>              ::= par <identifier> ':' <type> [: <unit>] : value <numerical_expression>

(17) <view>                   ::= view <identifier> '{' <assumption> <guarantee>  [<abstraction> ] '}'

(18) <assumption>             ::= assumption '{' <constraint>* '}'

<rule_assumption>        ::= assumption '{' <rule_CONSTR>* '}'

(19) <constraint>             ::= <CONSTR>  ';'

(20) <guarantee>              ::= guarantee '{' <constraint>* '}'

<rule_guarantee>         ::= guarantee '{' <rule_CONSTR>* '}'

(21) <rules>                  ::= rules '{'  <rule_constraint>* '}'

(22) <rule_constraint>        ::= <identifier> ':' <rule_CONSTR>  ';'

(23) <abstraction>            ::= abstracts ':' <PATH>

(24) <PATH>                   ::= <platform_name> '/' <library_name> '/' 'component_name>

[ '/' <view_name> ]

(25) <platform_name>          ::= <identifier>

(26) <library_name>           ::= <identifier>

(27) <component_name>         ::= <identifier>

(28) <view_name>              ::= <identifier>

(29) <identifier>             ::= <letter>{<letter>|<digit>|'_'}*

(30) <value>                  ::= <numerical_value> | <boolean_value>

(31) <numerical_value>        ::= <digit>+ ['.' <digit>+][e<digit>+]

(32) <numerical_expression>   ::= <numerical_value>  {<numerical_operator> <numerical_value> }*

(33) <numerical_operator>     ::= '+' | '-' | '*' | '/'

(34) <boolean_value>          ::= true | false

(35) <CONSTR>                 ::=  <variable_assignment> | <boolean_assertion>

<rule_CONSTR>            ::=  <ins_quantified_constraint>

(36) <variable_assignment>    ::= <identifier> '=' <identifier> <operator> <identifier>

| <identifier> '=' <identifier> <operator> <numerical_expression>

| <identifier> '=' <identifier> <operator> <boolean_value>

| <identifier> '=' <value>

(37) <boolean_entry>          ::= <identifier> | <identifier> '=' <value> | '(' <identifier> '=' <value> ')'

(38) <boolean_literal>        ::= [NOT] <boolean_entry>

(39) <boolean_expression>     ::=  <boolean_literal>  { {AND | OR} <boolean_literal> }*

(40) <boolean_assertion>      ::=  if  < boolean_expression >  then  < boolean_expression >

[ else < boolean_expression > ]

| [NOT] < boolean_expression >  { AND | OR }  [NOT] < boolean_expression >

(41) <operator>               ::=  '<' | '>' | '<='  |  '>='

In general <CONSTR> stands for an AMPL type constraint expression. For now the BNF syntax of CONSTR is limited.

The BNF for a platform instantiation is the following

(42) <platform_istantiation>    ::= specification <identifier> ':' <platform_name> '{'

{<component_instantiation> ';'} *

{<variable> ';'}*

<assumption> <guarantee>  '}'

(xx) <assumption>               ::= assumption '{' <ins_constraint>* '}'

(xx) <ins_constraint>           ::= constraint ':' <ins_CONSTR>  ';'

(xx) <guarantee>                ::= guarantee '{' <ins_constraint>* '}'

(43) <component_instantiation>  ::= <instance_name> instanceOf <component_name>

'{'  <instance_var> '= ' <value>}* ';' '}'

| <instance_name> instanceOf COMPONENT

'{'  <instance_var> '= ' <value>}* ';' '}'

(44) <instance_name>            ::= <identifier>

(45) <component_name>           ::= <PATH>

(46) <instance_var>             ::= <identifier> '.' <identifier>

(47) <ins_CONSTR>               ::=  <ins_variable_assignment> | <ins_boolean_assertion>

|      <connection>

|      <ins_quantified_constraint>

(48) <ins_variable_assignment>  ::= <instance_var> '=' <instance_var> <operator> <instance_var>

| <instance_var> '=' <instance_var> <operator> <numerical_expression >

| <instance_var> '=' <instance_var> <operator> <boolean_value>

| <instance_var> '='

{ <numerical_value> | <boolean_value> | <identifier> }

(49) <ins_boolean_entry>        ::= < instance_var>

| <instance_var> '=' {<numerical_expression> | <boolean_value> }

(50) <ins_boolean_literal>      ::= [NOT] <ins_boolean_entry>

(51) <ins_boolean_expression>   ::=  <ins_boolean_literal>  { {AND | OR} <ins_boolean_literal> }*

(52) <ins_boolean_assertion>    ::= if  <ins_boolean_expression>  then  <ins_boolean_expression>

[ else <ins_boolean_expression> ]   ';'

| [NOT] <ins_ boolean_expression>  { AND | OR }

[NOT] <ins_ boolean_expression>   ';'

(53) <connection>               :: = <instance_var>   '=' <instance_var>

| <identifier> '=' <instance_var>

(54) <ins_quantified_constraint>

::= <quantifier>

'(' <component_instantiation>  { ',' <component_instantiation>  }* ')'

'{' {<ins_variable_assignment> | <ins_boolean_assertion> | <ins_quantified_constraint>}+ '}'

:= for '(' <component_instantiation> [ ':' <filter>]

{ ',' <component_instantiation> [<filter>]}* ')'

<set_operator> '(' <instance_var> ')'

{'=' | <operator>} {<numerical_value> | <boolean_value> }

(55) <filter>                   ::=  <ins_boolean_expression>

(56) <quantifier>               ::= forall | exists

(57) <set_operator>             ::= Sum | Product | Max | Min | Avg

Comments start with ’%’ and extend to the end of the line, i.e. until a new line character is found.

The semantics of the language define what are the valid architectures that may be instantiated from a given platform, i.e. what is the meaning of the rules specified in the platform definition and platform instantiation.

Constraints, both in the platform definition and instantiation evaluate to either true or false.

A numerical expression evaluates to a numerical value with the usual meaning of operators +,−,*,/. A <variable_assignment> (line 36 in the platform definition BNF) creates identifiers for numerical expressions. The right hand side of a variable_assignment ia a numerical expression that will evaluate to a numerical value once all variables are assigned to a particular value. All variables do not have a value when a platform is defined. At the time the identifier on the left hand side of a <variable_assignment> is just a placeholder for the symbolic numerical expression on the right hand side which may have variables.

During a platform instantiation, variables are bound to other variables in other component instance by means of connections.

A <boolean_expression> (line 39 in the platform definition BNF) evaluates to a true or false value depending on the values of boolean literals that occur in it and the standard semantics of AND and OR. A <boolean_entry> is either a boolean identifier or a comparison of a variable with a value (line 37 in the platform definition BNF). A <boolean_literal> is a boolean entry or its negation and also evaluates to a boolean true or false value depending on the value of the boolean entry and the standard semantics of NOT.

A <boolean_assertion> also evaluates to true or false depending on the values of <boolean_expression>s in it. The expression if X then Y else Z evaluates to true if X and Y are both true or X is false and Z is true. Here X, Y, Z are boolean expressions. All <boolean_assertion>s must evaluate to true in a valid architecture definition. In other words, X being true or false forces once of Y or Z to be true or false. Both ways of stating a <boolean_assertion> semantically capture the same information.

A platform instantiation, i.e an architecture definition, is a set of component instantiations with connections between component instances. A connection (line 53 in the platform instantiation BNF) is a statement that an output variable of one component instance feeds to an input variable of another component instance or it can be specified by means of a connection object. One output can go to multiple inputs.

The semantics of <ins_boolean_expression>, <ins_boolean_assertion> and <ins_variable_assignment> is the same as the corresponding constructs in the platform definition. The only difference is that instead of dealing with variables these deal with variables in component instances, i.e. <instance_var> (line 46 in the platform instantiation BNF).

The rules in a platform definition intuitively encode what the criteria are for a platform instantiation to be valid so they must refer to component instances. Since component instances are not known at platform definition time, rules must be quantified to talk about possible instances of components and how they relate to one another. Thus rules are specified using <ins_quantified_constraint> which makes an assertion over all instances of a component (using forall) or makes an existential assertion over some instance of a component (using exists). An <ins_quantified_constraint> can also state set constraints, such as the sum of some attribute of all instances of a component (line 54 in the platform instantiation BNF). A quantified constraint also evaluates to true or false.

Quantified constraints can be recursive, i.e., a forall constraint can include another forall or exists constraint inside it. The quantifications are over component instances. Since the universe of component instances in any platform instantiation is finite, the quantification can be removed by enumerating component instances. Thus rules specified as part of a platform definition can be translated into quantifier free constraints to create a contract of rules that a valid architecture must satisfy.

The rules in a platform definition can be transformed into a contract, .  is the set of assumptions and  is the set of guarantees corresponding to the rules.  and  are obtained from rules by eliminating the quantifiers as described above. This is illustrated in the following example.

Consider the following quantified rule which states that the power drawn from a generator is within the generator capacity. p_in is the variable corresponding to the power drawn by a load.

power_balance_constraint :

forall (g instanceOf generator) {

for( c instanceOf connection : c.in = g ){

Sum(out.p_in) <= g.p_out ;

}

}

Here connection is also a component defined as follows

component connection {

var in : component;

var out : component;

var availability : real;

view logic {

assumption {

}

guarantee {

constraint : 1 - 10e-6 < availability

< 1 - 10e-5;

}

}

}

Say, in a given platform instance, we have two generators - ,  and three loads . Say is connected to  and  is connected to  and . Then the above rule may be rewritten as the following constraint:

AND

Once the set of rules R in a platform have been transformed into a set of clauses corresponding to a particular instance, the rules define a contract .  derives from the assumption section of the rules and  derives from the guarantees section of the rules in the platform definition.

Before the conditions on a platform instance or system architecture can be enunciated, recall that assumptions and guarantees in the rules in a platform definition are formulas quantified over component instances. Variables that occur in these formulas refer to variables inside components, e.g., c.i refers to the variable i of component c.

A variable is said to be an input variable of a platform instance or system architecture iff

•	the variable is an input variable of a component instance in the platform instantiation, and

•	the variable is bound to an input variable of the platform instance by means of a connection. Recall that a platform instance defines a set of input, output and parameter variables which are independent of component instances.

Denote the set of input variables of a platform instance, S, by Inputs(S).

A behavior of a component assigns a history of values to the ports of the component. An implementation M is a set of behaviors of a component. M implements the component correctly if it satisfies the component contract C=(A,G). M satisfies C, written as M⊧C iff M∩A⊆G.

The definition of refinement of contracts is revisited here from [1]. A contract C'=(A',G') refines C=(A,G), written as C'≤C iff the set of models that satisfy C' is a subset of models that satisfy C. In other words, C'≤C iff A'⊇A and G'⊆G. A system architecture or platform instance, S, is said to be valid iff

•	C, the composition of the contracts of all component instances in S is consistent.

•	 where C is the composition of the contracts of all component instances in A, and  is the contract corresponding to rules in the platform of which S is an instance.

•	Variables in  are input variables of the platform instance, i.e., .

# Definition of a design flow

A design flow is built as a sequence of design activity conducted at different abstraction layers. The design flow proceeds from the specification of requirements down to a level where a system can be considered ready for manufacturing. The layers are related to-down by flowing requirements downwards, and bottom-up by abstracting performance and behavior upwards.

## Definition of an abstraction layer

Our design flow relies on the successive refinement of a high level specification into a concrete artifact. Each refinement step is named abstraction layer. In this section we define an abstraction layer and the design activities that are typically used to refine a system specification. As a result of the design activity, a refinement design is obtained which defines a new specification for the level below. In defining an abstraction level, we will explicitly refer to the notion of contract. A brief review of contracts is given in appendix.

Figure 19: Overall view of the elements of an abstraction layer including library and tools.

The elements of an abstraction layer are shown in Figure 19. The pivotal point is the platform which is defined by a set of library elements (or components) and a set of composition rules. The components represent both agents and communication among them. For example, an electric power system library contains agents such as generators, power converters and inverters, transformer rectifier units, and communication media such as busses and contactors. The distinction among these is not fundamental. The fundamental distinction is between the models representing the components (e.g. their dynamic behavior) and the model representing the way in which they interact. This distinction is a semantic one. In addition to the components, additional rules may be specified in the platform. These rules apply to any architecture that is generated by putting together library elements. For example, one may impose a rule on an engine platform requiring any engine to have a fan. Composition rules restrict the set of possible architecture that can be generated using the library elements.

The way in which libraries are defined today is not a standardized process. The languages used to capture components and their properties vary depending on the system to be designed. Moreover, much effort is spent in developing high fidelity models with little attention to their abstractions. In our view of the library, each component can have a model associated with it which can also be a detailed simulation model. However, several abstract view of the model should be provided as contracts. For example, a detailed simulation model of a power generator could be associated with a performance view, namely rated power and overloading conditions; an efficiency view, namely a relation between the rated power, the loading condition and the efficiency number; and a weight and space view, namely a relation between the rated power and the weight and geometry of the generator. These views can be captured by contracts, and contracts can be joined to yield the overall contract associated with the generator.

## Supporting tools for the design activity

Given a platform, design space exploration can be used to select a set of architectures with different trade-offs among a set of metrics provided by the system engineers. The goal of the design exploration activity is to select non-dominated (with respect to the metrics such as cost, weight, performance) compositions of library elements that satisfy composition rules and at the same time fulfill a set of requirements coming from the previous abstraction layer. Notice that the previous abstraction layer might just be the input to a design flow rather than the result of a design step. Such specification should also be represented by a contract that needs to be refined by any selected architecture. In the example of Figure 19, the first architecture is the composition of three components and its contract can be computed as composition of contracts C1 || C2 ||  C3. The composition must be a refinement of both the specification and of the rules of the platform which means that it satisfies the specification and at the same time is a valid composition of library elements.

Typically, design space exploration yields architectures that are correct by construction with respect to those metrics and contracts that are taken into account during optimization. Because of the complexity of components models, often time some of the constraints and quantities associated with components are either abstraction or not considered. Thus, each candidate architecture needs to go through validation which takes into account the details left behind during optimization. Validation can be done in several ways. One of the most common tools used for validation is simulation. In this context, generating a simulation for an architecture means bringing together heterogeneous models which is one of the challenges to be addressed by the infrastructure supporting the meta-language. Besides simulation, formal methods can also be applied to check properties of the system. The types of verification problems to be solved in this context are compatibility checking and refinement checking among contracts.

The need for controller synthesis stems from the fact that the current design space exploration tools do not take into account the full system dynamics. A controller might be among the components of a platform but its behavioral view might not be used during design optimization. It is possible to include some performance bound due to limitations in the complexity of the controller, but the detailed control parameters need to be fixed as a further refinement. The type of controllers to be synthesized are both discrete (for fault management) and continuous controllers.

The result of the design activities at one abstraction layer is a set of architecture instances with their associated contracts. These contracts form the specification to be handed over to the next abstraction layer.

# Abstraction layers design

The design paradigm that we use requires the definition of abstraction layers. This means deciding what the order of design decisions is and what the types of models that are used at each layer to make those decisions are. There is currently a lack of computer aid to define abstraction layers for a given application domain. In this section we show how contracts are used in the context of creating abstractions of components. We should through an example that meaningful abstractions are the ones that serve the purpose of making decisions based on accurate metrics. Thus, the abstraction process needs to provide the user with feedback on the accuracy of metrics (or conversely a measure of the information loss) that will be used in design decisions. We think that rather than automatically derive a design flow, a system engineer should be provided with a variety of tools to judge the quality of an abstraction which can only be done in reference to the metrics of interest. For example, when deciding on the architecture of a system to minimize weight, the abstract models for the components need to provide an accurate weight metric, while other metrics (e.g. temperatures) might be inaccurate or even absent.

## General criteria for the selection of abstraction layers

We identify two axes along which abstractions can be defined (see Figure 20). One axis is scope. Scope abstraction consists in grouping components together to form abstract modules (or components). For example, one may look at the interconnection of diodes, capacitors and resistors that transform a sinusoidal waveform into a constant voltage source (DC), or it can group these components together into a transformer rectifier unit (TRU). The other axis is fidelity. The same component can be modeled at different level of fidelity. For example, one may loop at an actual voltage and current at the output of a generator, or it can simply look at the presence or absence of power flows. However, we are typically more interested in the general concept of time scale while we consider hiding details still as a scope abstraction. Time scale abstraction is very common in industry where models are typically organized into steady state and dynamical models. In general, one can have a range of time scales that are considered where mode detailed models take into account faster time scales.

The current approach to the design of system is center around reference architectures that are taken as starting point for any design. In terms of scope, reference architecture is typically very detailed. The design process then proceeds by refining models ad adding details until the reference architecture if finally implemented. Changes are made to the reference architecture to accommodate requirements. The complexity of making such changes is high because of the large number of details that are already presence in the reference architecture.

The design flow we advocate follows the joint refinement of the scope and time scales for a system. For example, consider the case of an electric power system. The abstraction process would hide the details of the actual topology of the system and only present a view which contains power sources (generators, batteries and APUs), power sinks (i.e. the loads) and connections among them. Each component would have properties such as voltage levels, power requirements and reliability. This view would be refined into a more detailed topology where each connection becomes an actual path. The properties of the connection (i.e. maximum voltage drop or reliability) must be maintained in the refinement process (vertical contracts) so that any analysis done at the abstract level is maintained true after refinement. The fidelity of the models is also increased during the refinement process going from a simple logic view (presence/absence of power), to steady state models, to transient models.

The number of abstraction layers should be minimized. In fact, having many abstraction layers does not help in terms of the optimality of the final architecture because abstract models are in general inaccurate and decisions made at high abstraction layers might rule out promising part of the design space. However, having very few abstraction layers might make design spaces to large to be explored efficiency.

Figure 20: General principles for abstraction layer design.

## Definition and computation of abstract models

In this section, we formalize a procedure for abstraction of contracts in the context of steady state models. In the context of contract based design, abstraction is defined as follows. Consider a contract C(VU,X,A,G) where V is a set of variables associated with the model. The set of variables is divided into uncontrollable variables U (provided by the environment) and controllable variables X determined by the model associated with the contract. The assumption A⊆DU is a subset of the set of all possible values of the uncontrollable variables U. The guarantees G⊆DU×DX is a relation between the uncontrollable variables and the controllable ones.

Consider a contract C and a contract C'. Then C' is an abstraction of C if and only if V'⊆V, A'⊆A and G⊆G' (where we have omitted projections operators to avoid further complexity in the definition). In simple words, an abstraction is a quantity which assumes less and guarantees more than the original system.

We now formalize an automated procedure to generate an abstraction of a detailed model in the form of a contract C. In this procedure, the user selects some subset of the variables V, e.g., (X’,U’). Then, the following procedure will generate the set of assumptions A’ and the set of guarantees G’ for the abstraction C’ as follows.

  - Inputs: C(VU,X,A,G), X',U'

  - Initialize: A'=∅, G'=∅.

  - For every u∈U∖U', generate data points u1,…,uN∈A using the detailed model C.

  - For every x'∈X', compute the maximum value gmax≔maxix'(u',ui) and the minimum value gmin≔minix'(u',ui) and add the constraints x'≤gmax and x'≥gmin to the set G’.

  - For every u'⊆U', and such that there is an au'⊆A, add a to A'.

In other words, the above procedure uses the detailed model to generate samples of values of the controllable variables x' using for various values of the variables which the user decides to remove, and computes the maximum and minimum of these values. Then the guarantees are that the variables x' so computed will take values in the interval gmin≤x'≤gmax.

In order to obtain a further compact approximation of these guarantees, we can determine the best fit polynomial curves which approximate these minimum and maximum values. This helps us determine the quality of the abstraction. We adopt the following procedure:

Let gmax≈fθ,u' denote a polynomial parameterized by the coefficients θ. We solve the least squares problem: minθ ∑gmax(u')-fθ,u'2, to obtain the vector of parameters θ*. Additionally, this procedure also provides us with the error resulting out of the approximation. This error is simply the value of the objective function at its minimum. This error can be used to compare between different abstractions based on the number of coefficients chosen in the approximation.

In the next section, we demonstrate the application of this methodology to a pump. We began with a very detailed model of a pump involving several variables. The abstract model considers only a few (two in this case) out of all controllable and uncontrollable variables. We then apply the above procedure to generate the maximum and minimum values for the controllable variables. Then, we determine the best fit planes (linear fit) to determine approximations to the set of guarantees in the abstract model.

## Application to thermal management

Figure 21: Abstraction of a pump component.

Figure 2 shows the main concept behind abstraction as well as an application to the steady state model of a pump. The abstraction process removes some uncontrollable and some controllable variables from a contract. As a consequence, each value assigned to the uncontrollable variables of the abstract contract will correspond to a set of possible values for its controllable ones.

Consider a pump which is an key component of a thermal management system. The uncontrollable variables include the temperature T, the fuel from rate m and the pressure P of the inlet, the assigned pressure across the pump ΔP, the speed N, the type of the pump (e.g. radial or gear) and other parameters such as the properties of the fuel. The controllable variables include the temperature, fuel flow and pressure of the outlet, the weight of the pump w, the power consume by the pump pwr and the detailed geometry of the pump. A desirable model for design space exploration only exposes the weight and the power consumption of the pump as a function of the flow rate and pressure across the pump. Figure 2 shows the result of the abstraction for both the weight and the power consumption. We have computed the upper and lower bounds for these metrics as interpolating hyper planes. We notice two interesting facts for the weight metrics. The weight metrics cannot be abstracted into a usable function for a gear pump (the bounds show a wide range) while the abstraction is fairly good for a radial pump. If the pump type is also abstracted, then the resulting contract shows two disjoint intervals for the weight metrics which will make design space exploration hard (since the weight is not a convex set anymore). The power absorbed by a pump is instead a usable metrics even at high abstraction levels (upper an lower bounds are close).

References

[1] L. Benvenuti, A. Ferrari, E. Mazzi, and A. L. Sangiovanni Vincentelli, Contract-based design

for Computation and Verification of a Closed-loop Hybrid System," In Hybrid Systems:

Computation and Control, vol. 4981 of Lecture notes in Computer Science, pp. 58--71, 2008.

[2] B. Delahaye, and B. Caillaud, and A. Legay, Probabilistic Contracts: A Compositional Reasoning Methodology for the Design of Systems with Stochastic and/or non-Deterministic Aspects", In proceedings of the 10-th International Conference on Application of Concurrency to

System Design, Portugal, 2010.

[3] Moir, I., & Seabridge, A. (2008). Aircraft Systems (3rd ed.). Wiley.

[4]	Eismin, T. K. (2002). Aircraft Electricity & Electronics (5th ed.). McGraw Hill.

[5]	Andrade, L., & Tenning, C. (1992). Design of Boeing 777 Electric System. Aerospace and Electronic Systems Magazine, IEEE , vol. 7, pp. 4-11.

[6]	Saadat, H. (2002). Power System Analysis (2nd ed.). McGraw Hill.

[7]	Balabanian, N., & Bickart, T. A. (1969). Electrical Network Theory. John Wiley & Sons.

[8]	Chen, C.-T. (1999). Linear System Theory and Design (3rd ed.). New York, NY: Oxford  University Press.

[9]	Franklin, G. F., Powell, J. D., & Emami-Naeini, A. (2002). Feedback Control of Dynamic Systems (4th ed.). Upper Saddle River, NJ: Prentice-Hall.