Report:

Service Oriented Like Architecture for META II Integrated Tools

Haifeng Zhu, Randy Skelding, Jay Seney,

Alessandro Pinto, Brian Murray

- Introduction

This subtask of the META II project is to integrate several developed tools together to allow data exchange and design process automation for jet engine designs.  META II project targets contract-based system design methodology ([1] [2], etc).

There are several tools developed or available in this project summarized in [3].  First, Pacelab suite is a commercial tool that allows preliminary architecture design especially for aircraft systems.  It provides a graphical interface for component library design and an engineering workbench for system design.  It also provides a repository for version controls.  IBM Rhapsody is another tool that can provide similar functionality.  An architecture enumeration engine (AEE) is developed in the META II team that accepts definitions of devices and rules and generates feasible architectures.  An optimization tool written in AMPL is developed to further exploring the optimal architectures based on multi-objective optimization techniques.  There are also other ad-hoc tools developed for analysis such as complexity, adaptability, and manufacturing costs.  Domain experts provided analysis programs written in Matlab, etc. for calculating specific metrics such as load in electrical power system or fuel consumption of jet engine based on steady states.  These are just examples of the tools developed but nevertheless exhausted.  Therefore there is a need to integrate these tools together to allow data exchange and design collaboration.

- SOA like architecture

We proposed an integration method based on Service-Oriented Architecture, Figure 1.  This is an ideal situation that fits generic needs.  In reality, we also proposed a trimmed down solution that allows fast prototype.

Figure 1.  Tool Integration Architecture

In the ideal case, when the components exchange data, the subscribe to the published services by other components that provide the computations for specific needs.  For example, when the Pacelab tool exports the design contracts via tinyCSL to architecture enumeration tool, a conversion service can be called to convert the needed formats.  When the architecture enumeration tool needs a certain metric for reducing the number of generated architectures, it can call certain service that talk with Matlab domain model code that returns the computation result.  A sequencer is design to coordinate the execution sequence of these activities.  There exist sysML code for engine designs such as device and flow types, design contracts and mission use cases.  These data can just be imported and exported via conversion services, as they are not frequently exchanged among different design tools in META II.  In Figure 1, the mission use case sysML can go through conversion service as well.  Here we let it directly imported into adaptability module because it is currently only used by adaptability module.  In the future if it is used elsewhere, it should go through conversion service as well.

Implementation wise, different software techniques can be used, for example Microsoft .NET framework or J2EE environment.  The following we illustrate how this can be done in Java, and methods of using Microsoft .NET can be designed in a similar way.  In the case of J2EE, UDDI (Universal Description, Discovery, and Integration specifications) provides supports to service publication and WSDL (Web Service Description Language) to provide interfaces that allows automatic interface retrieval, and binding with protocol and message format.  JAX-RPC requires SOAP and supports interoperability with other services built with other technologies such as Microsoft .NET. There are some J2EE-compliant application servers (e.g. Oracle Application Server Containers for J2EE, the Sun One Application Server, and IBM WebSphere V6), support JAX-RPC.

For fast research prototype, such SOA structure may not be the most efficient.

There are two trimmed-down solutions we are using, one is based on a simple socket architecture and the other is based on RMI services.  Both of these serve as approximation of such an architecture.  The architecture still is same as Figure 1, just that the service bus is now running a simple socket based protocol, or RMI services instead of SOAP.  Next we discuss both of them, one termed as socket-based solution, and the other termed as RMI solution.  They actually can co-exist.

Figure 2. Trimmed-down version of SOA

Socket-based solution employs a simple socket server (called neutrino server) on a host, and does the forwarding based on the receiver’s host and port number.   The sequencer is a controller program that reads a job script language file, and executes different modules according to it.  Three-way communication exists in Figure 2, where the sequencer talks with neutrino server first which further talks with module 2.  This module executes the task from sequencer and then sends execution result back to neutrino server which then delivers back to sequencer.   If we call the neutrino Tomcat like server as “cat”, and the client/server simple code in each module as “kitten”, it is like a “cat-kitten” architecture.

In addition to the socket-based solution above, another trimmed down version of the SOA architecture is the RMI-based solution.  In each component, we will develop a Java adapter that wraps up the component, and talks with other components via RMI.  If there is a need, certain components can be refactored to Java.  Under the worst case, another alternative is we can use traditional client/server model to approximate such an RMI structure.  The communication is done by passing XML among hosts through ports.  This is less extensible in terms of services, and requests more programming by ourselves.

This SOA-like architecture can handle multiple levels of abstractions. A process is described in [3] to achieve this.  The initial design starts from level 1, using the DesignGUI (for example, Pacelab tool suite), where level 1 devices and all the assumptions and guarantees will be put in to build the platform including the component library.  The sequencer will then call the conversion service to convert this platform to tinyCSL which will be further converted to formats used by AEE that generates all feasible architectures.  During enumeration, the AEE may have a need to calculate certain metrics from other modeling tools written in Matlab and AMPL, etc.  This need can be met by calling specific services that wrap these tools up with Java, through service bus.  The generated architectures will be passed to Optimization tool that optimize over certain major metrics, such as weight, size, etc. that are requested by user.  In addition, various analysis tools on complexity, adaptability, etc. will produce corresponding results.  Further levels can be designed in such a way.

Next we discuss specific wrapper techniques.

The Java wrapper for Excel or .NET applications can be done in several ways.  One way is to use OleAutomation invoke technique that allows manipulating the Excel file through Java.  From .NET, J# can provide a bridging, or alternatively, third party packages such as J-Integra or JNBridge can be used.  Under proper configuration, Java class can be called as objects by .NET code.  In addition, data can be exchanged with file, IO redirection or web services.

The wrapper for AMPL is more difficult to develop.  Currently to the best of our knowledge AMPL cannot be used as a subroutine library where different component allows calling from another program. A "batch" session of AMPL can be executed from a C program though.  At the bottom line, AMPL by use of the ProcessBuilder, can exchange data through standard input/output streams:

reqQ = new ArrayBlockingQueue<List<String>>(200);

ProcessBuilder pb = new ProcessBuilder("cmd","/C","ampl","-i-");

final Process process = pb.start();

InputStream output = process.getInputStream();

Console console = new Console(output);

console.start();

OutputStream input = process.getOutputStream();

Terminal terminal = new Terminal(input,reqQ);

terminal.start();

Therefore, a Java AMPL API was developed for the Java wrapper's use. The Java AMPL API launches a background AMPL process and communicates with it via standard input and output streams. The AMPL process stays active and can be used to solve multiple models. The Java AMPL API provides methods to set the AMPL model, solve it, obtain the results, and a reset method to prepare for the next model. The Java wrapper builds the AMPL model by using several pre-programmed AMPL templates that have place holders for device numbers from the architecture. Once the model is built, it is sent to AMPL via the Java AMPL API for analysis.

Interfacing with Matlab from Java can be done in multiple ways.  One is to compile Matlab code into DLLs and call them from Java.  Alternatively, several techniques are available for such purposes.  For example: Through MATLAB Builder™ JA, Mathworks supports calling and using Java objects from Matlab but not calling Matlab commands from Java. Joshua Kaplan from Brown University wrote a Java RMI for calling Matlab from Java (http://matlabcontrol.googlecode.com/) in the following way:

RemoteMatlabProxyFactory factory = new RemoteMatlabProxyFactory();

//Get a proxy, launching MATLAB in the process

RemoteMatlabProxy proxy = factory.getProxy();

//Use the proxy to display 'hello world' in the MATLAB Command Window

proxy.eval("disp('hello world')");

- Interface design

  - Data Exchange Needs

In general, three categories of data are expected to be exchanged: platform model, architectures, specific computation operations.

For example, for the engine problem, in low level, the Matlab code is responsible for flying the complete flight profile and obtaining parameter optimization for architectures.  Due to the fact that the AMPL needs knowledge of full model to be able to compute needed results, not only the architecture, but also the platform model should be passed from AEE to Matlab code.  Such information exchange can be repeated in each request without the need of keeping state information, or for simplicity, only the platform reference (full path name in the repository or storage) is passed in the service request, and the computational service retrieves it from the repository or storage by itself one time.

3.1.1 Platform models

The platform models will be passed in the following format:

<library name="epslib">

<component name="generator">

<var name="output_vol" type=”real”, “volt”/>

…

</component>

<rules>

<assumptions>

</assumptions>

<guarantees>  // structural rules

extpath=FULL PATH reference to external model

format = “SysML”

tool=”Rhapsody”

</guarantees>

</rules>

</library>

3.1.2 Architectures

The architectures will be passed in the following format:

<composite_argument name="enginearch1", type="architecture">

<architecture id="1", total="1", level="1", platform="engine">

<Device id="9"  name="Core_Diffuser" />

<Device id="10" name="Core" />

<Device id="12" name="Core_Nozzle" />

<Device id="14" name="Multiple_Rear_Props" />

<Flow id="1" name="Core" />

<Flow id="2" name="Core" />

<Link id="0" source="9"  sink="10" flow="1" />

<Link id="2" source="10" sink="14" flow="2" />

</composite_argument>

We will talk about a generic service request format in the next section to support specific computation requests, where the above data formats will be used as inner text inside.

  - Interface Design

The interface should also allow different components running on different machines.  Therefore, the service requests and responses can be in the following format as our service language.  Each request should include operation name, additional parameters and expected output, such as:

<service-request name="OperationName" id=”1”>

<service-protocol=URI>

<client="localhost/aee">  // optional, client (caller) program with host name

<abstraction level=1>  // optional

<service-arguments>

<argument name=arg1, type=type1, value=value1/>

….

</service-arguments>

</service-request>

<service-response name="OperationName", id=”1”>

<argument name=result1, type=type1, value=value1, unit=unit1>

…

</service-response>

Figure 2. Interface Design

Any variables that are not scalar are composite_argument, for example, vectors.

<composite_argument name="v1", type="vector">

<element id="0"  value="0" />

…

</composite_argument>

For example, applying the above format to the fuel consumption result in the following:

<service-request name="FuelConsumption" id=”1”>

<service-protocol="RMI://serverhost:port/serviceprogram">

<client="localhost/aee">

<service-arguments>

<argument name="alt", type="integer", value="10000", unit="meter"/>

<argument name="mach", type="real", value="0.6"/>

<argument name="thrust_req", type="integer", value="1000", unit="pound"/>

<argument name="inlet_area_req", type="real", value="10", unit="meter^2"/>

<composite_argument name="enginearch1", type="architecture">

<architecture id="1", total="1", level="1", platform="engine">

<Device id="9"  name="Core_Diffuser" />

<Device id="10" name="Core" />

<Device id="12" name="Core_Nozzle" />

<Device id="14" name="Multiple_Rear_Props" />

<Flow id="1" name="Core" />

<Flow id="2" name="Core" />

<Link id="0" source="9"  sink="10" flow="1" />

<Link id="2" source="10" sink="14" flow="2" />

</composite_argument>

</service-arguments>

</service-request>

<service-response name="FuelConsumption", id=”1”>

<argument name="fuelconsumption", type="real", value="40", unit="ton">

</service-response>

Figure 3. Example Service Interface for Fuel Consumption

Note that this is an example of a standalone query from any party to the fuel consumption module.  While controlled under a sequencer, please refer to the Sequencer section (Section 4.5).

- Different Parts

  - Design GUI and AEE

We tried both PaceLab suite and Rhapsody as our Design GUI.  Pacelab allows graphical design of components and rules. Resulted design can be version controlled, and exported.  We developed parsers that produce Pacelab design to TinyCSL, a contract specification language for exchanging information of contracts.  Rhapsody support SysML design which currently does not support rule descriptions well.  A combined description using SysML and TinyCSL can be used for such purpose but is difficult to process.

Figure 4. Pacelab Design Example on EPS

Figure 5. Rhapsody Design Example on Jet Engine

  - AEE and Optimization

The AEE is wrapped up with some Java code so that it can talk with the Java AMPL generator which gets architecture information from AEE and use the component AMPL templates to generate optimization code to compute parameter set points.

Figure 6. AEE and Optimization Structure

The service requests for the input side of AEE can be defined as following:

<service-request name="PassPlatform" id=”1”>

<service-protocol="RMI://serverhost:port/serviceprogram">

<client="localhost/aee">

<service-arguments>

<argument name=”platform” value=”engine”/>

<argument name=”level” value=”1”/>

<composite_argument name="device_table", type="devicetable"/>

<device abbrev=”Core_diffuser”, min=0, max=1, maxBPR=14/>

…

</composite_argument>

<composite_argument name="device_rule", type="devicerule">

<rule id=1, type=”DefineQuantity, Name=”ReqMaxTS”, ”/>

</composite_argument>

<composite_argument name="input_table", type="inputtable">

<flow id=1, device#=”AmbientAir”>

<dev=”singleFan”>

<dev=”MultipleFan”>

…

</flow>

</composite_argument>

<composite_argument name="output_table", type="outputtable">

<flow id=1, device#=”AmbientAir”>

<dev=”singleFan”>

<dev=”MultipleFan”>

…

</flow>

</composite_argument>

<composite_argument name="flow_table", type="flowtable">

<flow name=AmbientAir, blockname=”Gaspath”, available=1, pairing=…/>

</composite_argument>

<composite_argument name="flow_rule", type="flowrule">

<rule id=1, type=”DefineQuantity, Name=”ReqMaxTS”, ”/>

</composite_argument>

</service-arguments>

</service-request>

<service-response name="PassPlatform", id=”1”>

<argument name="success", type="boolean", value="true">

</service-response>

Figure 6.  Example of Passing Platform Info to AEE

  - Other Analysis

As described earlier, many analyses can be included.  Here is an example of how fuel consumption analysis can be done.  The service data is defined in Figure 3, while the sequence is described in the following figure.  This can be controlled by the sequencer.

Figure 7. Fuel Consumption Calculation Example

Service interfaces for Mission Reader:

<service-request name="GenMissions" id=”1”>

<service-protocol="RMI://serverhost:port/missionReader">

<client="localhost/missionenumerator">

<service-arguments>

</service-arguments>

</service-request>

<service-response name="GetMissions" id=”1”>

<composite_argument name=”base_mission”, type=”mission_segment”, required=”true”>

<argument name="alt", type="integer", value="10000", unit="meter"/>

<argument name="mach", type="real", value="0.6"/>

<argument name="thrust_req", type="integer", value="1000", unit="pound"/>

<argument name="inlet_area_req", type="real", value="10", unit="meter^2"/>

<argument name=”next_mission_segment1”, type=”mission_segment”, nextmission=”highclimbrate”/>

<argument name=”next_mission_segment2”, type=”mission_segment”, nextmission=”longcruisedistance”/>

<argument name=”benefit”, type=”financial”,value=”100000”, unit=”USD”>

<composite_argument>

….

</service-response>

Figure 8. Service interfaces for Mission Reader

Mission Enumerator will then receive sequencer’s next directive: produce mission data for fuel consumption module:

<service-request name="EnumMissions" id=”1”>

<composite_argument name="missions" type="missions">

<composite_argument name="base_mission" type="mission_segment" required="true">

<argument name="alt" type="integer" value="10000" unit="meter"/>

<argument name="mach" type="real" value="0.6"/>

<argument name="inlet_area_req" type="real" value="10" unit="meter^2"/>

<argument name="next_mission_segment1" type="mission_segment" nextmission="highclimbrate"/>

<argument name="next_mission_segment2" type="mission_segment" nextmission="longcruisedistance"/>

<argument name="benefit" type="financial" value="1000000" unit="USD"/>

</composite_argument>

<composite_argument name="highclimbrate" type="mission_segment" required="false">

<argument name="alt" type="integer" value="10000" unit="meter"/>

<argument name="mach" type="real" value="0.6"/>

<argument name="thrust_req" type="integer" value="1000" unit="pound"/>

<argument name="inlet_area_req" type="real" value="10" unit="meter^2"/>

<argument name="next_mission_segment1" type="mission_segment" nextmission="highclimbrate"/>

<argument name="next_mission_segment2" type="mission_segment" nextmission="longcruisedistance"/>

<argument name="benefit" type="financial" value="5000000" unit="USD"/>

</composite_argument>

<composite_argument name="largecruisedistance" type="mission_segment" required="false">

<argument name="alt" type="integer" value="10000" unit="meter"/>

<argument name="mach" type="real" value="0.6"/>

<argument name="thrust_req" type="integer" value="1000" unit="pound"/>

<argument name="inlet_area_req" type="real" value="10" unit="meter^2"/>

<argument name="next_mission_segment1" type="mission_segment" nextmission="highclimbrate"/>

<argument name="next_mission_segment2" type="mission_segment" nextmission="longcruisedistance"/>

<argument name="benefit" type="financial" value="6000000" unit="USD"/>

</composite_argument>

</composite_argument>

</service-request>

Figure 9. Passing Mission Data to Fuel Consumption Module

Now the fuel consumption module already has the mission data, and it needs to calculate fuel consumption for each architecture (will be directed by sequencer, see Sequencer section, for “FilterArch” task).  In that task, it will pull architectures from AEE or repository one by one, or all at once, and calculate fuel consumption and feasibility.  Resulted feasible architectures will be stored to repository.  The service interfaces are similar to what is defined for repository.

Service interfaces for Adaptability module:

Functionally it will read “feasible” optimized architectures (produced by Fuel Consumption module and stored to repository).

On receiving the sequencer’s task of CalAdaptAndCost:

Adaptability module will send the following architecture retrieval request to repository and get ALL feasible optimized architectures back.  The service interfaces are defined in the repository section, however, specifically, make sure the needed info (see below) is produced by Fuel Consumption module and stored in the repository:

Example of repository response to adaptability module on passing optimized architecture (not just an architecture):

<composite_argument name="enginearch1", type="optimized_architecture">

<architecture id="1", total="1", level="0", platform="engine">

<Device id="9"  name="Core_Diffuser">

</Device>

<Device id="10" name="Core">

<var name="m_core", type="real", value="0.6" unit=”kg/s”/>

<var name="BPR", type="real", value="3"/>

<var name="OPR", type="real", value="4"/>

<var name="M_in", type="real", value="0.5", unit="mach"/>

<var name="IsentropicEfficiency_compressor ", type="real", value="0.8"/>

<var name="Efficiency_turbine", type="real", value="0.7"/>

<var name="TurbineInletTemperature", type="integer", value="230", unit="kelvin"/>

<var name="cost", type="real", value="1000000", unit=”USD”/>

</Device>

<Device id="12" name="Core_Nozzle">

</Device>

<Device id="14" name="Multiple_Rear_Props">

</Device>

<Flow id="1" name="Core" />

<Flow id="2" name="Core" />

<Link id="0" source="9"  sink="10" flow="1" />

<Link id="2" source="10" sink="14" flow="2" />

</composite_argument>

Figure 10. Adaptability Module requests optimized architectures from repository

Optimized architectures are architectures assigned with parameter values by optimization code in fuel consumption module.

Adaptability module then calculates the adaptability and cost metrics and displays them.

If needed, it sends this out to any party (not expected to implement for now):

<service-response name="Adaptability", id=”1”>

<argument name="adaptability", architecture=”enginearch1”, type="real", value="40"/>

<argument name="cost", architecture=”enginearch1”, type="real", value="300000000", unit=”USD”/>

<argument name="adaptability", architecture=”enginearch2”, type="real", value="40"/>

…

</service-response>

For visual picker and GenSysML, they pretty much interact with the repository only, so refer to the repository services.

  - Repository

Ideally it should be implemented with certain version control database.  As a proof-of-concept, we simply use folders/files wrapped up with Java services to demonstrate the idea.  The hierarchical folder structures should be as following, starting with different platforms (e.g. engine, EPS, etc.).  Inside each platform, the structures are defined by abstraction levels:

Engine

Platform

SystemLevel

L0

L1

Architectures

FeasiblieArchitectures

SystemLevel

L0

L1

EPS

Same structure

Figure 11.  Repository Hierarchy

As a basic requirement, the repository must support heterogeneous formats in Matlab, AMPL, TinyCSL, SysML, etc.  There needs a uniform format to accommodate these models written in different languages, which is described as following.

    - Platform libraries

They are labeled with TinyCSL language in the following way such that it can point to models written in any languages:

platform engine_logic_view_level_0 {

library engine_logic_view_level_0_library {

component {

var…

assumptions {

extpath=FULL PATH reference to external model

format = “SysML”

tool=”Rhapsody”

}

guarantees {

extpath=FULL PATH reference to external model

format = “SysML”

tool=”Rhapsody”

}

}

rules

{

assumptions {

extpath=FULL PATH reference to external model

format = “SysML”

tool=”Rhapsody”

}

guarantees {

extpath=FULL PATH reference to external model

format = “SysML”

tool=”Rhapsody”

}

}

}

}

Figure 12.  Example of Platform Library in Repository

4.4.2 Architectures

They can use the same XML format as described in Section 3.1.2

    - Services defined

The minimum design should have at least the following services defined for repository:

First, services requests to the repository must support:

- Get and store platforms:

Here is how to get a platform:

<service-request name="GetPlatform" id=”1”>

<protocol URI="RMI://serverhost:port/repositoryserviceprogram"/>

<client URI="localhost/aee"/>

<service-arguments>

<argument name="platform", type="string", value="epslib"/>

<argument name="level", type="integer", value="1"/>

</service-arguments>

</service-request>

<service-response name="GetPlatform", id=”1”>

<composite_argument name="eps", type="platform">

<library name="epslib">

<level=1/>

<component name="generator">

<var name="output_vol" type=”real”, “volt”/>

…

</component>

<rules>

….

</rules>

</library>

</composite_argument>

</service-response>

Here is how to store a platform:

<service-request name="StorePlatform" id=”1”>

<service-protocol="RMI://serverhost:port/repositoryserviceprogram">

<client="localhost/aee">

<service-arguments>

<argument name="platform", type="string", value="eps"/>

<argument name="level", type="integer", value="1"/>

<composite_argument name="epslib", type="platform">

<library name="epslib">

<level=1/>

<component name="generator">

<var name="output_vol" type=”real”, “volt”/>

…

</component>

<rules>

….

</rules>

</library>

</composite_argument>

</service-arguments>

</service-request>

<service-response name="StorePlatform", id=”1”>

<argument name=”success”, type=”boolean”, value=”true”>

</service-response>

Additional services can be defined per needs.

- Get and store architectures:

There are two modes for getting architectures.  One by one, and all at a time.

Here is how to get architectures one by one:

<service-request name="GetArchitectures" id=”1”>

<service-protocol="RMI://serverhost:port/repositoryserviceprogram">

<client="localhost/aee">

<service-arguments>

<argument name="platform", type="string", value="engine"/>

<argument name="level", type="integer", value="1"/>

<argument name="mode", value="next-one"/>

</service-arguments>

</service-request>

<service-response name="GetArchitectures", id=”1”>

<argument name=”success”, type=”boolean”, value=”true”>

<argument name=”noMore”, type=”boolean”, value=”false”>

<composite_argument name="enginearch1", type="architecture">

<architecture id="1", total="120", level="1", platform="engine">

<Device id="9"  name="Core_Diffuser" />

<Device id="10" name="Core" />

<Device id="12" name="Core_Nozzle" />

<Device id="14" name="Multiple_Rear_Props" />

<Flow id="1" name="Core" />

<Flow id="2" name="Core" />

<Link id="0" source="9"  sink="10" flow="1" />

<Link id="2" source="10" sink="14" flow="2" />

</composite_argument>

</service-response>

When there is no more, one line above is changed to:

<argument name=”noMore”, type=”boolean”, value=”true”>

And the repository will reset its pointer back to the first architecture.

When fetching all at once:

<argument name="mode", value="all-in-one"/>

The response will include all feasible architectures listed one by one in the response.

Here is how to store architectures:

<service-request name="StoreArchitectures" id=”1”>

<service-protocol="RMI://serverhost:port/repositoryserviceprogram">

<client="localhost/aee">

<service-arguments>

<composite_argument name="enginearch1", type="architecture">

<architecture id="1", level="1", platform="engine">

<Device id="9"  name="Core_Diffuser" />

<Device id="10" name="Core" />

<Device id="12" name="Core_Nozzle" />

<Device id="14" name="Multiple_Rear_Props" />

<Flow id="1" name="Core" />

<Flow id="2" name="Core" />

<Link id="0" source="9"  sink="10" flow="1" />

<Link id="2" source="10" sink="14" flow="2" />

</composite_argument>

<composite_argument name="enginearch2", type="architecture">

<architecture id="2", level="1", platform="engine">

<Device id="9"  name="Core_Diffuser" />

<Device id="10" name="Core" />

<Device id="12" name="Core_Nozzle" />

<Device id="14" name="Multiple_Rear_Props" />

<Flow id="1" name="Core" />

<Flow id="2" name="Core" />

<Link id="0" source="9"  sink="10" flow="1" />

<Link id="2" source="10" sink="14" flow="2" />

</composite_argument>

</service-arguments>

</service-request>

<service-response name="StoreArchitectures", id=”1”>

<argument name=”success”, type=”boolean”, value=”true”>

</service-response>

Additional services can be defined per needs.

    - Optimized architectures

See section 4.3

  - Sequencer

The sequencer is responsible for calling different tools to complete specific tasks in the design flow.  For example, the following graph indicates the sequence of execution of different tools in the engine demo.  There can be global sequencer and local sequencer that share the same job control language.

Figure 13. Sequence Control

The sequencer job control language is similar to a shell command batch or simple XML format that includes at least the tool’s name/path and task to be performed. Different tool’s Java wrappers are then called to perform different tasks.  An example of a general interface of such services is described in Figure 2.  The job script can include another job script:

</include =”file://localost/anotherfile.job” />

Example for Engine Level 1 analysis:  EngineL1analysis.job

<?xml version="1.0"?>

<jobscript name = "EngineL0Analysis">

<app name = "GenerateAEEInputs" taskowner="run://localhost/conversiontoolv3.exe" task="Conversion" targetApp="run://localhost/aee.xlsm">

<input>"file://localhost/c/EngineL0.tinyCSL"</input>

<fromformat>tinyCSL_SysML</fromformat>

<toformat>AEEInput</toformat>

</app>

<app name= "MissionEnumerator" taskowner="run://localhost/missionenumv1.exe" task="GetMissions" targetApp= "run://localhost/missionreader.exe">

</app>

<app name= "MissionEnumerator"  taskowner="run://localhost/missionenumv1.exe" task="EnumMissions" targetApp="run://localhost/fuelconsumption.exe">

</app>

<app name= "FuelConsumption"  taskowner="run://localhost/fuelconsumption.exe" task="FilterArch" targetApp="run://localhost/aee.xlsm">

<mode>one-by-one</mode>

</app>

<app name= "AdaptabilityAndCost" taskowner="run://localhost/c/folder/adaptv3_8.exe" task="CalAdaptAndCost">

</app>

<app name= "VisualPicker"

taskowner="run://localhost/visualpicker.exe" task="PickArch">

</app>

<app name= "GenSysML"

taskowner="run://localhost/gensysml.exe" task="GenPickedArch" targetApp="run://localhost/rhapsody/rhapsody.exe">

</app>

</jobscript>

<result success=”true”>

</result>

Figure 13.  Example of EngineL0Analysis.job

Specification for Task “Conversion” for Conversion Tool:

1) perform format conversion for “input” file

Convert from “fromfomat” to “toformat”

2)Pass the converted result to “TargetApp” AEE module (which includes its java wrapper, same henceforth when referring module) using “PassData” service

The “AEEInput” format is defined in Section 4.2.  Note: if Jim has a similar format, please update this section.  One the AEE receives the “PassData” service, it resets the architecture generation as if it receives a new platform (or config).

Specification for Task “GetMissions” for MissionEnumerator:

Send service request to MissionReader and get the mission info back (in the format defined in Section 4.3)

Specification for Task “EnumMissions” for MissionEnumerator:

1)Read the input file and generate missions (in the format defined in Section 4.3)

2)Pass it to “targetApp” fuelconsumption module using “PassMissions” service

Fuelconsumption module will forget the old mission set once it receives a new one.

Specification for Task “FilterArch” for FuleConsumption:

- Get architectures one by one, or all at once (depending on the mode), from targetApp (aee) and compute and simulate fuelconsumption and optimize architecture with assigned parameters.  It will produce [fuel, feasible] as result for each architecture.  During the computation, it uses the mission set data it receives earlier.

- For each feasible architecture, it will store it to repository

Specification for Task “CalAdaptAndCost” for Adaptability module:

It reads “feasible” architectures from repository and together with the mission data it retrieves from mission reader, produces adaptability and cost data

Specification for Task “VisualPicker” for visual tool:

It reads the “feasible” architectures from repository and allow user to visually examine them with PSU visual tool, and pick a set of them for next level design.  It will also mark the picked ones in repository.

Specification for Task “GenPickedArch” for GenSysML module:

It reads the picked architectures from repository and convert them into SysML (in files), and launch Rhapsody that reads them back.  May not need a service to do this, just launch may be good enough.

On each task completion, the sequencer receives a service response stating success or failure and decides to move forward.  Some additional functions such as stopping a task can also be done.

- References

[1] Alessandro Pinto, Sandor Becz, Hayden Reeve, Correct-by-construction Design of Aircraft Electric Power System.

[2] Brian Murry, et. al.  META II Proposal, 2010.

[3] Haifeng Zhu, Rejesh Kumar, Alessandro Pinto, META II Tools Summary, 2011.