Manufacturing modeling for complex cyber-physical system design

Changsheng Guo and Yu Long

United Technologies Research Center

East Hartford, CT 06108

# 1. Summary

This report summarizes the work done under Task 2 (Manufacturing Modeling Integration) of UTRC META II project - Complex Systems Design and Analysis.

A team of industry leaders headed by United Technologies Research Center (UTRC) worked on a program to lay the groundwork for the development of a new design paradigm that can meet the stated goal of a 5X reduction in the time required to design, build, integrate and test complex systems. To meet the ultimate goal of producing complex systems 5X faster than today, consideration of the design process alone is not enough. The influence of manufacturing on cost, schedule and performance must also be considered. To meet this need, the UTRC team worked to couple manufacturing modeling with the design process.

Under this work, a knowledge-based manufacturing modeling framework was developed. Manufacturing models for some components from the selected challenge problem, the aircraft engine, were developed to demonstrate the modeling approach. Finally, model integration was demonstrated at high level abstraction by using codes developed under this task, and mid-level abstraction with software package such as PriceSystems true planning, and at lower level abstraction, the integration was demonstrated with CAD and feature-based cost software - Dewhurst DFM and DFA. A case study is included using DFMA for typical aircraft engine parts.

# 2. Manufacturing Cost Modeling Approach

The objective of Task II is to develop approaches to model the manufacturing models that can be integrated with design at various designed stages with the goal of significantly reducing product development time for complex cyber-physical system.

Corresponding to the platform-based correct by construction design flow and the filter-based enumeration, a multi-level manufacturing modeling framework is adopted.

As shown in the summary chart, knowledge-based design and manufacturing integration is represented for all the abstraction levels. On the early design stage at high abstraction level, no features are unknown. Designs are based on rules and knowledge. For example, the max temperature should not exceed a material limit which may be 2300F. To integrate manufacturing and cost consideration with design process, manufacturing rules can be written as constrains for design evaluation. Some parametric cost estimation relationships can also be established using historical data.

During the mid level design stages, the usage of rules is gradually reduced and more feature and knowledge are used. For example, feature information and knowledge database of a component or a subsystem can be obtained from database for same or similar components.

At detailed design stage, feature based mfg and knowledge based design are dominated because all the parts have specified geometry and function description. Dewhurst DFM and DFA are the best available software for this level mfg cost estimation when detailed CAD and mfg process model is available.

# 3. Manufacturing Cost Models

Based on the approach outlined in the precious section, we developed and implemented manufacturing rules, parametric cost models for engine and some components for Level 0 and Level 1 abstraction, and featured-based cost model for some complex engine components.

3.1 Manufacturing knowledge and rule implementation

During early design stages, there will be many concepts generated by design enumeration. Design evaluation will use rules to choose feasible ones. At this stage, we implemented manufacturing rules as constrains. It was realized that there can be many manufacturing and material rules written.

For the RLC circuit demonstration, we used rules of temperature capability of the wire and the unit cost difference for high temperature capability wires. The length of the wire needed to implement a particular design was also used in cost estimation.

For the engine design, material capability rules were used. The temperature capabilities of a few common materials used in aerospace engine such as aluminum, titanium, nickel alloy and power nickel alloy were contracted as manufacturing rules during design evaluation.

3.2 Parametric Cost Estimation for Jet Engine

Parametric cost models for abstraction Level 0 and Level 1 were developed for the engine challenge problem.

3.2.1 Level 0: mission level

At Level 0, engine mission profile or engine specification is known. Cost model was established as a function of engine specifications including SFC, thrust, mach number, weight, and altitude. The model was built using regression analysis based on historical engine cost data from public domain as shown in Appendix 1.

Considering all engines, the development cost is:

Dev_Cost = 0.00012*T+1163*TSFC+0.31*Wt-731.54

The above cost model includes all historical data. For new engine development, “newly-designed” engines data may be more suitable. The development cost for new engine is:

Dev_Cost = 0.11*T+474.71*TSFC-0.395*Wt-138.14

Where,

T = Thrust (lbs)

TSFC = Thrust Specific Fuel Consumption (1/hr)

Wt = dry engine weight (lbs)

Dev_Cost = Development Cost in 2001 US$ (millions)

3.2.2 Level 1 Engine model

In the second level, thermodynamic data of the engine cycle become available. These parameters include pressure, temperature, pressure ratio, efficiency, mass flow rate and area. The cost model is as shown in Appendix 2. Instead of using purely empirical data, the cost has been constructed using physics-based model with limited cost data from public domain.

EngineDevCost = (massflowrate/massflowrate_ref)^a * [Cost-fan + Cost-casing + Cost-augment-nozzle_ref ] + Cost-core_efficiency + Cost_Machnumber + Cost_TurbineInletTemperature

Where,

EngineDevCost: engine development cost, in 2001 US $million.

Cost-fan = Cost-fan_ref * (BPR/BPR_ref)^b;

Cost-casing = Cost-casing_ref * (OPR/OPR_ref)^c;

Cost_Machnumber = d * MachNumber;

Cost_TurbineInlettemperature  = e * TurbineInletTemperature;

When 0.9<both efficiencies<1:

Cost-core_efficiency = CostCompressor_ref * 1/ (1-EfficiencyCompressor)^2 + CostTurbine_ref * 1/ (1-EfficiencyTurbine)^2 + Cost-burner;

When both compressor and turbine efficiencies <0.9:

Cost-core_efficiency = 300.

The value of the baseline _ref variables and coefficients a, b, c, d, e are calibrated with public data:

a=1.5; b=2; c=1.5, d= 60; e= 0.08.

Massflowrate_ref = 3000 lbs/sec, BPR_ref = 8, OPR_ref = 40, Cost-burner = 0.8, Cost-Compressor_ref = 1.5, CostTurbine_ref = 1.5, Cost-fan_ref = 150, Cost-casing_ref = 100, Cost-angment-nozzle_ref = 80.

3.2.3 Cost model for other engine components

Cost models were also developed for a number of engine components including high compressor, low compressor, combustor, diffuser, fan, cod and hot nozzles. These models are coded in Matlab.

These codes have been integrated with engine design optimization codes in AMPL delivered to DARPA.

3.4 Feature-based cost model

3.4.1 Gearbox cost model

A cost model template for a simple gearbox was also developed under Matlab/Simulink. This model uses standard gear design parameters as inputs to calculate cost. The idea is to link this with design tools so cost can be estimated automatically.

3.4.2 Cost model for the Integrally-bladed rotor (IBR) or Blisk

This is done with a commercial software, Design for Assembly (DFA) and DFM by Boothroyd Dewhurst Inc. The DFA analyzes assembly difficulties and DFM Concurrent Costing analyzes item manufacturing costs. The software estimates cost based on part volume, area, and other geometry data. Data can be transferred link to a CAD model through the SolidView viewer to transfer geometry data into your analysis before you enter dimensions on the first screen. If necessary, geometry can also be sketched using the geometry calculator.

The machine setups for a number of machining processes are included in the Process Chart with the operations listed. The three libraries - material, machine, and operation, are fully editable.

Manufacturing cost estimation involves material, process, machine tool, setup, and operation reject. At the top level, the material and principal process is determined. Model is built for an engine disk.

# 4. Model Integration

Under this task, manufacturing cost is modeled using both codes developed under this program in format of Matlab, and C+ codes and commercial software DFM. Evaluation of PriceH for parametric cost estimation is also performed. The question is how to integrate these models/software with design tools such as Unigraphics to do integrated design and cost estimation. A preliminary model integration demonstration was investigated with Model Center. Model Center® is a visual environment for process integration. With Model Center, an engineering process can be quickly created to perform complex design exploration to find the best design. A simple design under Unigraphics is linked with cost models such as PriceH and public codes written in Matlab and Excel. More work is needed to demonstrate the capability of this integration.