Cost formula for HPC, LPC and Turbine

Cost = Cost_efficiency_size + Cost_Machnumber + Cost_TurbineInletTemperature

Cost_efficiency_size:

Cost_efficiency_size = Cost_efficiency * (Radius of gasflow/Radius_ref) ^2;

Where,

When 0.9<Efficiencies<1:

Cost_efficiency = Cost_ref * 1/ (1-efficiency)^2; Cost_ref =1.5

When the efficiencies <0.9:

Cost _efficiency = 150 $million

Radius_ref = 0.7 meter. This is a component average radius of GE90 engine, because all these constants in level-1 formula are calibrated using GE90 data.

Using the square of radius is because typically the manufacturing cost is considered proportional to the surface area to be made.

Cost_Machnumber = d * MachNumber * 0.5;

//MachNumber can be replaced by max blade tip rotation speed, which represents the requirement for material strength.  The higher speed is, the higher cost is.

//d is calibrated for an engine with a compressor and a turbine, so for a single compressor or turbine, I introduce 0.5 for this cost term. The basic idea is we assume the costs for a compressor and a turbine are almost equal, which may be not accurate but good as a first order estimation.

Only for turbine:

Cost_TurbineInletTemperature  = e * TurbineInletTemperature;

This term comes from the cost to develop heat resistance coating and cooling technology to protect turbine blade. In this formula, we estimate it is linear to the inlet temperature.

In HPC and LPC, the typical temperature is usually not high, so we do not need to consider the temperature requirement for compressor material.

- (level 1 in Yu’s slide)

In addition to the above level, P, T, PR, Efficiency, Mass flow rate and area

Optional: estimated stage count

EngineWeight = (mass-flow-rate/mass-flow-rate_ref)^a * [W-core_ref + W-fan + W-casing + W-augment-nozzle_ref]

Unit: lbs

Where,

W-fan = Wfan_ref * (BPR/BPR_ref)^b; BPR is the bypass ratio

W-casing = Wcasing_ref * (OPR/OPR_ref)^c; OPR is the overall pressure ratio

The value of the ref and a, b, c need calibrated by history data. Efficiency does not influence weight too much.

First try is a=1.5; b=2; c=1.5.

W-core_ref=9000lbs, Wfan_ref = 4000 lbs, Wcasing_ref = 1500 lbs, W-augment-nozzle_ref = 400 lbs, Mass-flow-rate_ref = 3000 lbs/sec, BPR_ref = 8, OPR_ref = 40. FYI: W-core_ref includes 40% weight from compressor, 45% weight from turbine, 14% from shafts, 1% from burner.

EngineCost = (massflowrate/massflowrate_ref)^a * [Cost-fan + Cost-casing + Cost-augment-nozzle_ref ] + Cost-core_efficiency + Cost_Machnumber + Cost_TurbineInletTemperature

Unit: $million, here the cost is the development cost.

Where,

Cost-fan = Cost-fan_ref * (BPR/BPR_ref)^b;

Cost-casing = Cost-casing_ref * (OPR/OPR_ref)^c;

When 0.9<Efficiencies<1:  Cost-core_efficiency = CostCompressor_ref * 1/ (1-EfficiencyCompressor)^2 + CostTurbine_ref * 1/ (1-EfficiencyTurbine)^2 + Cost-burner;

When the efficiencies <0.9: Cost-core_efficiency = CostTurbine + CostCompressor + Costburner = 300 $million

Cost_Machnumber = d * MachNumber;

Cost_TurbineInlettemperature  = e * TurbineInletTemperature;

The value of the ref and a, b, c, d, e need calibrated by history data.

First try is d= 60; e= 0.08.

Massflowrate_ref = 3000lbs/sec, BPR_ref = 8, OPR_ref = 40, Cost-burner = 0.8 $million, Cost-Compressor_ref = 1.5 $million, CostTurbine_ref = 1.5 $million, Cost-fan_ref = 150 $million, Cost-casing_ref = 100 $million, Cost-angment-nozzle_ref = 80 $million

IF mass-flow-rate is a variable along the gasflow, please use the specific mass-flowrate ratio for each component.