Delivered to the Government in Accordance with Contract FA8650-10-C-7080

META II

Metrics Toolbox

Synopsis:

This is a MATLABTM based toolbox for computing various complexity metrics related to large-scale engineered systems. This toolbox computes the structural complexity; dynamic complexity and cycle complexity associated to underlying physical and functional architecture. The main functionalities of this toolbox are described below:

- Structural Complexity - The structural complexity metric is computed based on the system architecture (represented by the binary DSM or the adjacency matrix). This metric expresses the complexity inherent in the physical architecture of the system. Factors apart from system architecture that contribute to the computed complexity metric include component complexity weights and connection complexity weights. Refer to the report for the functional form of the structural complexity metric.

- Dynamic Complexity – There are multiple dimensions of dynamic complexity. There are two components: (a) connectivity of functional responses of the system and (b) their associated uncertainty. The final dynamic complexity is multiplicative. This metric can be computed for Black-Box models where explicit mathematical models are unavailable. This metric captures the complexity associated to interactions among functional responses (e.g., for a jet engine example they include TSFC, fuel burn, etc) and the uncertainties associated to their inter-relationships. It does not consider computational complexity associated to evaluation of physical quantities. Other dimensions of complexities like computational complexity, nonlinear complexities also evaluated in cases where explicit mathematical models are available.

- Cycle Complexity - Cycle complexity is computed based on the underlying physical architecture of the system. This captures complexity arising from cycles alone in the physical architecture and also computes a clustering of components into different layers (i.e., a layered decomposition of the architecture). The functional form of cycle complexity is similar to that of the structural complexity metric, but focuses on independent cycles in the architecture.

Introduction:  Complexity Metrics

The term “complexity” stems from the Latin word “complexitas”, meaning comprehensive or inclusive. In everyday language, it is the opposite of simplicity, but this interpretation is not useful for rigorous scientific analysis. Various scientific disciplines have studied the concept and principles of complexity in evaluating natural and technical systems. An objective evaluation of complexity in nontrivial systems would benefit system designers, managers and users, because they would be able to compare objectively and perhaps optimize systems better by reducing or keeping complexity within acceptable limits.

This toolbox contains MATLABTM implementation of different complexity metrics, namely, structural complexity metric, dynamic complexity metric and cycle complexity metric.  In following sections, these complexity metrics are briefly described and detailed description of functions that computes them are provided.

Structural Complexity Metric:

This metric is based on the system architecture. There are three factors contributing to structural complexity – number of components, number of connections and influence of underlying structure in the system architecture. The system architecture is represented as connectivity of physical elements using binary dependency matrix A(nxn) (i.e., DSM). If ith and jth components are connected, the Aij is one, otherwise zero. The structural complexity metric has a term that is only due to the complexity of the individual components (the α’s), the second term brings in a  contribution of complexity due to the number of interactions and the way the components are arranged w.r.t each other. This second term is the interesting one and it is the scaled product of the total number of interfaces and graph energy/nuclear norm. This term captures the challenges associated with system integration.

where the singular value decomposition (SVD) and associated graph energy of the architecture are as follows:

Notionally, graph energy ss a measure that encapsulates the “intricateness” of structural dependency among components. This term help distinguish structural complexity of two very different architectures with the same number of components and interactions.

Usage:

Function call -

[Structural_Complexity, E, connection_complexity, component_complexity] = structural_complexity(A,alpha,W);

Input:-

A: binary (0/1) Adjacency matrix (nxn) representing the physical architecture where n = #components

alpha: component complexity weight vector (nx1) - default = ones(nx1)

W: connection complexity weights matrix (nxn) - default = A (optional argument)

Output:

Structural_Complexity

E = sum of singular values of adjacency matrix A

connection_complexity = sum of all connection complexity weights

component_complexity = sum of all component complexity weights

Example: Let a system architecture be represented by a 0/1 binary matrix ‘Arch’. Its component complexities are given in a vector ‘alpha’ and connection complexities are represented in a matrix ‘W’ of same dimension as ‘Arch’.  The following call computes the structural complexity with other outputs:

>>[structural_complexity,graph_energy,connection_complexity,component_complexity] = structural_complexity(Arch,alpha,W)

structural_complexity =

717.7131

graph_energy =

124.8384

connection_complexity =

377

component_complexity =

73

>>

For further detail, including the argument list, type "help structural_complexity“ on MATLAB prompt.

Dynamic Complexity Metric:

The dynamic complexity metric aims to capture the impact of underlying physics and associated uncertainties governing the system and is an inherent property of the system. It does not explicitly look at the computational aspects (the computational aspect is implicitly captured in the interaction/uncertainty term).  In essence, higher dynamic complexity implies increased ability to deliver surprises. High dynamical complexity does not necessarily imply many interconnected components. In fact, a system with very few components can be extremely difficult to understand and control, largely due to the associated interaction uncertainties between functional responses of the system.  Dynamic complexity is a function of two fundamental components:

(1)   Dependency structure among engineering attributes

(2) Inherent uncertainty in the dependency relationships among engineering attributes.

Structure is represented via the topology of the interaction structure among the functional responses of a system. This is represented via a graph in which the functional responses are the nodes (vertices) of the graph, connected via links. Entropy is used as an indicator that measures the amount of uncertainty of the interactions between the functional responses governing the system. Generation of connectivity map of functional responses hinges on the computation of correlation among functional responses (after normalizing each response in [0,1]). The basic method for computing correlation is based on Pearson’s correlation coefficient in this case.  Once the dependency structure is generated, next step would be to estimate the uncertainty involved in the dependency structure.  Uncertainty is estimated using the notion of information entropy (i.e., Shannon entropy).  In this case, the entropic content of the system is computed using the image entropy (Shannon entropy) of the 2D scatter plots of engineering attributes in pairs and finally summing them up.  Hence the entropy measure is the sum of the entropies for functional responses taken in pairs. It represents the uncertainty associated with the interactions among functional responses involved. The computational process involves plotting x-y scatter plots for each pair of functional responses. In case of minimal scatter the relationship between the considered responses is “deterministic” indicating that dependency structure dominates, while a large scatter indicates uncertainty dominates. Higher entropy reveals larger uncertainty.  We express the dynamic complexity as the product of a function of dependency structure) and function of uncertainty in functional relationships as follows:

Dynamic Complexity = E(B)*total entropy, where E(B) = sum of singular values of dependency matrix.

The first term reflects the amount of structured information content in the dependency matrix (i.e., connectivity map). A higher value is indicative of presence of a large number of possible functional modes of the system. The second term measures the amount of uncertainty of interactions between functional responses governing the system. Hence, a system can have high dynamical complexity due to: (a) rich dependency structure (i.e., higher coupling); and (b) high relational uncertainty between functional responses or both.

Usage:

Function call -

[C,normalized_C,E,normalized_E,total_entropy,normalized_entropy] = dynamical_complexity(filename,conf_level,A_init);

Input:-

csv file containing observed values of (n) functional attributes over (m) states of the system.

confidence level for the significance test on pairwise correlations among functional attributes (default: 0.95)

Intial A matrix (binary 0/1 matrix), expressing known interactions among functional responses (optional)

Output:

C = dynamical complexity

E = sum of singular values of dependency structure matrix A

total_entropy = sum of all elements of the entropy_matrix

normalized_entropy = total_entropy/(n*(n-1)) where n is the number of functional variables

Example: File '3_Spool_Turbofan.csv' contains the observed functional responses over the system operational states and let 0.95 be the confidence level for significance test of pairwise correlations among functional responses. The following call computes the dynamic complexity with other outputs:

>>[C,normalized_C,E,normalized_E,total_entropy,normalized_entropy]= dynamical_complexity('3_Spool_Turbofan.csv',0.95)

C =735.5038

E =34.4807

total_entropy =21.3309

>>

For further detail, including the argument list, type "help dynamic_complexity“ on MATLAB prompt.

There is another variant of dynamic complexity metric that looks through the computational lens and computes a vector of attributes related to dynamic complexity. This requires access to explicit mathematical models for the complete system.  There are 4 factors attributed to dynamic complexity in this approach:

(1) Computational Complexity: The notion of complexity captured here is related to the amount of CPU time and the memory storage required for such a task.

(2) Dimensional Complexity: The dimensional complexity is based on the size of the underlying state space.

(3) Nonlinear Complexity: Motivated by techniques used to prove well-posedness of nonlinear systems, we suggest the following metric to quantify NDC.

Here, NL denotes the nonlinear part of the system dynamics and L denotes the linear part. The NDC is defined as the ratio of norm of the nonlinear part to norm of the linear part.

(4) Time Scale Complexity: If the system dynamics has a mixture of oscillatory and slow modes, the analysis and simulation becomes nontrivial. We first form a vector consisting of all the time scale differences in the system and then take the L2 norm of this vector.

The table below shows the different components of dynamic complexity for different Turbofan engine architectures.

| Dynamic Complexity | 1 SP T | 2 SP T | 2.5 SP T | 3 SP T | 5 SP T | 12 SP T |
| --- | --- | --- | --- | --- | --- | --- |
| Computational | 3.2 | 9.7 | 11.75 | 19 | 24 | 572 |
| Dimensional | 11 | 15 | 19 | 20 | 29 | 57 |
| Nonlinear | 1.5 | 4.6 | 6.4 | 7.8 | 14 | 35.9 |
| Time scale | 91 | 129 | 225 | 185 | 161 | 234 |

Usage:

Function call –

Main.m: Execute this script on the MATLAB prompt. There are 6 supported architecture in the current version and there reference is controlled by variable 'curr_arch_idx' on line #19. For example,set curr_arch_idx = 3 to execute 2.5 Spool Architecture.

Example:

>> Main

CD to Map Processing

Running AutoLoad...

……

dimensional_complexity =19

computational_complexity =15.6333

nonlinear_complexity =6.3522

timescale_complexity =211.1204

>>

Cycle Complexity Metric:

Cycle complexity is computed based on the underlying physical architecture of the system. This captures complexity arising from cycles alone in the physical architecture. At first for a given architecture we remove all sources and corresponding edges until no sources are left. For the obtained graph we construct the edge weighted adjacency matrix. Then we normalize the matrix so that the sum of all elements in the row is 1 by diving each element in the row by the sum of all weights in the row. If the row contains only zeros, we put 1 on the diagonal element in that row. For the obtained normalized matrix we compute eigenvalues. Then we compute cycle complexity function as follows:

where K is number of non-zero eigenvalues of the obtained graph after excluding source nodes, λi are non-zero eigenvalues, K1 is number of non-zero eigenvalues in the first bin of the unit circle.

Usage:

Function call -

[sum_1, sum_2, sum_2_second, metrics, cycle_complexity] = ComputeComplexityAndClustering(A);

Inputs:

A - adjacency matrix

Outputs:

cycle_complexity

Example: Consider the same system architecture defined in matrix ‘Arch’ (as in structural complexity metric example).

>> [sum_1, sum_2, sum_2_second, metrics, cycle_complexity] = ComputeComplexityAndClustering(Arch);cycle_complexity

cycle_complexity =1.4215

>>

Putting it all together:

All the metrics described above are computed for a specific example by using a MATLABTM script that automates the entire execution process (including path setting and directory management).

Example:

>> run_metrics

….

=================================================================

Structural Complexity metric:

Structural_Complexity =717.7131

Graph_Energy =34.4807

number_of_components =73

number_of_connections =377

=================================================================

If explicit Mathematical Equations are not available, but only Black-Box models or experimental data:

dynamic_complexity =735.5038

dynamic_complexity_connectivity_component =34.4807

dynamic_complexity_uncertainty_component =21.3309

=================================================================

If explicit Mathematical Equations are available and analyzes the system dynamics:

dimensional_complexity =19

computational_complexity =15.6402

nonlinear_complexity =7.7852

timescale_complexity =211.1204

=================================================================

Cycle Complexity Measure:

Cycle_Complexity =1.4215

=================================================================

>>

The functions (and scripts) described above are main calling functions that compute various complexity metrics discussed earlier. There are multiple utility functions that constitute this toolbox and provide capabilities to the high level functions. For further details on the metrics, refer to META II reports.