Overview

The purpose of this checklist is to support the inspection, i.e., the RTCA/DO-178B “for certification credit” verification reviews of the Design Requirements Document (DRD) software requirements.  The DRD includes high-level software requirements (including model diagrams) and hardware requirements.  This checklist does not support the inspection of the DRD hardware requirements.

Constraints

This checklist shall be used for all DRD inspections.

Overall Inspection Process Information

Section (1) concerns the completeness of the inspection packet.

Section (2) must be applied to each of the individual requirements inspected in a section or each group of requirements in a section.  In the case when multiple requirements in a section are being inspected, then each question in Section (2) must be applied to each of the individual requirements in that section.  It is up to the producer and the moderator to determine the appropriate method for handling Section (2) questions for multiple requirements.  Possible methods include having a separate Section (2) checklist filled out for each of the individual requirement inspected in a section or having the checklist questions apply universally to all requirements in a section or a subset of the requirements in a section.  In the second case, the defect recorded in the inspection defect log would be used to identify which question failed for which requirement.  This is done regardless if multiple requirement checklists are used.  Note that Section (2) questions apply to model diagrams as well as textual requirements.

Section (3) applies specifically to MATLAB Simulink model diagrams.  Section (3) questions should be applied to each individual model diagram inspected.

Section (4) applies specifically to MATLAB StateFlow diagrams.  Section (4) questions should be applied to each individual state diagram inspected.

Section (5) applies to an entire section or sections under inspection.  Section (5) questions should only be applied to a section when all applicable questions of Sections (2), (3), and (4) individual requirement questions are answered Y (passed) or NA (not applicable).  In the case of multiple sections, it is up to the producer and the moderator to determine whether a single Section (5) checklist or multiple Section (5) checklists can be used.  Note that Section (5) questions apply to model diagrams and state diagrams well as textual requirements.

Section (6) applies to the entire document.  Section (6) questions should only be applied after all sections of the DRD have been inspected and approved.

Section (1) checklist questions are applied against the inspection packet itself.  Before the inspection meeting or before the inspection packet is routed (i.e., for a Desktop Inspection) Section (1) checklist questions shall be answered either by the inspection’s moderator or by the inspection’s producer.  In addition, the moderator before the inspection meeting or in the case of a Desktop Inspection before the inspection packet is routed shall review the answers to the Section (1) checklist questions.

Checklist Questions

Circle the letter:  Y - Passed; N - Not Passed; NA - Not Applicable.

| 1 |  |  |  | Packet Contents If a NO answer is found to any of the following questions then abort the inspection, return the inspection packet to the producer, and notify the moderator. |
| --- | --- | --- | --- | --- |
| 1.1 | Y | N | NA | Is the Inspection Cover Sheet present with the following information (Project ID, Inspection ID, Version of Inspection checklist, Inspection Method, Meeting Date, Meeting Time, Meeting Location, Estimated Duration, Domain, Inspection Type, Reference Change Request, Element Name(s) and appropriate Configured Version(s), Element Size(s), Total inspection packet size, Examined size documents or Examined size code, moderator, and producer)? |
| 1.2 | Y | N | NA | Are copies of the Inspection Defect Log present? |
| 1.3 | Y | N | NA | Are the DRD section(s) to be inspected present? |
| 1.4 | Y | N | NA | Are the applicable SRS section(s) present? |
| 1.5 | Y | N | NA | Have the applicable SRS section(s) been inspected and approved? |
| 1.6 | Y | N | NA | Are the applicable SRS filename(s) with associated PVCS revision(s) of the requirements document’s section(s) been included in Table 1 below? |
| 1.7 | Y | N | NA | If applicable, are other reference data present or available (i.e., SRS, FHA, Safety Analysis, Customer/Partner ICDs, SRDD, HRDD, other DRD sections, requirement and model-based development standards, model_cgr.txt Matlab Diagram text file, DO-160D, DO-254, DO-178B, ARP4761, etc)? |

Table 1 SRS Filename and Revision  (Note)

| SRS Filename (If applicable) | PVCS Revision Number |
| --- | --- |

Note: Table 1 can be supplemented with additional attached sheets.

This section (2) must be applied to each of the individual requirements inspected in a section or group of requirements in a section including model diagrams.  In the case when multiple requirements in a section are being inspected then each question in Section (2) must be applied to each of the individual requirements in that section.  It is up to the producer and the moderator to determine the appropriate method for handling Section (2) questions for multiple requirements.  Possible methods include having a separate Section (2) checklist filled out for each of the individual requirement inspected in a section or having the checklist questions apply universally to all requirements in a section or a subset of the requirements in a section.  In the second case, the defect recorded in the inspection defect log would be used to identify which question failed for which requirement.  This is done regardless if multiple requirement checklists are used.

{A2.X or A3.X  [ED-12B/DO-178B paragraph]}  right flushed at the end of a checklist question shows the traceability to DO-178B/ED-12B, where X is Annex Table A2.X or Annex Table A3.X Objective and [ED-12B/DO-178B] represents the corresponding DO-178B/ED-12B paragraph.

Applicable Requirements: __________________________________________________________________

(Requirement or group of requirements that Section (2) checklist questions are applied against)

| 2 |  |  |  |  |  |  | Individual Requirements or Group of Requirements |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Correctness and Completeness Requirement Attributes |  |
| 2.1 | Y |  | N |  | NA |  | Do the requirement(s) conform to the project requirement standards (e.g., trace anchors format, use of shall, etc)? A2.2 [5.1.2e]}, {A3.5 [6.3.1e]}, {A3.6 [6.3.1f}} |  |
| 2.2 | Y |  | N |  | NA |  | Do the requirement(s) satisfy the System Requirements Specification? {A3.1 [6.3.1a]} |  |
| 2.3 | Y |  | N |  | NA |  | Are the requirement(s) unambiguous (i.e., explicit, clear, concise, understandable, only one interpretation by all stakeholders)? |  |
| 2.4 | Y |  | N |  | NA |  | Are the requirement(s) complete (this includes the following items)? {A2.1 [5.1.1a]}, {A3.6 [6.3.1f]} {A2.2 [11.9b]}, {A2.2 [11.9c]} - Does the requirement trace to its identified system requirements specification? - Have constraints and assumptions been adequately defined? - Have performance criteria, e.g., precision and accuracy been specified? - Have normal and degraded function mode of operation been identified? - Have all initial conditions been identified? - Have all normal and abnormal inputs been considered? - Have prohibited behavior characteristics been explicitly stated? - Has the requirement been allocated to the next level of detail? - Has the requirement been allocated to the next level of detail? - Has the requirements been stated in quantitative terms with tolerances where applicable? - Is there no missing information? - Is the requirement stated for all required environmental conditions (atmospheric, phases of - flight, etc.)? - Are there no TBD’s, undefined terms, or similar uncertain identifications? |  |
| 2.5 | Y |  | N |  | NA |  | Are the requirement(s) correct (this includes the following items)? {A3.2 [6.3.1b]}, {A3.1 [6.3.1a]}, {A2.2 [5.1.2a]}, A2.2 [5.1.2j]} - Are assumptions and constraints correct (e.g., does the requirement and its associated design requirements satisfy the system requirements)? - Are there no inconsistencies? - For high-level requirements, design and verification details are not specified? - Is the requirement free from apparent errors? |  |
| 2.6 | Y |  | N |  | NA |  | Are the requirement(s) implementable from a practical standpoint (i.e., realizable including consistency with the capabilities and limitations of available technology)? { {A3.3 [6.3.1c]} |  |
| 2.7 | Y |  | N |  | NA |  | Are inputs to the requirements detected as inadequate or incorrect feedback to the source input for clarification or correction? {A2.2 [5.1.2.b]} |  |
| 2.8 | Y |  | N |  | NA |  | Is the level of detail of the requirement(s) appropriate for the document? |  |
|  |  |  |  |  |  | Allocation /Traceability, Verification, and Control and Data Coupling Requirement Attributes |  |  |
|  |  |  |  |  |  | Allocation/Traceability/Derived Requirements/Deactivated Code Related Requirements |  |  |
| 2.9 | Y | N |  | NA |  | With the exception of derived requirements, does the high-level requirements trace to the system requirements? A2.2 [5.1.2.h]} |  |  |
| 2.10 | Y | N |  | NA |  | Are derived requirements identified correctly? {A2.2 [5.1.1.b]} |  |  |
| 2.11 | Y | N |  | NA |  | If applicable, are the derived requirement(s) consistent with the system requirements or other requirements from which they are derived (e.g., assumptions, constraints, etc)? {A2.2 [5.1.1.b]}, {A3.2 [6.3.1b]} |  |  |
| 2.12 | Y | N |  | NA |  | If applicable, is the derived requirement, including its reason for existence, correctly defined? |  |  |
| 2.13 | Y | N |  | NA |  | If applicable, are the derived requirement(s) compatible with the target computer hardware design (e.g., system response, IO hardware)? {A3.3 [6.3.1c]} |  |  |
| 2.14 | Y | N |  | NA |  | Are derived requirements indicated to the system-safety assessment process? {A2.2 [5.1.1.b]} |  |  |
| 2.15 | Y | N |  | NA |  | Are the requirement(s) and corresponding design requirements in the proper DRD section? |  |  |
| 2.16 | Y | N |  | NA |  | Are the requirement(s) properly allocated to either hardware or software? |  |  |
| 2.17 | Y | N |  | NA |  | Is the requirements identified as being tied to a design which after implementation is deactivated code? |  |  |
| 2.18 | Y | N |  | NA |  | Are the requirements specified clearly when the deactivated code is executed and not executed? |  |  |
|  |  |  |  |  |  | Verification |  |  |
| 2.19 | Y | N |  | NA |  | Are the requirement(s) verifiable (i.e., inspection, analysis, and/or testing)? {A2.2 [5.1.2e]}, {A3.4 [6.3.1d]} |  |  |
|  |  |  |  |  |  | Control and Data Coupling |  |  |
| 2.20 | Y | N |  | NA |  | Is the applicable thread scheduling type correct for proper component operation (i.e., loose or schedule before, schedule after)? |  |  |
| 2.21 | Y | N |  | NA |  | If a component is coupled to another component of lower assurance level, could a malfunction of the other component result in a hazard not consistent with the level of assurance (i.e., examples of malfunctions could be shutdown/reset, undesired mode, e.g., maintenance mode in air, undesired reversions, corrupted files)? |  |  |
| 2.22 | Y | N |  | NA |  | Are all abnormal data from coupled components considered (e.g., validity parameters, stale data, data drop-out, atypical floating point values, test modes, initial condition values, asynchrous data, atypical data rates)? |  |  |

Section (3) applies to a model diagram.  Section (3) questions should be applied to each individual model diagram inspected.

{A2.X or A3.X  [ED-12B/DO-178B paragraph]} right flushed at the end of a checklist question shows the traceability to DO-178B/ED-12B, where X is Annex Table A2.X or Annex Table A3.X Objective and [ED-12B/DO-178B] represents the corresponding DO-178B/ED-12B paragraph.

Model-Based Diagram: ___________________________________________

(Model Name(s))

| 3 |  |  |  | Simulink Model Diagrams |
| --- | --- | --- | --- | --- |
|  |  |  |  | Model Requirements |
| 3.1 | Y | N | NA | Do requirements comply with the allocated functional, performance, and safety-related system requirements? {A3.1 [6.3.1a]} |
| 3.2 | Y | N | NA | Is the model consistent with its DRD design information (including the text) and does it satisfy the intended functionality? {A3.2 [6.3.1b]}, {A3.7 [6.3.1g]} |
| 3.3 | Y | N | NA | Does each MATLAB model have a unique trace anchor traceable to an SRS trace anchor? {A3.6 [6.3.1f]} |
|  |  |  |  | #### Model Design |
| 3.4 | Y | N | NA | Are the MATLAB model requirements verifiable? {A3.7 [6.3.1d]} |
| 3.5 | Y | N | NA | Is the design efficient with respect to meeting requirements and design goals (e.g., execution time, RAM usage efficiency)? |
|  |  |  |  | Model Standards {A3.5 [6.3.1e]} |
| 3.6 | Y | N | NA | Is the model including the linked models complete and understandable? |
| 3.7 | Y | N | NA | Is the information in each model’s Revision block current and accurate? |
| 3.8 | Y | N | NA | Does the revision notation (e.g., letter, number) of the model match exactly the revision notation of the Code Generation Result file? |
| 3.9 | Y | N | NA | Are the timestamps of the model and the Code Generation Result file within a short time period of each other, i. e. within 20 seconds? |
| 3.10 | Y | N | NA | Is there an artifact showing that the model has successfully generated code (i.e., the associated model_cgr.txt text file shows successful completion)? |
| 3.11 | Y | N | NA | Do top-level Inport/Outport block names match the naming convention? |
|  |  |  |  | Model Mechanics |
| 3.12 | Y | N | NA | Has the C-Code Review (CCR) Tool been run on the autogenerated code with one of the two following outcomes? (1) The CCR Tool resulted in a PASS. (2) The CCR Tool resulted in FAIL with a subsequent manual review dispositioning the fault condition. |
| 3.13 | Y | N | NA | Has memory been initialized before it is used (i.e., have initial conditions values been set)? |
| 3.14 | Y | N | NA | Are fader blocks only used on continuous signals (i.e., only signals that change in discrete increments should be input to the fader)? |
| 3.15 | Y | N | NA | For Variable Lag and Variable Washout Filters, are the time constants at least twice the sample time of the model? |
| 3.16 | Y | N | NA | Are all division blocks protected from divide by zero (i.e., Input to the denominator cannot be zero; protection must be resident within the model and not in glue code)? |
| 3.17 | Y | N | NA | Is the logic input to any Multiport Switch Block limited so it is never less than one or greater than the number of switch ports on the block? |
| 3.18 | Y | N | NA | Have all outputs put on ASCB-D been amplitude limited? |
| 3.19 | Y | N | NA | Have amplitude limits for a signal been placed before rate limiters for a signal? |
| 3.20 | Y | N | NA | All outputs from integrators, filters, and counters amplitude limited? |
| 3.21 | Y | N | NA | Is an overflow condition possible when a large number is converted either implicitly or explicitly from float to integer? |
| 3.22 | Y | N | NA | Have all integrators been limited (i.e., avoiding the use of non-limited integrators)? |
| 3.24 | Y | N | NA | Are model outputs not tied together? |
|  |  |  |  | Model Usage of Floating Point Arithmetic |
| 3.25 | Y | N | NA | Have all counters been limited (i.e., avoiding the use of non-limited counters)? |
| 3.25 | Y | N | NA | Are transcendental functions used correctly? - Acos input angle is in the range of [-1<= angle <=+1]? - Asin input angle is in the range of [-1<= angle <=+1]? - Atan2 inputs are not both zero? - For tangent functions, is the appropriate range of input angle used relative to input angles near multiples of π/2? |
| 3.26 | Y | N | NA | Are the square root and power blocks taking an even nth root used correctly? - Are the square root blocks only taking the root of a positive number? - Are power blocks, when taking an nth even root, only taking the root of a positive number? |
| 3.27 | Y | N | NA | Are computations/algorithms defined to use floating-point numbers correctly? - Does single precision with 7 significant digits provide a large enough number ( + 3.4 x 10 ** 38) for the computation/algorithm? - Does single precision with 7 significant digits provide a small enough number (+1.18 x 10**38) for the computation/algorithm? - Is the algorithm numerically stable with single precision floating-point data, i.e., focus especially on operations requiring more than seven significant digits of accuracy? |
| 3.28 | Y | N | NA | Is use of the equality operator limited to integer data types (i.e., should not be used for floating point)? |

Section (4) applies to a state chart.  Section (4) questions should be applied to each individual state chart inspected.

{A2.X or A3.X  [ED-12B/DO-178B paragraph]} right flushed at the end of a checklist question shows the traceability to DO-178B/ED-12B, where X is Annex Table A2.X or Annex Table A3.X Objective and [ED-12B/DO-178B] represents the corresponding DO-178B/ED-12B paragraph.

Model-Based State Chart: ___________________________________________

(State Chart Name)

| 4 |  |  |  | State Charts |
| --- | --- | --- | --- | --- |
|  |  |  |  | Chart Requirements |
| 4.1 | Y | N | NA | Do requirements comply with the allocated functional, performance, and safety-related system requirements? {A3.1 [6.3.1a]} |
| 4.2 | Y | N | NA | Is the chart consistent with its DRD design information, including the text, and does it satisfy the intended functionality? {A3.2 [6.3.1b]}, {A3.7 [6.3.1g]} |
| 4.3 | Y | N | NA | Does each State Chart have a unique trace anchor traceable to an SRS trace anchor? {A3.6 [6.3.1f]} |
|  |  |  |  | #### Chart Design |
| 4.4 | Y | N | NA | Are the State Chart requirements verifiable? {A3.7 [6.3.1d]} |
| 4.5 | Y | N | NA | Is the design efficient with respect to meeting requirements and design goals (e.g., execution time, and RAM usage efficiency)? |
|  |  |  |  | Chart Standards {A3.5 [6.3.1e]} |
| 4.6 | Y | N | NA | Is the state chart including all sub states complete and understandable? |
| 4.7 | Y | N | NA | Are complex mathematical expressions avoided in the state chart (i.e, Complex mathematical expressions should be performed in Simulink)? |
| 4.8 | Y | N | NA | Are all events scoped to be within this State Chart (i.e. there should be no interaction with other state charts in the model)? |
| 4.9 | Y | N | NA | Are event broadcast rules followed? - Direct event broadcasts are the only type allowed - The send syntax or qualified event names are used to direct the event to a particular state - Multiple send statements are used to direct an event to more than one state |
| 4.10 | Y | N | NA | For directed events in parallel states, is the master state that is directing the events located below the other states (i.e. it must have a higher number in the upper right corner)? |
| 4.11 | Y | N | NA | Are all MATLAB functions, instructions and operators avoided in the state chart? |
| 4.12 | Y | N | NA | Have pointers to custom code variables been avoided? |
| 4.13 | Y | N | NA | Are history junctions avoided? |
| 4.14 | Y | N | NA | Do all state chart inputs and outputs follow the standard naming convention? - All inputs have a prefix of i - All outputs have a prefix of o - All names are in reverse camelback notation - Length of names do not exceed 32 characters - No forward slashes (\) are in the names |
| 4.15 | Y | N | NA | Have all StateFlow/Simulink interface standards been followed? - State chart has only input signals and input vectors, no input busses - Input signals are labeled - State chart has only output signals and output vectors - Strong data typing is used - Tunable parameters may be used - State Chart can have trigger outputs - State Chart can be triggered |
| 4.16 | Y | N | NA | Do all state chart internal signals and variables follow the standard naming convention? - All variables have a prefix of v - All constants have a prefix of c - All names are in reverse camelback notation - Length of names do not exceed 32 characters - No forward slashes (\) are in the names |
| 4.17 | Y | N | NA | Are all signals and variables within the state chart local in scope (i.e. they are not shared across other state charts in the model)? |
|  |  |  |  | Chart Usage of Fixed Point and Floating Point Arithmetic |
| 4.18 | Y | N | NA | Have all counters been limited (i.e., avoiding the use of non-limited counters)? |
| 4.19 | Y | N | NA | Are all division operations protected from divide by zero (i.e., Input to the denominator cannot be zero; protection must be resident within the chart and not in glue code)? |
| 4.20 | Y | N | NA | Are transcendental functions used correctly? - Acos input angle is in the range of [-1<= angle <=+1]? - Asin input angle is in the range of [-1<= angle <=+1]? - Atan2 inputs are not both zero? - For tangent functions, is the appropriate range of input angle used relative to input angles near multiples of π/2? |
| 4.21 | Y | N | NA | Are the square root and power functions used correctly? - Are the square root blocks only taking the root of a positive number? - Is the power block, when taking a root, only taking the root of a positive number? |
| 4.22 | Y | N | NA | Are computations/algorithms defined to use floating-point numbers correctly? - Does single precision with 7 significant digits provide a large enough number ( + 3.4 x 10 ** 38) for the computation/algorithm? - Does single precision with 7 significant digits provide a small enough number (+1.18 x 10**38) for the computation/algorithm? - Is the algorithm numerically stable with single precision floating-point data, i.e., focus especially on operations requiring more than seven significant digits of accuracy? |
| 4.23 | Y | N | NA | Is use of the equality operator limited to integer data types (i.e., should not be used for floating point)? |

Section (5) applies to an entire section or sections under inspection including model diagrams.  Section (5) questions should only be applied to a section when all applicable questions of Sections (2) and (3) individual requirement questions are answered Y (passed) or NA (not applicable).  In the case of multiple sections, it is up to the producer and the moderator to determine whether a single Section (5) checklist or multiple section (5) checklists can be used.

{A2.X or A3.X  [ED-12B/DO-178B paragraph]} right flushed at the end of a checklist question shows the traceability to DO-178B/ED-12B, where X is Annex Table A2.X or Annex Table A3.X Objective and [ED-12B/DO-178B] represents the corresponding DO-178B/ED-12B paragraph.

Applicable Section(s): _______________________________________________________________

(Section or sections of the material inspected that Section (5) checklist questions are applied)

| 5 |  |  |  | Entire DRD Section |
| --- | --- | --- | --- | --- |
|  |  |  |  | Correctness and Completeness Requirement Attributes |
| 5.1 | Y | N | NA | Are the requirements sufficient to cover the intended scope of the DRD section? |
| 5.2 | Y | N | NA | Are there no conflicting requirements (i.e., conflict can arise in areas such as: environmental, accuracy, interface, etc.)? {A3.2 [6.3.1b]} |
| 5.3 | Y | N | NA | Are the requirements free from duplication and overlap by other requirements? {A3.2 [6.3.1b]} |
| 5.4 | Y | N | NA | Does the section conform to requirement standards (i.e., including that the requirements can be changed without excessive impact to this document or to other documents)? {A.3.5 [6.3.1e]} |
| 5.5 | Y | N | NA | Have all time-critical sequencing requirements/constraints been documented ## (i.e., functions that are sensitive to data latency or freshness have been addressed)? {A3.3 [6.3.1.c]} |
| 5.6 | Y | N | NA | ## Are prohibited behavior characteristics explicitly stated (e.g., failures, invalid modes/states, invalid transitions, etc)? |
|  |  |  |  | Safety, External Interfaces, and Other Requirement Attributes |
| 5.7 | Y | N | NA | ## Have reliability, availability, and fault tolerance requirements been addressed by the requirements and any associated design requirements (i.e., for functions as well as products or systems)? |
| 5.8 | Y | N | NA | Have all interfaces been defined (i.e., internal, external, physical, functional, human, etc.)? {A3.3 [6.3.1c]} |
| 5.9 | Y | N | NA | Can all design assumptions be justified by supporting data (i.e., if it is obvious from the direct/indirect trace/reference to the supporting data then this information can be used to justify the assumptions)? {A3.1 [6.3.1a]}, A3.2 [6.3.1b]} |
| 5.10 | Y | N | NA | Are document references listed in the reference section of the document (i.e., including unique identification and document numbers)? |

Section (6) applies to the entire document.  Section (6) questions should only be applied after all sections of the DRD have been inspected and approved.

| 6 |  |  |  | Entire DRD Document |
| --- | --- | --- | --- | --- |
|  |  |  |  | Introduction |
| 6.1 | Y | N | NA | Are all reference documents listed in the document (i.e., including unique identification and document numbers)? |
| 6.2 | Y | N | NA | Are all acronyms and abbreviations defined in the document? |
| 6.3 | Y | N | NA | Are all definitions defined in the document? |
|  |  |  |  | Software Requirement Data |
| 6.4 | Y | N | NA | Is there a description of the allocation of system requirements to software, with attention to safety-related requirements and potential failure conditions? |
| 6.5 | Y | N | NA | Are functional and operational requirements under each mode of operation identified? |
| 6.6 | Y | N | NA | Are performance criteria (e.g., precision and accuracy)? |
| 6.7 | Y | N | NA | Are timing requirements and constraints identified? |
| 6.8 | Y | N | NA | Are memory requirements and constraints identified? |
| 6.9 | Y | N | NA | Are hardware and software interfaces, e.g., protocols, formats, frequency of inputs and frequency of outputs, identified? |
| 6.10 | Y | N | NA | Are failure detection and safety monitoring requirements identified? |
| 6.11 | Y | N | NA | Are partitioning requirements allocated to software, how the partitioned software components interact with each other, and the software levels of each partition identified? |